3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231532

# Online, April 17th – 26th 2023

Agenda Item:	13.1

Source:	Ericsson

Title:	Discussion of SA2 FS_VMR Solutions

Document for:	Agreement

# Introduction

SA2 sent an LS to RAN3 in S2-2207070 during the RAN3#117-bis-e seeking a clarification of several issues related to the SA2 work on VMR mIAB. After RAN3 replied in R3-226048, SA2 has sent a further LS in R3-230032 asking for additional clarifications for some of the issues. Finally, RAN3 replied to some of the questions in an LS in R3-231011.

In this paper we discuss the “mIAB” indication in NGAP signalling and the remaining issues raised in the LS. A further draft reply LS to SA2 is provided in the Annex.

# Discussion

We discuss the following:

- The “mIAB” indication in NGAP signalling.

- Points 2, 7 and 6 from the initial SA2 LS and the additional SA2 LS.

## The “mIAB” indication in NGAP signalling

RAN3#119 meeting notes capture the following TBC:

To be continued: NGAP Initial UE message to include an optional “mobile IAB-node indication”.

The intention is to allow the IAB-donor to select an AMF that supports mIAB-nodes. RAN2 has already agreed that it is up to RAN3 or SA2 to decide whether to support such an enhancement. Meanwhile, SA2 has agreed to support mIAB indication in the NGAP INITIAL UE MESSAGE, and this is reflected in the TS 23.501 v18.0.0, section 5.35A.1, as shown below:

------------------------------------Start of excerpt from TS 23.501------------------------------------

For a MBSR node, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization as described in clause 5.35A.4.

------------------------------------End of excerpt from TS 23.501------------------------------------

Proposal 1: Include an optional mIAB-node indication in the NGAP INITIAL UE MESSAGE.

## Point 2: TAC update at the mIAB-DU

Regarding point #2, the SA2 LS in R3-230032 states:

For point#2 (regarding KI#3), SA2 has concluded the study from SA2 perspective (as in clause 8.3 of TR 23.700-05v1.2.0). Corresponding system impacts of supporting dynamic TAC or static TAC were documented in clause 6.16.4 and 6.17.4. SA2 will align the normative specification of the work item based on RAN 2/3 feedback.

SA2 needs to be informed about any further updates and the conclusion of the discussion about TAC/RANAC update. In paper R3-231536 submitted to AI 13.3, we provide a set of proposals for handling the TAC/RANAC update. In case they are agreed, SA2 needs to be notified.

Proposal 2: Inform SA2 that the TAC update is handled as follows:

- The mIAB-DU always inherits the TAC/RANAC pertaining to the cell that serves its co-located mIAB-MT.

- The TAC/RANAC of the mIAB-DU is updated when the TAC/RANAC of the co-located mIAB-MT is changed.

- The mIAB-MT can indicate the TAC/RANAC to its co-located mIAB-DU by means of node-internal signalling, up to implementation.

- The mIAB-DU can indicate the new TAC/RANAC to its serving donor CU by means of legacy F1AP signalling (i.e., no signalling enhancements are needed).

## Point 7: Additional ULI

The SA2 LS in R3-230032 also states:

For point#7 (regarding KI#6), SA2 would like to clarify that additional information besides existing ULI from donor-gNB (as defined in Rel-17) is needed, so that the 5GC can understand that the existing ULI cannot be used directly. This is crucial for the support of services that rely on the cell ID to infer the UE locations, e.g. emergency services. Therefore, SA2 would like to request RAN3 to either confirm the support of additional ULI for the UE serviced by an MBSR or provide an alternative solution.

In our understanding, SA2’s concern is that the mIAB-MT and the mIAB-DU may be connected to different donors, where the physical location of the mIAB node is best indicated by the information about the cell that serves the mIAB-MT. However, we notice that RAN3’s decision that the TAC on the mIAB-DU reflects the physical location of the mIAB-node, in fact, resolves the SA2’s concern, provided that the mIAB-DU inherits the TAC of the cell serving the co-located mIAB-MT. In other words, the possibility to update the TAC on the mIAB-DU will ensure that the TAC of the mIAB-DU reflects the location of the mIAB node. Consequently, no further ULI information is required.

Proposal 3: Inform SA2 that, since RAN3 has agreed to pursue a dynamic TAC solution, the TAC of the mIAB-DU will reflect the location of the mIAB node, and thus no ULI enhancements are needed.

A draft reply LS is provided in the Annex.

Proposal 4: Agree the draft reply LS to SA2 provided in the Annex.

## Point 6: MBSR positioning

The SA2 LS in R3-230032 also states:

For point#6 (regarding KI#5), based on the SA2 study, NRPPa triggered procedure for the LMF to obtain MBSR location information i.e., location and velocity at a specific scheduled time could be a good alternative to the GMLC based MT-LR solution. Additionally, SA2 would also like to allow the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB. SA2 would like to ask RAN3 to consider supporting such solution within Rel-18 timeframe.

During the SA2 January meeting, a preliminary CR for TS 23.273 in S2-2301478 was agreed that implements two agreed options for LMF to obtain MBSR location information. The figure below shows the information flow from the accepted CR to TS 23.273.

Figure 1: 5GC-MT-LR procedure involving Mobile Base Station Relay from the agreed CR for TS 23.273 in S2-2301478

We consider below the impacts to RAN3 specifications from the SA2 agreements.

First, and as also described in the positioning stage-2 TS 38.305, the OAM triggers the LMF to trigger the TRP Information Exchange procedure before the start of the positioning session for a target UE. This is done to, e.g., determine the positioning capabilities of the TRPs within the NG-RAN and to obtain the Cell ID of the TRP. Based on SA2’s CR, the procedure should be enhanced so that the LMF can learn whether a TRP in the gNB is mobile (i.e., whether it is an MBSR) from the F1AP and NRPPa TRP INFORMATION RESPONSE messages. If a TRP is an MBSR, the gNB(gNB-DU) shall also include the mIAB-MT UE ID, so that the LMF triggers the actions specified in TS 23.273. In fact, we note that the need for the mIAB-MT UE ID is common to both Option 1 (for the LMF to be aware of the IAB-MT UE ID associated to a specific TRP before UE positioning) and Option 2 (to trigger the MT-LR procedure) agreed by SA2.

Observation 1: Based on the SA2 agreement, during the determination of the NG-RAN’s TRPs positioning capabilities, the LMF learns whether a TRP in the gNB is mobile and learns the mIAB-MT UE ID from the F1 and NRPPa response of the TRP Information Exchange procedures. The mIAB-MT UE ID is needed in both Option 1 and Option 2.

Therefore, we propose that both the NRPPa and F1AP TRP Information Exchange procedures present in TS 38.455 and TS 38.473 are enhanced by, e.g., adding a new codepoint ‘mbsr’ in the TRP Type IE in the TRP Information IE (clause 9.2.25 of TS 38.455 and clause 9.3.1.176 of TS 38.473), so that LMF is aware which TRPs hosted by the gNB are mobile. Also, we propose that the Mobile IAB-MT UE ID IE is included in the TRP Type IE when the TRP Type is MBSR, so that LMF knows the UE ID of the IAB-MT associated with the mobile TRP and triggers the procedures defined in TS 23.273 to determine its updated location (Option 2).

Proposal 5: Add a new ‘mobile trp’ codepoint in the TRP Type IE in the TRP Information IE in clause 9.2.25 of TS 38.455 and clause 9.3.1.176 of TS 38.473.

Proposal 6: Add a new Mobile IAB-MT UE ID IE in the TRP Information IE in clause 9.2.25 of TS 38.455 and clause 9.3.1.176 of TS 38.473, conditionally present if the TRP is MBSR.

After the LMF learns about the (mobile) TRP capabilities and its UE ID, the position of the MBSR needs to be determined. Below are the two options agreed by SA2 for MBSR positioning:

Option 1) Obtaining the location of the MBSR, velocity and the timestamp for obtaining them, via NRPPa/F1AP TRP Information Exchange procedure (described in step 5-7 of the SA2 CR).

In this option, the LMF triggers the NRPPa TRP INFORMATION REQUEST message to get the mIAB’s geo-coordinates, velocity, and the time for obtaining them. This request for MBSR updated location can be done by adding a new 'mbsr location info' codepoint in the TRP Information Type Item IE in the F1AP and NRPPA TRP INFORMATION REQUEST messages.

The MBSR location information contains the location (geo-coordinates), velocity and location time stamp. The geo-coordinates are already a part of the NRPPa and F1AP TRP Information Exchange signalling. However, they are designed for static TRP. Furthermore, the NRPPa and the equivalent F1AP procedures need to be enhanced to provide additional information about the velocity and location time stamp during step 7 of the call flow shown in Figure 1.

We note that the location information, velocity information and location time stamps are all present in the CommonIEsProvideLocationInformation message defined in the LPP specification TS 37.355 – which can be re-used to report the mobile TRP location information to the LMF, since the MBSR is considered as a UE from the LMF perspective.

------------------------------------Start of excerpt from TS 37.355------------------------------------

CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common Ies for a Provide Location Information LPP message Type.

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate			LocationCoordinates		OPTIONAL,

velocityEstimate			Velocity				OPTIONAL,

locationError				LocationError			OPTIONAL,

...,

[[	earlyFixReport-r12		EarlyFixReport-r12		OPTIONAL

]],

[[	locationSource-r13		LocationSource-r13		OPTIONAL,

locationTimestamp-r13	UTCTime					OPTIONAL

]],

[…]

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |

VelocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each velocity shape follow the rules in TS 23.032 [15].

LocationTimestamp This field provides the UTC time when the location estimate is valid and should take the form of YYMMDDhhmmssZ.

------------------------------------End of excerpt from TS 37.355------------------------------------

The location time stamp can follow the NGAP encoding:

------------------------------------Start of excerpt from TS 38.413------------------------------------

9.3.1.75        Time Stamp

This IE contains UTC time information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time Stamp | M |  | OCTET STRING (SIZE(4)) | Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [25]. |

------------------------------------End of excerpt from TS 38.413------------------------------------

Observation 2: To support Option 1 for MBSR positioning, additional information about the location, velocity, and time stamp (UTC format) of the MBSR must be provided in the F1AP and NRPPa TRP INFORMATION RESPONSE messages. This information is already part of the LPP TS 37.355 and can be exported to F1AP/NRPPa as OCTET STRINGs.

Proposal 7:

- Add a new 'mbsr location info' codepoint in the TRP Information Type Item IE in the F1AP and NRPPA TRP INFORMATION REQUEST messages, to request the RAN to trigger the MO-LR procedure.

- Add a new TRP Location Information IE in TRP INFORMATION RESPONSE messages in TS 38.455 and TS 38.473, containing the location information (location, velocity and time stamp) for one mobile TRP (e.g., a TRP hosted by an mIAB-MT).

Option 2) Trigger the MT-LR procedure towards GMLC to retrieve mIAB location

In this option, the LMF triggers the MT-LR procedure towards the GMLC, to retrieve the mIAB location information via the AMF that handles the mIAB-node. This is done using the UE ID of the mIAB received during the F1AP/NRPPA TRP Information Exchange procedures. In this solution, the impact is adding a UE ID IE in the TRP INFORMATION RESPONSE messages in TS 38.473 and TS 38.455.

Proposal 8: To support Option 2 for MBSR positioning, add mIAB-MT UE ID in the TRP INFORMATION RESPONSE TS 38.455 and TS 38.473 messages.

The following note in TS 23.273 refers to the UE-ID information:

- The LMF may also decide whether the cell Id(s) corresponds to MBSR(s) based on information received in a TRP information exchange i.e. that the cell-ID belongs to a MBSR and the UE-ID (GPSI) associated with MBSR.

We note that the two solutions agreed by SA2 in the in CR for TS 23.273 in S2-2301478 have some common impacts.

Proposal 9: Discuss the support for the two solutions for MBSR positioning agreed by SA2 in CR for TS 23.273 in S2-2301478.

We provide a CR for TS 38.455 in R3-231534 and a draftCR for TS 38.305 in R3-231534.

Additionally, during mIAB-node inter-donor migration, the LMF can trigger a new TRP Information Exchange procedure to obtain the TRP information based on knowledge from the AMF about cell ID change, since the AMF has updated ULI information. In fact, when a UE connects to the mIAB-DU, the donor-gNB CU will send the cell ID to AMF (the one broadcasted by the mIAB-DU). This cell ID may not be recognized by the LMF when received from the AMF. Then the LMF can trigger TRP message.

Observation 3: Considering that the AMF will always have updated ULI information in case of inter-CU migration, the LMF can trigger the TRP Information Exchange procedure from the AMF, based on the AMF’s knowledge of cell ID change.

Consequently, inter-CU migration incurs no NRPPa impact.

Proposal 10: No NRPPa enhancements are needed to support inter-donor migration.

# Conclusion

This paper presents our view on SA2 questions about VMR. The following is observed and proposed:

“mIAB” indication in NGAP signalling

Proposal 1: Include an optional mIAB-node indication in the NGAP INITIAL UE MESSAGE.

Point 2: TAC update at the mIAB-DU

Proposal 2: Inform SA2 that the TAC update is handled as follows:

- The mIAB-DU always inherits the TAC/RANAC pertaining to the cell that serves its co-located mIAB-MT.

- The TAC/RANAC of the mIAB-DU is updated when the TAC/RANAC of the co-located mIAB-MT is changed.

- The mIAB-MT can indicate the TAC/RANAC to its co-located mIAB-DU by means of node-internal signalling, up to implementation.

- The mIAB-DU can indicate the new TAC/RANAC to its serving donor CU by means of legacy F1AP signalling (i.e., no signalling enhancements are needed).

Point 7: Additional ULI

Proposal 3: Inform SA2 that, since RAN3 has agreed to pursue a dynamic TAC solution, the TAC of the mIAB-DU will reflect the location of the mIAB node, and thus no ULI enhancements are needed.

Proposal 4: Agree the draft reply LS to SA2 provided in the Annex.

Point 6: MBSR positioning

Observation 1: Based on the SA2 agreement, during the determination of the NG-RAN’s TRPs positioning capabilities, the LMF learns whether a TRP in the gNB is mobile and learns the mIAB-MT UE ID from the F1 and NRPPa response of the TRP Information Exchange procedures. The mIAB-MT UE ID is needed in both Option 1 and Option 2.

Proposal 5: Add a new ‘mobile trp’ codepoint in the TRP Type IE in the TRP Information IE in clause 9.2.25 of TS 38.455 and clause 9.3.1.176 of TS 38.473.

Proposal 6: Add a new Mobile IAB-MT UE ID IE in the TRP Information IE in clause 9.2.25 of TS 38.455 and clause 9.3.1.176 of TS 38.473, conditionally present if the TRP is MBSR.

Observation 2: To support Option 1 for MBSR positioning, additional information about the location, velocity, and time stamp (UTC format) of the MBSR must be provided in the F1AP and NRPPa TRP INFORMATION RESPONSE messages. This information is already part of the LPP TS 37.355 and can be exported to F1AP/NRPPa as OCTET STRINGs.

Proposal 7:

- Add a new 'mbsr location info' codepoint in the TRP Information Type Item IE in the F1AP and NRPPA TRP INFORMATION REQUEST messages, to request the RAN to trigger the MO-LR procedure.

- Add a new TRP Location Information IE in TRP INFORMATION RESPONSE messages in TS 38.455 and TS 38.473, containing the location information (location, velocity and time stamp) for one mobile TRP (e.g., a TRP hosted by an mIAB-MT).

Proposal 8: To support Option 2 for MBSR positioning, add mIAB-MT UE ID in the TRP INFORMATION RESPONSE TS 38.455 and TS 38.473 messages.

Proposal 9: Discuss the support for the two solutions for MBSR positioning agreed by SA2 in CR for TS 23.273 in S2-2301478.

Observation 3: Considering that the AMF will always have updated ULI information in case of inter-CU migration, the LMF can trigger the TRP Information Exchange procedure from the AMF, based on the AMF’s knowledge of cell ID change.

Proposal 10: No NRPPa enhancements are needed to support inter-donor migration.

# Annex: Draft reply LS to SA2

3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-23xxxx

# Online, April 17th – 26th 2023

Title:	[Draft] Further reply LS on FS_VMR solutions review

Response to:	R3-230032 (S2-2211437)

Release:	Rel-18

Work Item:	FS_VMR

Source:	Ericsson [To be RAN3]

To:	SA2

Cc:	RAN, RAN2, RAN4

Contact Person:

Name: 		Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA2 for the LS on FS_VMR solutions review. In addition to the previous RAN3 reply LS in R3-231011/S2-2303963, RAN3 provides the following feedback on the remaining points raised by SA2:

-	SA2 point #2 (regarding KI#3): SA2 has concluded the study from SA2 perspective (as in clause 8.3 of TR 23.700-05v1.2.0). Corresponding system impacts of supporting dynamic TAC or static TAC were documented in clause 6.16.4 and 6.17.4. SA2 will align the normative specification of the work item based on RAN 2/3 feedback.

RAN3’s feedback on point #2: In addition to the RAN3#119 agreements communicated in S2-2303963 (R3-231011), RAN3 agreed the following at the RAN3#119-bis-e meeting:

- The mIAB-DU always inherits the TAC/RANAC pertaining to the cell that serves its co-located mIAB-MT.

- The TAC/RANAC of the mIAB-DU is updated when the TAC/RANAC of the co-located mIAB-MT is changed.

- The mIAB-MT can indicate the TAC/RANAC to its co-located mIAB-DU by means of node-internal signalling, up to implementation.

- The mIAB-DU can indicate the new TAC/RANAC to its serving donor CU by means of legacy F1AP signalling (i.e., no signalling enhancements are needed).

-	SA2 point #6 (regarding KI#5), based on the SA2 study, NRPPa triggered procedure for the LMF to obtain MBSR location information i.e., location and velocity at a specific scheduled time could be a good alternative to the GMLC based MT-LR solution. Additionally, SA2 would also like to allow the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB. SA2 would like to ask RAN3 to consider supporting such solution within Rel-18 timeframe.

RAN3’s feedback on point #6: RAN3 has taken the preliminary CR for TS 23.375 in S2-2301478 as the baseline for the discussion on MBSR positioning. RAN3 has observed that both options require similar enhancements of NRPPA and F1AP TRP Information Exchange procedures: addition of mobile TRP capability and UE ID identified as GPSI to be included in the TRP Information IE. RAN3 has agreed to introduce Mobile TRP location information with location coordinates, velocity, and location time stamp to support the options described in TS 23.273

-	SA2 point #7 (regarding KI#6): SA2 would like to clarify that additional information besides existing ULI from donor-gNB (as defined in Rel-17) is needed, so that the 5GC can understand that the existing ULI cannot be used directly. This is crucial for the support of services that rely on the cell ID to infer the UE locations, e.g., emergency services. Therefore, SA2 would like to request RAN3 to either confirm the support of additional ULI for the UE serviced by an MBSR or provide an alternative solution.

RAN3’s feedback on point #7: Given RAN3 has agreed to pursue a dynamic TAC solution, the TAC of the mIAB-DU will reflect the location of the mIAB node, and thus no ULI enhancements are needed.

2. Actions:

To SA2 group.

ACTION: 	RAN3 respectfully asks SA2 to take the above feedback into account.

3. Dates of Next RAN3 Meeting:

RAN3#120	22nd – 26th May 2023	Incheon, Korea