3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231535

# Online, April 17th – 26th 2023

Agenda Item:	13.2

Source:	Ericsson

Title:	Migration Procedure for Mobile IAB-Nodes

Document for:	Agreement

# Introduction

In this paper we discuss the migration procedure for mIAB-nodes based on the RAN3#119 and earlier agreements and TBCs.

# Discussion

We discuss the following issues:

- Triggering of F1 setup towards mIAB-DU’s target donor CU.

- Indication of the IDs of the mIAB-MT and its donor CU to the donor CU of the mIAB-DU.

- mIAB-DU migration and co-located mIAB-MT HO towards different donor CUs.

- Integration of mIAB-MT and mIAB-DU to different donor CUs.

- Backhaul transport to/from two logical mIAB-DUs.

- NGAP support for mIAB-node mobility.

## Triggering of F1 setup towards mIAB-DU’s target donor CU

The RAN3#119 meeting notes capture the following agreement:

When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the information of target logical mIAB-DU’s CU (e.g. IP address, gNB-ID).

At this point, it is only certain that the gNB-ID of the target CU for mIAB-DU migration needs to be indicated to the mIAB-DU. Other potential parameters can be discussed later.

Proposal 1: The source donor CU for mIAB-DU migration triggers the mIAB-DU to initiate F1 setup towards the target donor CU by indicating to the mIAB-DU the gNB-ID of the target donor CU.

## Indication of the IDs of the mIAB-MT and its donor CU to the donor CU of the mIAB-DU

Another RAN3#119 agreement states:

In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU.

It should be clarified that the above is executed by including the mIAB-MT’s ID and the ID of mIAB-MT’s CU in the F1 SETUP REQUEST message.

Proposal 2: When mIAB-DU’s target CU is different from the mIAB-MT’s CU, the mIAB-DU includes the gNB-ID of the mIAB-MT’s CU and the mIAB-MT’s ID in the F1 SETUP REQUEST message towards the target CU.

Meanwhile, the following RAN3#118 agreement pertains to the scenario where the mIAB-MT undergoes consecutive partial migrations:

The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the:

- gNB ID of the target donor CU for the mIAB-MT HO.

- ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed.

The above agreement implies that the parameters are passed to the mIAB-DU’s CU via XnAP signalling. Nevertheless, we note that allowing the mIAB-DU to pass the two parameters to its donor CU via F1AP would have at least two benefits:

- It would also work for the scenario where there is no XnAP connectivity between mIAB-DU’s and mIAB-MT’s donor CUs.

- It would principally align with the mIAB-DU migration scenario, where the gNB-ID of the mIAB-MT’s CU and the mIAB-MT’s ID at this CU are sent to the mIAB-DU’s target CU from the mIAB-DU.

Proposal 3: During consecutive partial migrations of the mIAB-node, the mIAB-DU provides to the CU of the mIAB-DU the gNB-ID of the mIAB-MT’s target CU and the ID of the mIAB-MT at this CU.

## mIAB-DU migration and co-located mIAB-MT HO towards different donor CUs

The following RAN3#118 WA pertains to migrating the mIAB-DU and handing over the mIAB-MT towards different target donor CUs:

WA: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. This WA is subject to validation that the impact involved is affordable.

So, the use case and benefits of supporting the migration of mIAB-DU to a donor CU different than the donor CU for the mIAB-MT are acknowledged by RAN3, and further discussion should focus on the consequent impact. We foresee that, in such a scenario (illustrated in Figure 1), the migration procedure requires the following procedure executions (not necessarily shown in a chronological order):

- F1 setup from the second logical mIAB-DU towards the target donor CU for the mIAB-DU migration is triggered.

  - NOTE: This step creates no additional impact - it needs to be supported for any scenario of mIAB-DU migration, i.e., regardless of whether the mIAB-DU is migrated towards the mIAB-MT’s donor CU or to another donor CU.

- The first logical mIAB-DU informs the source donor CU for mIAB-DU migration that the second logical mIAB-DU successfully established an F1 connection towards the target CU, and indicates the cell ID(s) served by the second logical mIAB-DU.

- NOTE: This was agreed at the RAN3#119 meeting. No additional impact.

Figure 1: Migration of mIAB-DU to a target CU different than the CU serving the mIAB-MT

- The second logical mIAB-DU informs the target donor CU for mIAB-DU migration about the gNB ID of the donor CU serving the mIAB-MT and the ID(s) of the mIAB-MT, so that the target CU can coordinate with the mIAB-MT’s CU the migration of F1 transport.

  - NOTE: This was agreed at the RAN3#119 meeting. No additional impact.

- The mIAB-DU’s target donor CU initiates the (existing) XnAP IAB Transport Migration Management procedure towards the mIAB-MT’s donor CU, to coordinate the proxying of F1 transport terminated at the target donor CU.

  - NOTE: This was agreed at the RAN3#119 meeting. No additional impact.

- The UEs are handed over between the two logical mIAB-DUs. After all the UEs have been handed over, the F1 of the first logical mIAB-DU and the F1 transport migration association between mIAB-DU’s source CU and the mIAB-MT’s CU can be released.

Based on the above analysis, we conclude that the impact of enabling mIAB-DU towards a donor CU different than the donor CU for the mIAB-MT HO is minimal and certainly affordable. Not only that the Rel-17 XnAP IAB Transport Migration Management procedure can be reused, but also that the new signalling needed for this scenario will anyway need to be defined, even for the scenarios where the target CU for the mIAB-DU migration and the mIAB-MT handover is the same.

Observation: All the signalling needed to support migrating the mIAB-DU to a donor CU different that the donor CU of the mIAB-MT either already exists or has been agreed, even for the scenarios where the target CU for the mIAB-DU migration and the mIAB-MT handover is the same.

Proposal 4: Turn into an agreement the WA that the mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs.

## Integration of mIAB-MT and mIAB-DU to different donor CUs

As explained in our earlier papers, it is essential that the donor CU of the mIAB-DU provides a good “Xn coverage” and good transport connectivity, whereas the radio coverage is of secondary importance. Conversely, the mIAB-MT should connect, and be handed over to, the donor CU offering the best radio coverage. The above holds not only for the topology adaptation, but also for initial connection to the network. We therefore propose that the mIAB-MT and mIAB-DU should also be allowed to connect to different donor CUs at start-up, i.e., during the mIAB-node integration procedure.

Proposal 5: The mIAB-MT and its co-located mIAB-DU can connect to different donor CUs when joining the network.

## Backhaul transport to/from two logical mIAB-DUs

The following was agreed at the RAN2#121 meeting:

For the upstream data handling at the BAP of mobile IAB MT, one common default BAP configuration to be used by both logical DUs is the baseline. RAN2 to further discuss the need of using logical-DU-specific default BAP configuration (e.g. when the two logical DUs use different donor-DUs).

For the upstream data handling at the BAP of mobile IAB MT, RAN2 assume that the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection (To be confirmed by RAN3).

For the downstream data handling arriving at the mobile IAB node, RAN2 assume upper layers (e.g. IP layer) can differentiate the data to different logical DUs based on e.g. the IP address, i.e. no need to introduce logical-DU-specific BAP address. (To be confirmed by RAN3).

According to the above, the last two agreements need to be verified by RAN3, and we propose that RAN3 does so, given their low (or even non-existing) impact.

Proposal 6: Confirm the following RAN2 agreements:

- For the upstream data handling at the BAP of mobile IAB MT, the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection.

- For the downstream data handling arriving at the mobile IAB-node, the upper layers (e.g., IP layer) can differentiate the data to different logical DUs based on, e.g., the IP address.

## NGAP support for mIAB-node mobility

An RAN3#119 agreement states the following:

For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF.

For partial migration, the Rel-17 XnAP IAB TM procedures can indeed be ported to the NGAP in a transparent way, as the agreement states. As agreed for the XnAP scenario, the source CU for mIAB-DU migration indicates the “contact details” of the target CU to the mIAB-DU. After that, the new F1 connection is established, and the target donor CU can initiate the IAB TM procedure towards the donor CU serving the mIAB-MT. If the Rel-17 XnAP IAB TM procedures are ported to the NGAP, and if the source CU and the target CU receive the necessary information directly from the mIAB-node (i.e., from their respective logical mIAB-DUs), the mIAB-DU migration in the absence of XnAP connectivity incurs no additional NGAP impact.

Proposal 7: In the absence of XnAP connectivity, during the mIAB-DU migration, the source CU and target CU for mIAB-DU migration can receive the necessary information directly from the mIAB-node’s logical mIAB-DU’s.

With respect to the above agreement, we currently see no obstacles to supporting it.

Proposal 8: In the absence of XnAP connectivity, the IAB TM procedure between the target CU for mIAB-DU migration and the mIAB-MT’s CU can be executed via NGAP.

# Conclusion

This paper discusses the IAB-node mobility procedure. The following is observed and proposed:

Proposal 1: The source donor CU for mIAB-DU migration triggers the mIAB-DU to initiate F1 setup towards the target donor CU by indicating to the mIAB-DU the gNB-ID of the target donor CU.

Proposal 2: When mIAB-DU’s target CU is different from the mIAB-MT’s CU, the mIAB-DU includes the gNB-ID of the mIAB-MT’s CU and the mIAB-MT’s ID in the F1 SETUP REQUEST message towards the target CU.

Proposal 3: During consecutive partial migrations of the mIAB-node, the mIAB-DU provides to the CU of the mIAB-DU the gNB-ID of the mIAB-MT’s target CU and the ID of the mIAB-MT at this CU.

Observation: All the signalling needed to support migrating the mIAB-DU to a donor CU different that the donor CU of the mIAB-MT either already exists or has been agreed, even for the scenarios where the target CU for the mIAB-DU migration and the mIAB-MT handover is the same.

Proposal 4: Turn into an agreement the WA that the mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs.

Proposal 5: The mIAB-MT and its co-located mIAB-DU can connect to different donor CUs when joining the network.

Proposal 6: Confirm the following RAN2 agreements:

- For the upstream data handling at the BAP of mobile IAB MT, the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection.

- For the downstream data handling arriving at the mobile IAB-node, the upper layers (e.g., IP layer) can differentiate the data to different logical DUs based on, e.g., the IP address.

Proposal 7: In the absence of XnAP connectivity between them, the source CU and the target CU for mIAB-DU migration can receive the necessary information directly from the mIAB-node’s logical mIAB-DU’s.

Proposal 8: In the absence of XnAP connectivity, the IAB TM procedure between the target CU for mIAB-DU migration and the mIAB-MT’s CU can be executed via NGAP.