3GPP TSG-RAN3 #119bis-e	R3-231575

Online, 17th – 26th April 2023

Agenda Item:	14.3

Source:	Ericsson

Title:	CHO with candidate SCG(s)

Document for:	Approval

# 1	Introduction

At the RAN3#118 meeting, some agreements have been reached and FFS remains as follows.

RAN3#118:

Direct data forwarding is supported by current specification, FFS on further signaling enhancement.

Optimization on indirect data forwarding is by network implementation.

RAN3 acknowledges unnecessary signaling exchange between MN and the target SN would cause inefficiency and extra latency for CHO + NR-DC, the solution is FFS.

The issue on new problem of CHO with multiple SCGs at the target side is FFS.

WA: In CHO with (multiple) SCG configuration, the (candidate) SN can acknowledge whether it has direct data forwarding path with source SN. If existed, it can assign the same data forwarding address for multiple data forwarding paths, otherwise, it is up to the candidate SN implementation.

This paper further expands upon our analysis.

# 2	Discussion

## 2.1	Avoid unnecessary signaling

RAN plenary updated the Mobility WID by adding the second bullet for Objective 3.

- For CHO including target MCG and target SCG in NR-DC [RAN3]:

  - to specify data forwarding optimizations; and

  - to specify, if needed, a solution to avoid unnecessary signaling exchange between source MN and target SN.

When only CHO is configured, the MN can know what type of reconfiguration it makes and determine whether the target node needs to be contacted in order to update the CHO configuration. When CHO is configured with CPC or MR-DC, the source SN may decide to reconfigure the UE, e.g., measConfig update, however the MN has no clue whether the reconfiguration will impact the target configuration for CHO + CPC or CHO + MR-DC.

According to current standards, if the source SN intends to update the currently used source SN configuration, it forwards the new configuration to the MN. The MN does not decode the configuration, but blindly forwards it to the target SN. After receiving the updated config, the target SN responds to the MN potentially updating the CPC config. However, the source SN may not intend to update anything related to conditional config in the UE. In that case the MN indeed does not have to forward the configuration to the target SN, which causes unnecessary delay.

Since the source SN would decide whether to update some configuration, like MeasConfig which covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps, the possibility is rather high to reconfigure the UE without impact to CHO. While some companies have suggested using SRB3 as a means of avoiding this possibility, it remains unclear whether configuring SRB3 is the appropriate course of action in this particular case.

The following figure depicts an example during SN reconfigurations, in which the MN would not be able to tell whether this updated configuration includes only SN measurement config update or others.

Figure 1. Example signalings for SN reconfigurations

To make the network signaling exchange more efficient, the SN should tell explicitly the MN whether the reconfiguration impacts the conditional target configuration or not, for instance, by an indicator.

The source SN informs the MN whether the target CHO + MR-DC or CHO+CPC configuration needs to be updated, if there is a reconfiguration.

The above solution has been listed as Solution 1 in [2]. Several other solutions have been summarized in SoD as follows.

Solution 3: Introduce a a bitmap (e.g. 16 bits) is added to the CHO Request Acknowledge, where each bit corresponds to a configurable option that may or may not be kept at the target for the UE

Solution 4: Solution 1 + using SRB3

Solution 5: Enhance HO REQ ACK message so that, during CHO preparation phase, T-MN can provide (a) whether CHO(+NR-DC/CPAC) configuration was generated by full config, or (b) full config was used only on SN part

Compared among the above solutions, solution 3 is more complicated and would impact on UE. Instead, a simple indicator in Solution 1 works well. Solution 5 is proposed from target perspective, which requires more signaling. Thus, Solution 1 is the best solution for this optimization.

There is another discussion related to coexistence between Rel-16 and Rel-17 CPC, and an indicator was agreed to be added for the source SN to tell the MN whether the SCG reconfiguration is executed or updated. One way would be to reuse the same indicator by extending the value. And MN’s behavior needs to be specified but not to release all the conditional reconfigurations. Thus, two options are given below about how to avoid unnecessary signaling between the MN and the target SN.

Option 1: Introduce a new indicator in the SN MODIFICATION REQUIRED message to indicate whether the source SCG reconfiguration has impact on the target SCG or not.

Option 2: reuse the existing indicator SCG Reconfiguration Notification IE by extending the values.

While both options are being compared, there is a slight preference towards Option 1 as it is deemed a more straightforward approach. The corresponding CR is provided in [3].

RAN3 agrees to introduce a new indicator to indicate whether the source SCG reconfiguration would impact the target SCG or not, i.e., Solution 1.

## 2.2	Data forwarding aspects

RAN3 has agreed to support early data forwarding for CHO with target SCG. In general, we believe that the duplicated data forwarding for CHO with single SCG is not a critical issue. As known, the maximum number of cells that can be prepared for a CHO is 8, thus it is assumed that maximum 8 target MNs (one cell per node) might be involved. That means a worst case would be 8 target MNs (most unlikely) select the same target SN, then 7 times of data forwarding could be redundant. However, these prerequisites most probably will not happen at the same time.

It is clear that the target SN can understand that the multiple SN Addition procedures are part of the same CHO preparation. For direct data forwarding in case of CHO with SCG becomes similar as direct data forwarding for normal handover with SCG. For indirect data forwarding, once the target MN does not get any information from the target SN about the availability of direct path, it may choose to use indirect path. In that case, then the duplicated data forwarding may not be considered as an issue.

No specific handling is seen for direct data forwarding for CHO + target SCG.

Indirect data forwarding can be supported by an implementation way.

Rel-18 data forwarding optimization should focus on adding early data forwarding signaling flows for CHO with target SCG(s) in the newly added sections for CHO + MR-DC at stage-2.

In last meeting, it has been confirmed that the early data forwarding for CHO with multiple SCGs is a new problem. Considering there could be similarities between CHO with single SCG and multiple SCGs, or certain mechanism can be reused, we propose to take commonality of the solutions for both single and multiple SCGs.

Adopt a similar mechanism to support early data forwarding for CHO with multiple SCGs.

## 2.3	CHO associated with CPAC configurations

In RAN2#121 the following agreements were made:

RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18

The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above

The simultaneous evaluation of CHO and CPC will be supported, while the sequence of execution is still open.

After RAN2 figure out more details on the execution conditions, how does the UE get the execution conditions from the target MN to the source MN should be taken into account. Two ways are possible, either the info is transferred via an inter-node message or explicitly over XnAP.

The support of CHO associated with CPAC configurations may need enhancements on network signaling. The discussion is pending in RAN2.

RAN3 to check the possible impacts over Xn to support CHO associated with CPAC configurations.

# 3	Conclusion

In the previous sections we made the following observation:

Observation 1	No specific handling is seen for direct data forwarding for CHO + target SCG.

Observation 2	Indirect data forwarding can be supported by an implementation way.

Observation 3	The support of CHO associated with CPAC configurations may need enhancements on network signaling. The discussion is pending in RAN2.

In this paper we propose:

Proposal 1	The source SN informs the MN whether the target CHO + MR-DC or CHO+CPC configuration needs to be updated, if there is a reconfiguration.

Proposal 2	RAN3 agrees to introduce a new indicator to indicate whether the source SCG reconfiguration would impact the target SCG or not, i.e., Solution 1.

Proposal 3	Rel-18 data forwarding optimization should focus on adding early data forwarding signaling flows for CHO with target SCG(s) in the newly added sections for CHO + MR-DC at stage-2.

Proposal 4	Adopt a similar mechanism to support early data forwarding for CHO with multiple SCGs.

Proposal 5	RAN3 to check the possible impacts over Xn to support CHO associated with CPAC configurations.

# 4	References

RP-223520

R3-230891, Summary of Offline Discussion on CB: # 23_mobilily_CHO_MRDC, Lenovo

R3-231577, Avoid unnecessary signaling due to SCG reconfigurations, Ericsson, ZTE, Lenovo