3GPP TSG-RAN WG3 Meeting #119bises	R3-231597

Moyeuvre-Petite, 17- 26 April, 2023

Title: 	Further discussion on UE context release issue

Source: 	Huawei

Agenda item:	18.2

Document Type:	Discussion

# 1. Introduction

In the last meeting, RAN3 has the following agreement as well as a minute.

There is no need to enhance the existing S1AP procedure to support the tracking area reported by RAN before AN release. RAN3 relies on the fact that the procedure will be UE context release complete procedure without failure.

In this paper, based on the agreements we achieved, we analyze some potential issues that were probably ignored by the group.

# 2. Discussion

2. 1 SA2’s requirement

In the conclusion of TR 23.700-28 [1], SA2 states the following:

- AMF/MME takes the Tracking Area reported by RAN before AN release as the paging area.

Moreover, in SA2’s latest CR S2-2301853, they have agreed the following:

### 5.4.X.y Paging In case of NR satellite access that provides discontinuous network coverage, AMF may utilize sub-area paging as described in clause 4.2.3.3 of TS 23.502 [3] (e.g. first page in the last known cell-id or TA and retransmission in all registered TAs). The AMF may utilize the location information as received at or before the AN release due to the discontinuous coverage for paging optimisation.

We do understand, from SA2 agreements, that RAN is require to report the Tracking Area to CN every time a UE context release is about to be happened. SA2 applies those statements to IoT NTN and NR NTN.

Observation 1: RAN needs to make sure the latest UE location obtained at or before UE context release is successfully forwarded to CN

In the last meeting, companies think no matter it is eNB or MME trigger the UE context release, we will always have a UE CONTEXT RELEASE COMPLETE message sent back to MME, which includes User location information (ULI), so no enhancement is needed. Correspondingly, we have the following agreement:

There is no need to enhance the existing S1AP procedure to support the tracking area reported by RAN before AN release. RAN3 relies on the fact that the procedure will be UE context release complete procedure without failure.

However, we note that, the ULI included in UE CONTEXT RELEASE COMPLETE message is an optional IE, which is not mandated to be provided in case of NTN as requested by SA2.

Observation 2: The ULI included in UE CONTEXT RELEASE COMPLETE message is optional, which is mandate for the IoT NTN and NR operations.

There is no Text procedure today specifying the behaviour of an Information Element for NTN. We do prefer to minimize the specification impact. Therefore, this is a NTN specific case, and the potential solution is adding a description in the Semantics description, avoiding also usage of shall in favour of SA2 reference by declaring that ‘In NTN, contains the LTE NTN TAI Information IE as specified in TS 23.502 [51].. The corresponding TP is attached in the annex for S1AP and NG AP

Proposal 1: Add a description for ULI IE in UE CONTEXT RELEASE COMPLETE message to force the presence of ULI in the context of NTN

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: RAN needs to make sure the latest UE location obtained at or before UE context release is successfully forwarded to CN

Observation 2: The ULI included in UE CONTEXT RELEASE COMPLETE message is optional, which is mandate for the IoT NTN and NR operations.

Proposal 1: Add a description for ULI IE in UE CONTEXT RELEASE COMPLETE message to force the presence of ULI in the context of NTN. The TP are proposed for TS 36.413 and TS 38.413.

# 4. Appendix – TP for 36.413

****************** START CHANGE ***************

#### 9.1.4.7	UE CONTEXT RELEASE COMPLETE

This message is sent by the eNB to confirm the release of the UE-associated S1-logical connection over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| User Location Information | O |  | 9.2.1.93 | In NTN, contains the LTE NTN TAI Information IE as specified in TS 23.502 [51]. | YES | ignore |
| Information on Recommended Cells and eNBs for Paging | O |  | 9.2.1.105 |  | YES | ignore |
| Cell Identifier and Coverage Enhancement Level | O |  | 9.2.1.109 |  | YES | ignore |
| Secondary RAT Usage Report List | O |  | 9.2.1.124 |  | Yes | ignore |
| Time Since Secondary Node Release | O |  | 9.2.1.143 |  | Yes | ignore |

# 5. Appendix – TP for 38.413

****************** START CHANGE ***************

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the NG-RAN node to confirm the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 | In NTN, contains the NR NTN TAI Information IE as specified in TS 23.502 [10]. | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| PDU Session Resource List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session Resource Release Response Transfer | O |  | OCTET STRING | Containing the PDU Session Resource Release Response Transfer IE specified in subclause 9.3.4.21. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |