3GPP TSG-RAN3 Meeting #119bis-e	R3-231607

Online, 17th April – 26th April 2023

Title:	[DRAFT] LS on Continuous MDT

Release:	Rel-18

Work Item:	NR_AIML_NGRAN-Core

Source:	Ericsson, Deutsche Telekom, InterDigital (to become RAN3)

To:	SA5

Cc:	RAN2

Contact Person:

#### Name:                Angelo Centonza

E-mail Address:   angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

During the work on AI/ML for NG-RAN, RAN3 discussed the concept of Continuous MDT.

Such concept consists of providing the NG-RAN with Management Based Immediate and Logged MDT configurations using different Trace Sessions.

The NG-RAN selects and configures UEs with the Management Based immediate MDT measurements and Logged MDT configuration. 
This allows such UEs to collect MDT data continuously when moving from RRC_Connected to RRC_Idle/Inactive.

To achieve continuous MDT data collection during transitions from RRC_Idle/Inactive to RRC_Connected, the solution foresees UEs to provide a “Continuous MDT Indication” to the NG-RAN as soon as the UE moves to RRC_Connected.

This enables the selection of the same UE (previously configured with Logged MDT) for following Immediate MDT measurement configuration. The details of the Continuous MDT solution in RAN3 are for further study.

A number of companies identified benefits with Continuous MDT, due to the possibility for UEs to provide a continuous time series of data, which can be of use for AI/ML model training/re-training.

RAN3 would like to ask the following questions to SA5 concerning Continuous MDT:

- Can Continuous MDT be achieved by means of current MDT specifications?

- If the answer to 1) is “no”, would it be feasible to achieve Continuous MDT via the enhancements described above, given the current status of specifications?

2. Actions to SA5:

RAN3 kindly asks SA5 to answer the questions above and to provide feedback on Continuous MDT, if any.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #120		22 May -26 May 2023                         Incheon, KR