3GPP TSG-RAN Meeting #119bis	R3-231690

# 17th – 26th Apr 2023

# Online

Agenda item:	10.2.5

Source: 	ZTE

Title: 	Further discussion on NR-U optimizations

Document for:	Discussion and Approval

- Introduction

In last meeting, RAN3 has made some agreements as below.

DU shall report the UL COT to CU in resource status report procedure when it is requested by CU, how to obtained this information by DU is up to implementation. The granularity of UL COT reporting in F1 interface should be NR-U channel level.

However, there are still some open issues on the NR-U for MRO and MLB, e.g., the enhancement of RLF report and RA report. In this contribution, we provide our further consideration on these issues.

- Discussion

  - MRO

    - RLF report enhancements

In last meeting, for the RLF report enhancements, the EDT in UL has been discussed. While some companies had some concerns on which type of the EDT value should be applied, i.e., exact value, average value or the maximum value.

As shown in TS 37.213, the UE shall set the energy detection threshold when accessing a channel with UL transmission, which means the UL Energy Detection Threshold should be set by the UE. And the UE is able to set the EDT in UL to be less than or equal to the maximum EDT. In addition, according to TS 38.331, the network is able to configure the maximum energy detection threshold to the UE via ServingCellConfig IE. Furthermore, the exact value is too dynamic to reflect the overall status. Since the average value of EDT could provide the status of the NR-U channel within the target cell, the NG-RAN node is able to adjust the handover strategy to avoid the HOF due to the consistent LBT failures. Therefore, the average value of the energy detection threshold should be added in the RLF report.

Proposal 1: Add the average value of EDT in UL from the UE with consistent LBT failure in the RLF report.

    - RA report enhancements

Regarding the RA report, the information of LBT failures occurring during the RA procedure could be beneficial. On the top of the indication of LBT failure per RA procedure, the number of LBT failure could also be considered, while the granularity should be clarified, it should be per BWP per RA procedure. For the EDT in UL and Measured RSSI, we think the interference status of the NR-U channel should be included in the RLF report other than the RA report. For the UL LBT duration time, the exact time information seems to be not necessary for the RA procedure.

Proposal 2: Add the number of the LBT failure per BWP per RA procedure in the RA report.

    - SCG failure information enhancements

For the SCG failure information, similar as the RLF report, both of the measured RSSI and the indication of SCG failure due to consistent LBT failure could be beneficial for the receiving node.

Proposal 3: Add the measured RSSI and indication of SCG failure due to consistent LBT failure in the SCG failure information.

  - UL MLB

In last meeting, the Channel Occupancy Time Percentage for UL has been introduced over F1[1], while the presence of this IE is FFS. Since channel occupancy time percentage is able to provide the utilization of the channel resources for the corresponding NR-U channel and the channel occupancy time percentage for DL has already been introduced as mandatory in Rel-17. It is reasonable to make the channel occupancy time percentage for UL as mandatory.

Proposal 4: The presence of Channel Occupancy Time Percentage UL over F1 should be mandatory.

- Conclusion

Proposal 1: Add the average value of EDT in UL from the UE with consistent LBT failure in the RLF report.

Proposal 2: Add the number of the LBT failure per BWP per RA procedure in the RA report.

Proposal 3: Add the measured RSSI and indication of SCG failure due to consistent LBT failure in the SCG failure information.

Proposal 4: The presence of Channel Occupancy Time Percentage UL over F1 should be mandatory.

The corresponding TP for SON BL CR 38.473 is given in the Annex.

- References

[1] R3-230980 (TP to 38.473) F1 enhancement to support UL COT reporting, Samsung, Ericsson, ZTE

[2] TS 37.213

[3] TS38.331

- Annex

TP for SON BL CR 38.473 on UL MLB for NR-U

<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>

9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 corresponds to the duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-ChannelOccupancyTimePercentageUL,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

[snip]

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						INTEGER(1..maxnoofNR-UChannelIDs),

channelOccupancyTimePercentageDL		ChannelOccupancyTimePercentage,

energyDetectionThreshold				EnergyDetectionThreshold,

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Item-ExtIEs} } OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore EXTENSION ChannelOccupancyTimePercentage PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>