3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231707

E-meeting, 17 April – 26 April, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0062 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Packet Delay Budget |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CATT,China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to support service that can be used over satellite access type, SA2 already agreed to extend PDB value for 5QI 10 to 1100ms in Rel-17 [S2-2106653]. While in current specification, the maximum PDB value is 512ms.(i.e. An integer value 1023 with granularity of 0.5ms) Therefore, the maximum PDB value in RAN side should be extended to more than 1100ms for Rel-17. It is noted there is no requirements to introduce new granularity for Packet Delay Budget IE, then in order to correct the issue in backward compatible way, it is proposed to use the Packet Delay Budget IE with same unit of 0.5ms. In order to align the PDB value in all RAN interfaces, it is proposed to extend the PDB value in E1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce a new value range of 1024.. 2199 for Packet Delay Budget IE with granularity of 0.5ms. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has limited impact on the protocol and the impact can be considered isolated. This CR has no functionality impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between NGAP/XnAP for Value range of PDB IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.47, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0980 TS 38.423 CR 1030 TS 38.473 CR 1162 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

<Unchanged Text Omitted>

9.3.1.47	Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Delay Budget | M |  | INTEGER (0..1023, … ,1024.. 2199) | Upper bound value for the delay that a packet may experience expressed in unit of 0.5ms. |

<Unchanged Text Omitted>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<Unchanged Text Omitted>

-- P

PacketDelayBudget ::= INTEGER (0..1023, ... ,1024.. 2199)

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>