3GPP TSG-RAN3 Meeting #119b-e	R3-231756

# Online, 17th April – 26th  April, 2023

Agenda Item:	17.2

Source: 	Huawei

Title: 		Further discussion on cell ID usage and TAC

Document for:	Discussion

Introduction

In the last meeting, with some agreements achieved, we yet have the following technical issues to be further discussed:

Turn WA to agreement: The Uu cell ID is used as target Cell ID in both NG and Xn handover signaling. Whether to use Uu cell ID or mapped cell ID over Xn configuration update procedure?

In this paper, we provide some further considerations regarding these issues. In general, we try to find issue like if something is broken in NR NTN and we see nothing wrong. We do not preclude future discussion or improvement on automation and OAM based on SON for NR NTN, which is however not the focus for now.

Discussion

2.1 Cell ID usage in scenarios other than handover

In the last meeting, RAN3 agreed to use Uu cell ID as target cell ID in both Xn and NG handover signalling. However, it was still controversial on which cell ID to use in non-UE associated Xn signalling, i.e. Xn setup procedure and NG-RAN node configuration update procedure.

First, we think it is reasonable to assume tracking area is always larger than the geographical area represented by a mapped cell ID, since mapped cell is designed to enable the AMF to be aware of the UE location for Paging, PWS, etc., which usually corresponds to a smaller geographical area. Then, based on this assumption, it seems exchanging mapped cell IDs for declaring neighbouring is straightforward and follows legacy behaviour. By which method, RAN nodes can declare all the served cell and corresponding neighbouring cells using mapped cell IDs combined with a single TAC, where the single TAC can be used to confirm whether the neighbouring cell indicated by the mapped cell ID is complied with the service area restriction or not.

One straightforward solution is exchanging mapped cell ID with a single TAC in non-UE associated Xn signalling.

On the other hand, the other option to exchange the Uu cell ID, which is associated with a beam, during Xn setup procedure is also valid in our understanding. However, we may need to be careful about the TAC exchange, as in this case Uu cell has larger coverage than TAC, and there may be some issues regarding MRL. Before discussing more about whether an enhancement to TAC is needed, we are a bit unsure of the need of specifying clearly the cell ID type in this case. Note that, so far, we have only specified the following cell ID corresponds to a mapped cell ID

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported: - The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information; - The Cell Identity used for Paging Optimization in NG interface; - The Cell Identity used for Area of Interest; - The Cell Identity used for PWS.

And according to the agreement achieved in the last meeting, we agree the cell identity used in the NG and Xn handover messages is expected to be Uu Cell ID. However, what about all the other cases where a CGI is used? What about the other interfaces like F1 and E1? We are wondering is there really a need to specify clearly the cell ID usage for all the cases. The NR CGI is at least also used in cell activation and node configuration update in Xn; F1 setup request, GNB-DU/GNB-CU configuration update, Resource Status Report/Update, UE context setup, Access success, initial UL RRC Message and Paging in F1; and E1 setup request in E1. In our understanding, with the OAM effort, which is anyway needed, each node should know the mapping relationship of mapped cell ID and Uu cell ID at a certain time. Then we do not see the need to specify cell ID usage for all the cases, and RAN3 has never discussed the other cases.

There are many other appearance of cell IDs in various interfaces, whose cell ID usage has never been discussed.

There is no need to specify the cell ID usage for all the cases including the non-UE associated message like Xn setup request.

2.2 TAC usage

In this subsection, we would like to go back to the case of TAC usage when we exchange the Uu Cell ID during Xn setup/configuration update procedure. Exchanging Uu cell with a single TAC has no specification impact, but may cause some issues regarding MRL. For example, it is difficult for the NG-RAN node to judge whether it is suitable to handover UE to target cell, as the target cell may contain TACs that are not allowed. In addition, some companies have proposed to add a TAC list to exchange all the TACs within a certain Uu cell at a certain time. However, such change deviates the legacy behaviour, which may cause unexpected side effects in future.

In the last meeting, company in [1] raised that the list of served cells in NR as well as the TACs are optional, and in the scenario of NTN, we can simply not exchange the neighbour relations and leave it to OAM. We do have sympathy on this. We want to note that OAM is anyway needed to help nodes understand the serving/neighbour cells of peer nodes. Also the neighbouring relationship is defined in the neighbour relation table (NRT). In 38.300 section 15.3.3, NRT has been clearly defined.

Independent of the OAM effort, such neighbouring relationship are also exchanged via Xn with Xn setup/configuration update messages. When Xn is not deployed, however, such information is not exchanged via NG, and OAM is the only reliable way. Specifically, this means OAM can help gNB understand the PCI and Uu cell ID of the serving and neighbouring cells of peer nodes, and also the mapped cells IDs and TACs within the coverage of the serving and neighbouring Uu cells of peer nodes at a certain time. Therefore, we can simply not exchange the TAC(s) in the scenario of NTN and leave it to implementation.

OAM effort and NRT is anyway needed to help nodes understand the neighbour relationship of peer nodes.

In the scenario of NTN, we do not exchange the TAC(s) and leave it to OAM

Summary

Based on the discussion, we have following observations and proposals:

- One straightforward solution is exchanging mapped cell ID with a single TAC in non-UE associated Xn signalling.

There are many other appearance of cell IDs in various interfaces, whose cell ID usage has never been discussed.

OAM effort and NRT is anyway needed to help nodes understand the neighbour relationship of peer nodes.

- There is no need to specify the cell ID usage for all the cases including the non-UE associated message like Xn setup request.

- In the scenario of NTN, we do not exchange the TAC(s) and leave it to OAM

Reference

[1] R3-230148 Discussion on Mobility and Service Continuity enhancements for NTN, CATT