3GPP TSG-RAN WG3 #119bis-e		                                                                 R3-231787

Online 17th – 26th Apr 2023

Title: 	Discussion on SA3 LS on Mapping of F1-C IP addresses

Source: 	ZTE

Agenda item:	9.2.4

Document for:	Discussion and Approval

Introduction

During RAN3#117bis-e meeting, an LS on mapping of F1-C IP addresses  was received from SA3 [1]. In the LS, SA3 requests RAN3 to provide a suitable method for Source/Initial Donor-IAB node to know the mapping between these  F1-C IP addresses in the IAB inter-CU topology adaptation and backhaul RLF recovery procedures in order for SA3 to progress with its security work. In this paper, we will discuss the issue in the SA3 LS and provide a draft CR to TS 38.423.

Discussion

According to the SA3 LS [1], during the inter-CU topology adaptation and backhaul RLF recovery procedures, the source/initial donor needs to know the mapping between the old IP address of the F1-C interface and the new IP address. And the mapping is used to identify the security credentials that will be used to re-establish (using IKE) the IPsec connection used to protect the FI-C interface.

1 Overall description SA3 is currently trying to specify the security handling of IAB inter-CU topology adaptation and backhaul RLF recovery procedures. In this work, SA3 would need more information for defining one aspect in a way sympathetic to the current flows in TS 38.401, when using dynamic PSK. SA3 believe that the Source/Initial Donor IAB-node needs to know the mapping between the old IP address of the F1-C interface and the new IP address. This mapping is needed to identify the security credentials that will be used to re-establish (using IKE) the IPsec connection used to protect the FI-C interface. In particular this needs to be done for IPsec transport mode and also when there are one or more old/new IP addresses. SA3 requests RAN3 to provide a suitable method for Source/Initial Donor-IAB node to know the mapping between these IP addresses in order for SA3 to progress with its security work. 2 Actions To RAN3 ACTION: SA3 asks RAN3 to provide a suitable method for Source/Initial Donor-IAB node to know the mapping between the old and new IP addresses in order for SA3 to progress with its security work.

According to TS 38.401, after the migrating/recovery IAB node receives the RRCReconfiguration message which includes the new IP address anchored at the target IAB-donor-DU for the migrating/recovery IAB node, the F1-C connection between the migrating/recovery IAB node and the source IAB-donor-CU are switched to the target path using the new F1-C IP address. More specifically, the following three steps need to be performed to switch the F1-C connection to the target path:

- Step 1: If new TNL address for F1-C traffic is configured, new SCTP association needs to be established between the migrating/recovery IAB-node and the F1-terminating donor.

- Step 2: IPsec connection needs to be established between the migrating/recovery IAB-node and the F1-terminating donor to protect the F1-C interface. In this step, the F1-terminating donor needs to be able to identify the migrating/recovery IAB-node so that it can choose the appropriate stored security credentials to establish the IPsec connection.

- Step 3: after the successful SCTP/IPsec connection establishment, the migrating/recovery IAB-DU sends F1AP gNB-DU CONFIGURATION UPDATE message to the F1-terminating IAB-donor, so that the F1-C connection is associated with the new SCTP association.

- The issue identified in SA3 LS is how could the F1-terminating donor identify the migrating/recovery IAB-node during the IPsec connection establishment phase. In our view, the following two options could be considered:

- Option 1: the migrating/recovery IAB node sends the mapping info to source/initial donor

- In this option, after receiving new IP address info in the RRCreconfiguration message, the migrating/recovery IAB node sends the old and new F1-C IP address mapping info to the source/initial donor, e.g. via the F1AP gNB-DU CONFIGURATION UPDATE message. And then the source/initial donor can derive the old F1-C IP address and corresponding security credential according to the mapping info and the new IP address used for SCTP/IPsec connection establishment.

- As discussed above, after receiving new IP address info in the RRCreconfiguration message, the migrating/recovery IAB node would disconnect with the source parent node and initiate random access with the target parent node. And then the migrating/recovery IAB node would initiate SCTP/IPsec connection establishment with the initial/source donor using the new IP address(es). After the completion of SCTP/IPsec connection establishment, the migrating/recovery IAB node sends F1AP gNB-DU CONFIGURATION UPDATE message to the source/initial donor. In a sum, the F1AP gNB-DU CONFIGURATION UPDATE message is sent after the completion of SCTP/IPsec connection establishment. However, the source/initial IAB donor needs to identify the migrating/recovery IAB-node during the IPsec connection establishment phase based on the SA3 LS. So option 2 (i.e. the migrating/recovery IAB node sends the mapping info to source/initial donor) cannot meet the requirement in the SA3 LS. In this way, we suggest that option 1 is adopted and RAN3 reply to SA3 that the initial/source donor obtains the new IP address(es) for F1-C from the target/new IAB donor via XnAP message, e.g., XnAP HANDOVER REQUEST ACKNOWLEDGE and RETRIEVE UE CONTEXT REQUEST message. In this case, the corresponding XnAP messages need to be enhanced to include the new IP address(es) of the migrating/recovery IAB node for F1-C.

- Observation 1: Based on the SA3 LS, the source/initial IAB donor needs to identify the migrating/recovery IAB-node during the IPsec connection establishment phase.

- Observation 2:  During inter-CU topology adaptation and backhaul RLF recovery procedures, after receiving the new IP address(es) via RRCreconfiguration message, the F1AP gNB-DU CONFIGURATION UPDATE message is sent after the completion of SCTP/IPsec connection establishment. So the option of the migrating/recovery IAB node sends the mapping info to source/initial donor cannot solve the issue raised in the SA3 LS.

- Option 2: the initial/source donor obtains the new IP address(es) for F1-C from the target/new IAB donor via Xn

- In this option, the target/new target donor sends the new IP address(es) for F1-C to the initial/source donor explicitly via XnAP message. Besides, the new IP address(es) for F1-C are sent to the migrating/recovery IAB-node. After that, the migrating/recovery IAB-node initiates SCTP/IPsec connection establishment with the initial/source donor using the new IP address(es). In this way, the initial/source donor can identify the migrating/recovery IAB-node and derive the corresponding security credential for IPsec connection establishment based on the new IP address used for SCTP/IPsec connection establishment and the IP address for F1-C received from the target/new IAB donor.

- Proposal 1: RAN3 reply to SA3 that the initial/source donor obtains the new IP address(es) for F1-C from the target/new IAB donor via XnAP message.

- In inter-donor migration scenario, the XnAP HANDOVER REQUEST ACKNOWLEDGE could be enhanced to transfer the new IP address(es) for F1-C from the target donor to the source donor. In inter-donor backhaul RLF recovery scenario, after the recovery IAB node performs random access and sends RRCreestablishment request to the new IAB donor, the new IAB donor initiates Xn retrieve UE context procedure to the initial IAB donor. After receiving Xn retrieve UE context request message from the new IAB donor, the initial donor respond with the Xn retrieve UE context respond message, which includes the IAB node indication. In this way, the new IAB donor can determine that the RLF recovery procedure is for an IAB node. After UE context setup with the new parent node, the new IAB donor sends XnAP UE context release message to the initial IAB donor. In our view, the XnAP UE context release message could be enhanced to transfer the new IP address(es) for F1-C from the new IAB donor to the initial IAB donor.

Proposal 2: The XnAP HANDOVER REQUEST ACKNOWLEDGE and UE CONTEXT RELEASE messages are enhanced to include the new F1-C IP address(es) of the migrating/recovery IAB node.

A draft CR to TS 38.423 is provided in the Annex. And a draft reply LS to SA3 is provided in [2].

Proposal 3: RAN3 agree the CR to TS 38.423 in the Annex and the draft reply LS to SA3 provided in R3-231788.

Conclusion

In this paper, we discussed the issue in the SA3 LS and provided a draft CR to TS 38.423. The following observations and proposals are presented:

- Observation 1: Based on the SA3 LS, the source/initial IAB donor needs to identify the migrating/recovery IAB-node during the IPsec connection establishment phase.

- Observation 2:  During inter-CU topology adaptation and backhaul RLF recovery procedures, after receiving the new IP address(es) via RRCreconfiguration message, the F1AP gNB-DU CONFIGURATION UPDATE message is sent after the completion of SCTP/IPsec connection establishment. So the option of the migrating/recovery IAB node sends the mapping info to source/initial donor cannot solve the issue raised in the SA3 LS.

- Proposal 1: RAN3 reply to SA3 that the initial/source donor obtains the new IP address(es) for F1-C from the target/new IAB donor via XnAP message.

Proposal 2: The XnAP HANDOVER REQUEST ACKNOWLEDGE and UE CONTEXT RELEASE messages are enhanced to include the new F1-C IP address(es) of the migrating/recovery IAB node.

Proposal 3: RAN3 agree the CR to TS 38.423 in the Annex and the draft reply LS to SA3 provided in R3-231788.

Reference

- R3-230872 LS on Mapping of F1-C IP addresses in the IAB inter-CU topology adaptation and backhaul RLF recovery procedures (SA3, Qualcomm)

- R3-231788 [draft] Reply LS on Mapping of F1-C IP addresses

- Annex: CR to TS 38.423

3GPP TSG-RAN WG3 #119bis-e		                                                             R3-2xxxxx

Online 17th – 26th Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transferring of new F1-C IP address in inter-donor migration and backhaul RLF recovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-3-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the SA3 LS R3-230872, the source/initial donor needs to know the mapping between the old/new F1-C IP addresses of the migrating/recovery IAB node to re-establish the IPsec connection between the migrating/recovery IAB node and the source/initial donor. Based on RAN3 agreement, the XnAP HANDOVER REQUEST ACKNOWLEDGE, UE CONTEXT RELEASE messages are enhanced to include the new F1-C IP address(es) of the migrating/recovery IAB node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of the new F1-C IP address(es) of the migrating/recovery IAB node in the XnAP HANDOVER REQUEST ACKNOWLEDGE, UE CONTEXT RELEASE messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The source/initial donor is not able to re-establish the IPsec connection between the migrating/recovery IAB node and the source/initial donor during the inter-donor migration and backhaul RLF recovery procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.2, 9.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

      - 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| IAB F1-C TNL Address List |  | 1 |  | Identifies the TNL address(es) allocated for the IAB-node for F1-C traffic. |  |  |
| >IAB Allocated TNL Address Item |  | 1..<maxnoofTLAsIAB> |  |  |  |  |
| >>IAB TNL Address | M |  | 9.2.2.92 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum total no. of IPv4 address(es), IPv6 address(es) and IPv6 address prefix(es) that can be requested in one procedure execution. The value is 1024. |

-------------------------------------------Next change-------------------------------------------

      - 9.1.1.5	UE CONTEXT RELEASE

This message is sent by the target NG-RAN node to the source NG-RAN node to indicate that resources can be released.

Direction: target NG-RAN node  source NG-RAN node, M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the source NG-RAN node or for dual connectivity at the S-NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the target NG-RAN node or for dual connectivity at the M-NG-RAN node. | YES | reject |
| IAB F1-C TNL Address List |  | 1 |  | Identifies the TNL address(es) allocated for the IAB-node for F1-C traffic. |  |  |  |  |  |
| >IAB Allocated TNL Address Item |  | 1..<maxnoofTLAsIAB> |  |  |  |  |  |  |  |
| >>IAB TNL Address | M |  | 9.2.2.92 |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum total no. of IPv4 address(es), IPv6 address(es) and IPv6 address prefix(es) that can be requested in one procedure execution. The value is 1024. |

-------------------------------------------End of change-------------------------------------------