3GPP TSG-RAN WG3 Meeting #119bis-e	          R3-231789

Online, April 17–26, 2023

Agenda Item:	10.1

Source:	CMCC (Rapporteur)

Title:	Update of work plan for Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI

Document for:	Discussion and Decision

1	Introduction

A Rel-18 WI “Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI” was first approved at RAN #94 meeting and updated at RAN #96 meeting in [1].

In this contribution, we provide an updated work plan for this work item, focusing on RAN3 part.

2	Background

2.1 Scope and objectives

The objective of this work item is to specify data collection enhancement in NR for SON/MDT purpose. The specific objectives of this work item are [RAN3, RAN2]:

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces    
[RAN3]

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

If needed, co-operate with RAN1, SA2, SA5, CT4.

2.2 Time budget

RAN3 and RAN2 are the primary and secondary responsible groups of the WI respectively. The time budget that was approved at RAN#96 meeting in [2] was shown as below,

Table 1: Time budget allocation

| Date | 2022 Q3 |  | September 2022 | 2022 Q4 |  |  |  | December 2021 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Meeting | R3#117 | R2#119 | RAN#97 | R3#117bis | R2#119bis | R3#118 | R2#120 | RAN#98 |
| TUs | 1 | 1 |  | 1 | 1 | 1 | 1 |  |

| Date | 2023 Q1 |  | March 2023 | 2023 Q2 |  |  |  | June 2023 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Meeting | R3#119 | R2#121 | RAN#99 | R3#119bis | R2#121bis | R3#120 | R2#122 | RAN#100 |
| TUs | 1 | 1 |  | 1 | 0.5 | 1 | 1 |  |

| Date | 2023 Q3 |  | September 2023 | 2023 Q4 |  |  |  | December 2023 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Meeting | R3#121 | R2#123 | RAN#101 | R3#121bis | R2#123bis | R3#122 | R3#124 | RAN#102 |
| TUs | 1 | 1 |  | 1 | 1 | 1 | 1 |  |

Note: upcoming RAN plenary may make adjustments to the whole Rel-18 time frame, hence the time plan for this WI might be subject to such adjustments

2.3 Work split among WGs

It will follow the similar work split among the WGs as we did in Rel-17 SON/MDT for NR.

- RAN3: The first responsible working group of the WI and take the lead of specification enhancement of SON features and MDT for network signalling

- RAN2: Take the lead of specification of enhancement of MDT, L2 measurements, and specific L2/L3 changes needed to fulfil SON functionalities over the Uu interface

All WGs will then have to involve the other WGs on some aspects when needed, e.g.,

- RAN3 will also have to involve RAN2 on the exchange of information over the Uu interface

- RAN2 will have to trigger RAN3 on network aspects for MDT

3	Work plan

3.1 Status of the features to be developed by RAN3

| Features | Status of the Tasks |
| --- | --- |
| >Inter-RAT Successful Handover Report | SHR for intra-system inter-RAT, HO from NR to LTE will be treated first LS to RAN2 on the scope for the support of SON/MDT enhancements T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE RAN3 thinks that at least the following parameters can be useful for optimizing inter-RAT successful handover from NR to LTE. LS RAN2 to confirm and request support. Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision. - Source NR cell information - Target LTE cell information - Measurement results for source, target and neighbours - Cause to indicate which inter-RAT SHR triggering condition was met - UE location Information RAN3 thinks that all CHO related information in intra-NR SHR (e.g., time from CHO configuration to execution) is not applicable for inter-RAT SHR. LS to RAN2 on inter-RAT SHR and SPCR in Agreed WA: Support inter-RAT SHR from LTE to NR at least for T304 if no impact on LTE. RAN3 sees benefits to support inter-RAT SHR from LTE to NR for T304 trigger with no impact on LTE in Rel-18. Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18. WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell. T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE RAN3 thinks that at least the following parameters can be useful for optimizing inter-RAT successful handover from NR to LTE. LS RAN2 to confirm and request support. Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision. - Source NR cell information - Target LTE cell information - Measurement results for source, target and neighbours - Cause to indicate which inter-RAT SHR triggering condition was met - FFS on Target C-RNTI - UE location Information FFS whether the presence of Target C-RNTI IE in inter-RAT SHR is related to the decision on supporting T304 trigger CATT, E///: Has concern on Target C-RNTI, needs further check RAN3 thinks that all CHO related information in intra-NR SHR (e.g., time from CHO configuration to execution) is not applicable for inter-RAT SHR. |
| >Successful PScell change report | The following information can be included as part of SPCR (parallel discussion happening in RAN2 as well, no need to LS RAN2 if already agreed in RAN2) - Source PSCell information, in case of PSCell change/CPC - Target PSCell information - SPCR cause - Latest measurement results - Location information of the UE - Time elapsed between the CPAC execution and reception of CPAC configuration, in case of CPAC LS RAN2 to check the reporting of SPCR (delayed or immediate). Ask RAN2 whether the SPCR can be stored at the UE and sent later or is sent immediately after the successful PSCell change or addition. T310 of SCG and T312 of SCG are not considered as SPCR triggers for classic PSCell addition or CPA (since there is no source SN undergoing RLF). Send LS to RAN2 to check which node (MN or SN) retrieves the SPCR from the UE, and which node may send the configuration to the UE. SPR as abbreviation for Successful PScell Change Report feature. For SN-initiated classic PScell change the source SN node decides the T310/T312 triggers (e.g timer threshold) and the target SN node decides the T304 triggers (e.g timer threshold). For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis. For intra-SN classic PSCell change/CPC, the source SN decides SPR triggers of T310 and T312 and performs root cause analysis. If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility. If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations. |
| >MRO for CPAC | MRO for CPC and CPA based on the R17 NR-DC MRO solution Not consider too late CPA. CPA Execution to wrong PSCell will be considered, e.g. UE receives CPA configuration and CPA execution condition is satisfied, CPA execution fails or an SCG failure occurs shortly after a successful CPA execution; a suitable PSCell different with target PSCell is found based on the measurements reported from the UE. Too Late CPC Execution, Too Early CPC Execution and CPC Execution to wrong PSCell will be considered: - Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different with source PSCell is found based on the measurements reported for the UE. - Too Early CPC Execution: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; source PSCell is still the suitable PSCell based on the measurements reported from the UE. - CPC Execution to wrong PSCell: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE. For MRO for CPAC, deprioritize Case i/ii/iii/iv: - Case i: mixed scenarios of legacy PA and CPA, i.e. UE receives CPA configuration, a legacy PSCell addition is performed but fails, or a legacy PSCell addition is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell addition. - Case ii: mixed scenarios of legacy PC and CPC, i.e. UE receives CPC configuration, a legacy PSCell change is performed but fails, or a legacy PSCell change is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell change. - Case iii: MCG RLF or handover failure or CHO execution failure before CPA/CPC execution. - Case iv: CHO-CPC coexistence scenarios with low priority. Too Early CPA Execution will be considered. Naming for too early CPA execution? No need to rename. Update the definition to wrong PSCell change/addition should be splitting to sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the target node. Information available in the network nodes should not be included in the SCGFailureInformation. Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN. For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events. |
| >MRO for fast MCG recovery | - SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure) - the signalling delay is longer than the time the UE waits for the response (T316 expired); - other problem are not precluded if legacy MRO mechanism cannot cope with it. It is beneficial for the UE to report at least the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure) and also, if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx). Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure. It is beneficial for the UE to report at least PSCell where SCG failure happened, the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure, SCG was deactivated or other cases that SCG is not available), and also if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx). |
| >MRO for inter-system handover for voice fallback | Consider Case 1-2 for MRO enhancements for inter-system inter-RAT handover for voice fallback: - Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell. - Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR. WA: The RLF Report needs to indicate that the last failed inter-system inter-RAT HO was triggered due to voice fallback. FFS on whether an explicit or implicit method is needed or not. Deprioritize Case 5 for MRO enhancements for inter-system inter-RAT handover for voice fallback: - Case 5: the UE successfully performs inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, but the handover is about to failure. Deprioritize MRO enhancements for redirection for voice fallback. Introduce stage 2 descriptions of failure type definition for inter-system inter-RAT HO from NR to E-UTRA for voice fallback. Turn the WA into an agreement: The RLF Report needs to indicate that the last failed inter-system inter-RAT HO was triggered due to voice fallback. |
| >MRO for MR-DC SCG failure | Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios. Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for NE-DC SCG failure, and necessary updates can be added on top of it if needed. Take Stage 2 descriptions of PSCell change failure in TS38.300 as baseline for NE-DC SCG failure, and necessary updates can be added on top of it if needed. Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for NGEN-DC SCG failure, and necessary updates can be added on top of it if needed. Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC SCG failure, and necessary updates can be added on top of it if needed. For MRO for MR-DC SCG failure, deprioritize dual failure case (i.e. both MCG failure and SCG failure occur). |
| >RACH enhancements | Scenarios - RACH optimization for feature or feature combinations involving RACH partitioning (SDT, RedCap, Coverage Enhancement, network slicing, …) - RACH report retrieval - SN RACH report in MR-DC RACH report retrieval: - WA: RAN3 works on the network based solution for RACH report retrieval, i.e., gNB-DU indicates to gNB-CU about RACH occurrence - WA: SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN. - Send LS to RAN2 with RAN3 assumption and ask RAN2 to provide feedback on UE based solution RAN3 supports a network-based solution for RACH report retrieval over F1AP based on an indication from the gNB-DU to the gNB-CU of successful RACH procedures which are not known to the gNB-CU (e.g., when RACH is triggered due to beam failure recovery, no PUCCH resource available, UL sync issue) Define a new class-2 F1AP message (e.g., RACH INDICATION) to indicate certain RACH occurrence(s) from gNB-DU to gNB-CU SN RACH report in MR-DC RAN3 has supported EN-DC, (NG)EN-DC, and NR-DC scenarios in Rel-17. No further work will be triggered in RAN3. If RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, UE should report the PScell identity outside the RACH report to help the network forward the report to the correct node. Reply LS to RAN2 on SN RACH report in MR-DC is agreed RACH optimization for feature or feature combinations RA report is enhanced to include feature combination related information. SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN Define a new class-2 message (e.g., RACH INDICATION) over Xn so that the S-NG-RAN can inform M-NG-RAN that one or more RACH reports are available at the UE. The new Xn message should be non-UE associated. XnAP and F1AP TPs agreed Using non-UE associated sigalling for RACH Indication over F1. |
| >SON/MDT enhancements for Non-Public Networks | support of Signaling based MDT and Management based MDT for NPNs support both immediate MDT and logged MDT for NPN user consent handling for NPNs, in particular SNPNs area scope for NPNs support of NPNs in RLF Report and other UE reports used for SON and MDT RAN3 sends an LS to SA3 (cc SA5, RAN2) for user consent of NPN including the following aspects: 1: For PNI-NPN, whether existing user consent for management-based MDT (i.e., Management Based MDT PLMN List IE) can also apply for MDT in PNI-NPNs (no need of CAG-ID in user consent). 2: For SNPN, whether user consent for SNPN should include a list of SNPNs (PLMNs + NIDs) where management based MDT is allowed to take place. Introduce a CAG list for MDT area scope. LS to SA3 on user consent for NPN is agreed Wait SA3's response for NPN user consent to check if any RAN3 impact. The use cases RAN3 should support are: Use Case 1: Enhanced area scope information should allow collection of MDT measurements in specific PNI-NPNs, i.e. MDT measurements should be collected only within specific CAGs. Use Case 2: Enhanced area scope information should allow collection of MDT measurements both in specific PNI-NPNs (i.e. in specific CAGs) and in public network areas (e.g. specific PN cells, TAIs, etc.). Agree to the addition of a CAG list inside and outside the current choice structure for the MDT Area Scope. Further enhancements are FFS RAN3 to focus on the following use case for SNPN and to continue discussions on how to address MDT Area Scope for specific cells or TAs of an SNPN: Use Case 3: Enable collection of MDT measurements in the SNPN where the UE is registered. Wait for SA3 LS on MDT user consent for NPN. |
| >SON/MDT enhancements for NR-U | NR-U for MRO Add to RLF report indications concerning Measured RSSI and HOF due to consistent LBT failure. Send an LS to RAN2 requesting: to support latest Measured RSSI and Indication of HOF due to consistent LBT failure in RLF report to support “Indication of consistent LBT failure” in RA report Keep existing failure type definition and detection to indicate RLF or HOF or PSCell change failure due to consistent LBT failure. LS to RAN2 on NR-U support for MRO NR-U for MLB Exchange over Xn of Energy Detection Threshold for UL, and Channel Occupancy Time in UL is supported. Rename existing Energy Detection Threshold IE in F1AP and XnAP , and the details to be discussed in CB8. Add indications of consistent LBT failures in RA report. For XnAP, add in the RESOURCE STATUS UPDATE message a Channel Occupancy Time Percentage UL IE and an Energy Detection Threshold UL IE as sub-IEs of NR-U Channel Item IE. Corresponding TP is in R3-226040. Rename the existing Channel occupancy time percentage DL IE as Channel Occupancy Time Percentage DL both in F1AP and XnAP from R17. Reply LS in R3-226809 Agreed DU shall report the UL COT to CU in resource status report procedure when it is requested by CU, how to obtained this information by DU is up to implementation. The granularity of UL COT reporting in F1 interface should be NR-U channel level. |
| >Signalling MDT overide protection | It is RAN3’s understanding that the Rel-17 feature of signaling based logged MDT override protection applies only during Intra-NR reselection and applies to Intra-5GS (gNB–>gNB). Whether to support cross-RAT logged MDT reporting (i.e., whether the NR node needs to retrieve LTE logged MDT report) for signaling based logged MDT override protection is pending on RAN2 progress. Wait for RAN2’s progress on cross-RAT logged MDT reporting before discussing whether any enhancements are needed for NG-RAN to forward the LTE logged MDT reports to the correct TCE. RAN3 has the common understanding that according to the WI, only option 1 (inter-system inter-RAT) is included. No need to send LS to RAN2 for supporting the reverse use case. |

3.2 Way forward for RAN3 #119bis-e meeting

According to the progress of RAN3 and RAN2 in August meeting, the following topics can be focused at RAN3 #119 meeting,

- Inter-RAT SHR and SPR

- MRO enhancement

  - MRO for CPAC

  - MRO for fast MCG recovery

  - MRO for inter-system voice fallback

- RACH enhancements (not much left, could focus on RACH optimization for feature or feature combinations)

- SON/MDT enhancements for Non-Public Networks

- SON/MDT enhancements for NR-U

4	Summary

Proposal 1: It is proposed to endorse the work plan.

5	Reference

[1] 	RP-221825, “New WID on Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC”, CMCC

[2]	RP-221060, Time budget after RAN#96