3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231791

Online, April 17–26, 2023

Agenda Item:	10.2.1

Source:	CMCC

Title:	SON enhancement for Inter-RAT SHR

Document for:	Discussion and Decision

# 1 Introduction

At last RAN3 meeting, the agreement on inter-RAT SHR forwarding mechanism were achieved:

Inter-RAT SHR:

Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18.

But there is still no conclusion achieved on if and how to correlate NR SHR and LTE RLF Report.

RAN3 should specify mechanisms on how to correlate NR SHR and LTE RLF Report in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE?

Further information from the UE is FFS depending on if and how we support correlation mechanism and UE context retrieval mechanism.

Moreover, RAN2 also has the discussion on SHR content and inter-RAT report and achieved the agreements in RAN2#121.

In this contribution, we would further discuss on correlation mechanism of NR SHR and LTE RLF Report and provide some proposals on the open issues of inter-RAT SHR.

# 2 Discussion

2.1 Correlation mechanism of SHR and RLF Report

During the past few RAN3 meetings, there were some discussion on the scenario that there is a RLF shortly after a successful inter-RAT HO from NR to LTE and no conclusion achieved. In our opinion, inappropriate configuration of inter-RAT handover in source NR node may cause RLF in target LTE node shortly after the successful handover. It is beneficial to correlate NR SHR and LTE RLF Report in this case for the optimization in source node to avoid this kind of failure.

Observation 1: It is beneficial to correlate NR SHR and LTE RLF Report in the case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE for the optimization in source node.

Proposal 1: Correlating NR SHR and LTE RLF Report is supported for inter-RAT SHR from NR to LTE in Rel-18.

From the discussion in the last RAN3 meeting, we can see two potential NW-based solutions for correlation mechanism as shown below.

- Option 1: Correlation in target eNB

- Option 2: Correlation in source gNB

For Option 1, it is similar to the correlation mechanism as Rel-17 intra-NR SHR. But in the light of the agreement for inter-RAT SHR forwarding mechanism in Rel-18 achieved in RAN3#119, it seems that the target LTE node is not able to retrieve NR SHR directly. Source NR node could not know if there is a RLF in target LTE node shortly after the successful inter-RAT HO, so it needs to forward NR SHR to target LTE node every time after receiving NR SHR even if there is no RLF in target LTE node. This is certainly not consistent with the agreement for inter-RAT SHR forwarding mechanism in Rel-18 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR). Furthermore, correlation in target eNB could bring the LTE specification impact.

For Option 2, similarly, source NR node can not be able to retrieve LTE RLF Report directly. The target LTE node or the 3rd LTE node receiving RLF Report could not know if there is inter-RAT SHR generated in source NR node, it needs to forward RLF Report to source NR node every time after receiving RLF Report even if the LTE RLF after the successful inter-RAT HO for a long time. This operation no doubt also bring the LTE specification impact.

Observation 2: The existing two potential NW-based solutions for correlation mechanism will bring the impact on LTE specification.

For the case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, UE generates both NR SHR and LTE RLF report. And the UE could know for sure whether there is a RLF after a successful inter-RAT HO and the time between inter-RAT SHR generated in source NR node and RLF in target LTE node.

Observation 3: For the case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, UE generates both NR SHR and LTE RLF report. And the UE could know for sure if there is a RLF after a successful inter-RAT HO and the time between inter-RAT SHR generated in source NR node and RLF in target LTE node.

In this case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, UE could report NR SHR and LTE RLF report together to the receiving node (i.e. a NR node once UE is back to NR). Then the receiving node forwards the NR SHR and LTE RLF report together to source NR node for root cause analysis and optimization. From our point of view, this kind of UE-based correlation mechanism will only bring minimal enhancements in UE but no impact on LTE node. And the procedure shown in Figure 1 is consistent with the inter-RAT SHR forwarding mechanism in Rel-18.

UE

Source NR node

Target LTE node

Receiving node(NR)

Successful Inter-RAT HO (SHR trigger condition met)

UE stores NR SHR

RLF shortly after a successful inter-RAT HO

UE stores RLF report

NR SHR and RLF report

NR SHR and RLF report container

Correlates NR SHR and RLF report

Figure 1.  Procedure of UE-based correlation mechanism

Proposal 2: RAN3 to consider the UE-based solution for correlation mechanism of SHR and RLF Report for inter-RAT SHR from NR to LTE.

In this solution, we think a timer between NR SHR generated (SHR trigger condition is met) and RLF report generation (RLF occurs in target LTE node) on the UE side is needed. When there is a RLF after a successful inter-RAT HO from NR to LTE and the timer has not expired, UE can know the RLF happened shortly after the successful inter-RAT HO and need to correlate NR SHR and RLF report and send them to receiving node together.

Proposal 3: A timer between NR SHR generation (SHR trigger condition is met) and RLF report generation (RLF occurs in target LTE node) on the UE side is introduced for correlation mechanism between NR SHR and LTE RLF report.

Based on the solution of correlation mechanism, the following information from the UE is needed for inter-RAT SHR when HO from NR to LTE:

- A correlation indication of NR SHR and RLF report, that is used to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

- Time elapsed between NR SHR generation (SHR trigger condition met)  and RLF report generation(RLF occurs in target LTE node).

Proposal 4: UE reports the following correlation related information between NR SHR and LTE RLF report for inter-RAT HO from NR to LTE:

- A correlation indication of NR SHR and RLF report, that is used to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

- Time elapsed between NR SHR generation (SHR trigger condition met)  and RLF report generation (RLF occurs in target LTE node).

- Proposal 5: The receiving node forwards the following correlation related information between NR SHR and LTE RLF report to source NR node for root cause analysis:

- A correlation indication of NR SHR and RLF report, that is used to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

- Time elapsed between NR SHR generation (SHR trigger condition met)  and RLF report generation (RLF occurs in target LTE node).

2.2 SHR content and reporting

As we can notice, both RAN2 and RAN3 are discussing on SHR content and inter-RAT report. And RAN2 has achieved the agreements in RAN2#121 [3] as below:

Agreement:

2: For handover from NR to LTE, UE generates the NR SHR when SHR for inter-RAT mobility is triggered due to T310 or T312 trigger threshold is fulfilled.

7: For handover from NR to LTE, cross-RAT reporting is not supported, i.e., UE reports the SHR report to the network when it comes back to NR.

That means, UE encodes the inter-RAT SHR in NR format and reports the inter-RAT SHR once UE is back to NR.

Proposal 6: RAN3 to confirm UE generates SHR in NR format for inter-RAT HO from NR to LTE when inter-RAT SHR triggered due to T310 or T312 trigger threshold is fulfilled.

Proposal 7: RAN3 to confirm UE reports the SHR report to NR node for inter-RAT SHR from NR to LTE when UE comes back to NR.

For inter-RAT SHR from NR to LTE, the receiving node is also NR node just like the source NR node and can understand the NR format of inter-RAT SHR. Similar to intra-NR SHR, the receiving node can get the source NR cell ID from sourceCellInfo included in SuccessHO-Report, and there is no need to report the source cell ID outside of inter-RAT SHR from NR to LTE.

Proposal 8: The parameter of the source NR cell ID is included in sourceCellInfo IE in inter-RAT SHR from NR to LTE.

# 3 Conclusion

In this paper, we discuss correlation mechanism of NR SHR and LTE RLF Report and SHR content and reporting, and we have the following observations and proposals:

Observation 1: It is beneficial to correlate NR SHR and LTE RLF Report in the case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE for the optimization in source node.

Proposal 1: Correlating NR SHR and LTE RLF Report is supported for inter-RAT SHR from NR to LTE in Rel-18.

Observation 2: The existing two potential NW-based solutions for correlation mechanism will bring the impact on LTE specification.

Observation 3: For the case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, UE generates both NR SHR and LTE RLF report. And the UE could know for sure if there is a RLF after a successful inter-RAT HO and the time between inter-RAT SHR generated in source NR node and RLF in target LTE node.

Proposal 2: RAN3 to consider the UE-based solution for correlation mechanism of SHR and RLF Report for inter-RAT SHR from NR to LTE.

Proposal 3: A timer between NR SHR generation (SHR trigger condition is met) and RLF report generation (RLF occurs in target LTE node) on the UE side is introduced for correlation mechanism between NR SHR and LTE RLF report.

Proposal 4: UE reports the following correlation related information between NR SHR and LTE RLF report for inter-RAT HO from NR to LTE:

- A correlation indication of NR SHR and RLF report, that is used to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

- Time elapsed between NR SHR generation (SHR trigger condition met)  and RLF report generation (RLF occurs in target LTE node).

- Proposal 5: The receiving node forwards the following correlation related information between NR SHR and LTE RLF report to source NR node for root cause analysis:

- A correlation indication of NR SHR and RLF report, that is used to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

- Time elapsed between NR SHR generation (SHR trigger condition met)  and RLF report generation (RLF occurs in target LTE node).

Proposal 6: RAN3 to confirm UE generates SHR in NR format for inter-RAT HO from NR to LTE when inter-RAT SHR triggered due to T310 or T312 trigger threshold is fulfilled.

Proposal 7: RAN3 to confirm UE reports the SHR report to NR node for inter-RAT SHR from NR to LTE when UE comes back to NR.

Proposal 8: The parameter of the source NR cell ID is included in sourceCellInfo IE in inter-RAT SHR from NR to LTE.

# Reference

- R3-230363, Successful Handover Report and Successful PSCell Change Report, Qualcomm Incorporated, March 2023.

- R3-230846, Summary of Offline Discussion on CB: # 12_SONMDT1_SHRandSPR, ZTE (Moderator), March 2023.

- R2-2301906, Report from SON/MDT session, 3GPP TSG-RAN WG2 Meeting #121, March 2023.