3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231817

Electronic meeting, 17th – 26th April 2023

Agenda Item:	26.1

Source: 				LG Electronics Inc.

Title:			Consideration on RAN impact of eNPN Phase 2

Document for:		Discussion and Decision

1.	Introduction

In Rel-18, one WID item [1] was approved to support the RAN functionalities related to the SA2 WI “Enhanced support of Non-Public Networks Phase 2”.

RP-230788 [1]: … The objective of this work item is to perform normative work for RAN functionalities required to support 5GS functions of the SA2 WID “Enhanced support of Non-Public Networks Phase 2”, with the following features being of RAN relevance: - Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection [RAN3, RAN2] - Support for non-3GPP access for SNPN [RAN3] …

The SA2 already made some progress for these issues in TS 23.501 and 23.502. In this contribution, we analyze the RAN3 impact to support the RAN functionalities related to the SA2 eNPN phase 2 WI, and also provide our view on it.

2.	Discussion

2.1 Support for equivalent SNPNs

In the TS 23.501, the RAN3 impact related to the equivalent SNPNs can be highlighted as follows:

Clause 5.18.4 in TS 23.501: … In the case of handover or network controlled release to an SNPN in a shared network, the following applies: - When multiple SNPN IDs are broadcasted in a cell selected by NG-RAN, NG-RAN shall select a target SNPN, taking into account the prioritized list of SNPN IDs provided via Mobility Restriction List from AMF. - For Xn based HO procedure, Source NG-RAN indicates the selected SNPN ID to the target NG-RAN, see TS 38.300 [27]. - For N2 based HO procedure, the NG-RAN indicates a selected SNPN ID to the AMF together with the TAI sent in the HO required message. Source AMF uses the selected SNPN ID together with the TAI information supplied by the source NG-RAN to select the target AMF. The source AMF should forward the selected SNPN ID to the target AMF. The target AMF indicates the selected SNPN ID to the target NG-RAN so that the target NG-RAN can select target cells for future handover appropriately. A change in serving SNPN is indicated to the UE as part of the UE registration with the selected network. …

Clause 4.9.1.2 in TS 23.502: … The handover preparation and execution phases are performed as specified in TS 38.300 [9], in the case of handover to a shared network, source NG-RAN determines a PLMN or an SNPN to be used in the target network as specified by TS 23.501 [2]. If the serving PLMN or SNPN changes during Xn-based handover, the source NG-RAN node shall indicate to the target NG-RAN node (in the Mobility Restriction List) the selected PLMN ID or SNPN ID to be used in the target network. During Xn based handover into a shared NG-RAN node the source NG RAN node shall include the serving NID (if available) in the Mobility Restriction List to be used by the target NG-RAN node. …

Clause 4.9.1.3.1 in TS 23.502: … In the case of handover to a shared network, the source NG-RAN determines a PLMN (or PLMN ID and NID, see clause 5.30 of TS 23.501 [2]) to be used in the target network as specified by TS 23.501 [2]. The source NG-RAN shall indicate the selected PLMN ID to be used in the target network to the AMF as part of the Tracking Area sent, or the selected SNPN ID to be used in the target network to the AMF, in the HO Required message. …

According to the TS 23.501, if the UE and network supports equivalent SNPNs, the AMF may provide list of equivalent SNPNs to the NG-RAN via Mobility Restriction List. Therefore, from RAN3 point of view, the list of equivalent SNPNs should be newly defined in the Mobility Restriction List IE in NGAP. Similar to the equivalent PLMN list, this equivalent SNPN list needs to be sent to the target NG-RAN via the Mobility Restriction List IE in XnAP for subsequent mobility action.

Proposal 1: The Equivalent SNPNs IE should be included into the Mobility Restriction List IE in NGAP and XnAP.

For the Xn-based handover, the SA2 clearly mentioned in TS 23.502 that the source NG-RAN sends the selected SNPN ID to the target NG-RAN by using the serving NID IE in the Mobility Restriction List IE in XnAP. Therefore, our understanding is that there is no need to enhance the XnAP HANDOVER REQUEST message to send the SNPN ID selected by the source NG-RAN.

Observation 1: In XnAP, the SNPN ID selected by the source NG-RAN can be sent to the target NG-RAN by using the existing serving NID IE in the Mobility Restriction List IE.

During the NG-based handover, the source NG-RAN determines the SNPN ID and then indicates it to the AMF via the NGAP HANDOVER REQUIRED message. Since the Target ID IE to is already defined to identify the target for the handover into the NGAP HANDOVER REQUIRED message, the Selected SNPN ID IE can be also sent to the AMF by using this IE. Then, the AMF can provide the Selected SNPN ID IE to the target NG-RAN via Mobility Restriction List.

Proposal 2: The Selected SNPN ID IE should be included into the Target ID IE in NGAP.

2.2 Support for non-3GPP access for SNPN

According to the conclusion of TR 23.700-08, the SA2 clearly agreed to include the selected NID in the NGAP INITIAL UE MESSAGE message to support the non-3GPP access for SNPN.

Clause 8.2 in TR 23.700-08: … - When accessing SNPN services via Trusted or Untrusted non-3GPP access the UE needs to be able to construct a prioritized list of WLAN access networks by using enhanced WLAN Selection Policy (WLANSP) rules from ANDSP (currently supported only for PLMN in TS 23.402 [9]) or based on local configuration. - N3IWF and TNGF needs to be able to include the "selected NID" in the [NGAP] INITIAL UE MESSAGE, which is up to RAN WG3 to define. - Access to SNPN services via wireline access network and use of Credential Holder is to be supported by defining a new GCI including a "NID". …

Since the NPN Access Information IE to is already defined to perform access control for NPN into the NGAP INITIAL UE MESSAGE message, the Selected NID IE can be also sent to the AMF by using this IE.

Proposal 3: The Selected NID IE should be included into the NPN Access Information IE in NGAP.

According to the current NGAP for non-3GPP access (i.e., TS 29.413), the NPN Access Information IE is ignored. However, if the Proposal 3 is agreed, the NPN Access Information IE should be sent to the AMF to forward the Selected NID IE.

Proposal 4: In TS 29.413, the NPN Access Information IE of INITIAL UE MESSAGE message should be sent to the AMF.

In order to capture the above proposals, the following proposals are also suggested to RAN3:

Proposal 5: It is proposed to agree the corresponding TP in Appendix #1 as a BL CR to TS 38.413.

Proposal 6: It is proposed to agree the corresponding TP in Appendix #2 as a BL CR to TS 29.413.

Proposal 7: It is proposed to agree the corresponding TP in Appendix #3 as a BL CR to TS 38.423.

3.	Conclusion

In this contribution, we focused on the RAN3 impact on the SA2 eNPN phase 2 WI and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: The Equivalent SNPNs IE should be included into the Mobility Restriction List IE in NGAP and XnAP.

Proposal 2: The Selected SNPN ID IE should be included into the Target ID IE in NGAP.

Proposal 3: The Selected NID IE should be included into the NPN Access Information IE in NGAP.

Proposal 4: In TS 29.413, the NPN Access Information IE of INITIAL UE MESSAGE message should be sent to the AMF.

Proposal 5: It is proposed to agree the corresponding TP in Appendix #1 as a BL CR to TS 38.413.

Proposal 6: It is proposed to agree the corresponding TP in Appendix #2 as a BL CR to TS 29.413.

Proposal 7: It is proposed to agree the corresponding TP in Appendix #3 as a BL CR to TS 38.423.

4.	Reference

- RP-230788, “New WID on Further Enhancement of Private Network Support for NG-RAN,” China Telecom, March, 2023.

5. Appendix #1: TP for TS 38.413

This appendix provides the Text proposal for TS 38.413 based on the proposals of this contribution.

----------------Start of the First Change---------------

9.3.1.25	Target ID

This IE identifies the target for the handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Target ID | M |  |  |  | - |  |
| >NG-RAN |  |  |  |  |  |  |
| >>Global RAN Node ID | M |  | 9.3.1.5 |  | - |  |
| >>Selected TAI | M |  | TAI 9.3.3.11 |  | - |  |
| >>Selected NID | O |  | NID 9.3.3.42 |  |  |  |
| >E-UTRAN |  |  |  |  |  |  |
| >>Global eNB ID | M |  | Global ng-eNB ID 9.3.1.8 |  | - |  |
| >>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  | - |  |
| >Target RNC-ID |  |  |  |  | YES | reject |
| >>LAI | M |  | 9.3.3.30 |  | - |  |
| >>RNC-ID | M |  | 9.3.1.123 | This IE is ignored if the Extended RNC-ID IE is included in the Target ID IE. | - |  |
| >>Extended RNC-ID | O |  | 9.3.1.124 | The Extended RNC-ID IE is used if the RNC identity has a value larger than 4095. | - |  |
| >Target Home eNB ID |  |  |  |  | YES | reject |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the E-UTRA Cell Identity IE contained in the E-UTRA CGI IE of the cell served by the eNB. | - |  |
| >>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  | - |  |

*** skip unchanged ***

9.3.1.184	NPN Mobility Information

This IE indicates the access restrictions related to an NPN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE NPN Mobility Information | M |  |  |  |  |  |
| >SNPN Mobility Information |  |  |  |  |  |  |
| >>Serving NID | M |  | NID 9.3.3.42 |  |  |  |
| >>Equivalent SNPNs |  | 0..<maxnoofESNPNs> |  | Allowed SNPNs in addition to Serving SNPN. This list corresponds to the list of "equivalent SNPNs" as defined in TS 24.501 [26]. This list is part of the roaming restriction information. Roaming restrictions apply to SNPNs other than the Serving SNPN and Equivalent SNPNs. | YES | reject |
| >>>PLMN Identity | M |  | 9.3.3.5 |  |  |  |
| >>>NID | M |  | NID 9.3.3.42 |  |  |  |
| >PNI-NPN Mobility Information |  |  |  |  |  |  |
| >>Allowed PNI-NPN List | M |  | 9.3.3.45 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofESNPNs | Maximum no. of equivalent SNPNs. Value is 15. |

*** skip unchanged ***

9.3.3.46	NPN Access Information

This IE contains information to perform access control for NPN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE NPN Access Information | M |  |  |  |  |  |
| >PNI-NPN Access Information |  |  |  |  | - |  |
| >>Cell CAG List | M |  | 9.3.3.47 |  |  |  |
| >SNPN Access Information |  |  |  |  | YES | reject |
| >>Selected NID | M |  | NID 9.3.3.42 |  |  |  |

-----------------End of the First Change---------------

6. Appendix #2: TP for TS 29.413

This appendix provides the Text proposal for TS 29.413 based on the proposals of this contribution.

----------------Start of the First Change---------------

INITIAL UE MESSAGE message:

-	the following IEs shall be ignored, when received:

-	IAB Node Indication IE

-	CE-mode-B Support Indicator IE

-	LTE-M Indication IE

-	EDT Session IE

-	RedCap Indication IE

-	RRC Establishment Cause IE: the information given within this IE is to indicate the Establishment cause for non-3GPP access as specified in TS 24.502 [7].

-	Selected PLMN Identity IE: the information given within this IE provides the selected PLMN ID for untrusted non-3GPP access as specified in TS 23.502 [4].

-	Authenticated Indication IE: the information given within this IE between the W-AGF and the AMF is to indicate that the FN-RG has been authenticated by the wireline 5G access network as specified in TS 23.316 [6].

-	Selected PLMN Identity IE: the information given within this IE contains the PLMN Identity for wireline access as specified in TS 23.316 [6], or for trusted non-3GPP access as specified in TS 23.502 [4].

-----------------End of the First Change---------------

7. Appendix #3: TP for TS 38.423

This appendix provides the Text proposal for TS 38.423 based on the proposals of this contribution.

----------------Start of the First Change---------------

9.2.3.119	NPN Mobility Information

This information element indicates the access restrictions related to an NPN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE NPN Mobility Information | M |  |  |  |  |  |
| >SNPN Mobility Information |  |  |  |  |  |  |
| >>Serving NID | M |  | NID 9.2.2.65 |  |  |  |
| >>Equivalent SNPNs |  | 0..<maxnoofESNPNs> |  | Allowed SNPNs in addition to Serving SNPN. This list corresponds to the list of "equivalent SNPNs" as defined in TS 24.501 [30]. This list is part of the roaming restriction information. Roaming restrictions apply to SNPNs other than the Serving SNPN and Equivalent SNPNs. | YES | reject |
| >>>PLMN Identity | M |  | 9.2.2.4 |  |  |  |
| >>>NID | M |  | NID 9.2.2.65 |  |  |  |
| >PNI-NPN Mobility Information |  |  |  |  |  |  |
| >>Allowed PNI-NPN ID List | M |  | 9.2.3.120 |  |  |  |

-----------------End of the First Change---------------