3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231841

# Online, 17-26 Apri 2023

Agenda item:		12.2.1

Source:	ZTE

Title:	(TP to BLCR for 38.300) further discussion on AI/ML RAN function

Document for:		Others

Introduction

In the previous meeting, after the heated discussion, “WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.” was agreed. And there are still a list of open issue to be further discussed.

In this contribution, we elaborate our analysis on the open issues above that is the common issues across all use cases, and provide the corresponding TP thereof.

Discussion

New Procedure

After the heated discussion on whether to use single procedure or multiple procedures for various data, e.g., inputs, outputs and feedback, the WA was agreed that procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, which means there is no need to distinguish the AI/ML related data type with separate procedures.

Observation 1: There is no need to distinguish the AI/ML related data type with separate procedures.

Since it was agreed the new procedure used for transferring AI/ML related information, performance feedback and predicted information could use the same procedure. However, in the stage2 specification, it is essential to clarify that the new AI/ML procedure could be used to request the predicted information or feedback information as input, output, and feedback.

Proposal 1: The performance feedback and predicted information could be involved in the agreed new procedure.

Furthermore, based on the TR37.817 [1], the solutions involve the same flow of procedures when AI/ML training and AI/ML inference are in the NG-RAN nodes, and the same procedure is used to transfer predicted information or feedback information as input, output and feedback. Following figure could be captured in the 38.300.

Figure 1. The procedure flow for AI/ML function between NG-RAN nodes

Proposal 3: Clarify in TS38.300 that the new agreed procedure could be used to request the predicted information or feedback information as input, output and feedback.

Proposal 4: Agree the TP in the Annex.

Conclusion

We propose the following observations and proposals:

Observation 1: There is no need to distinguish the AI/ML related data type with separate procedures.

Proposal 1: The performance feedback and predicted information could be involved in the agreed new procedure.

Proposal 2: Clarify in TS38.300 that the new agreed procedure could be used to request the predicted information or feedback information as input, output and feedback.

- References

- TR37.817

Annex (TP to BLCR for 38.300)

**************** Start of Change ****************

- X.X AI/ML for NG-RAN

- X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

- X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

For the AI/ML use cases, the unified AI/ML related procedure, which is use case agnostic and data type agnostic, is used to support to transfer predicted information or performance feedback information, as shown in Figure X.

Figure X. The procedure flow for AI/ML function between NG-RAN nodes

**************** End of Change ****************