3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231740

Electronic meeting, Apr. 17th – Apr. 26th, 2023

Title: 	(TPs for SON BLCRs for TS 36.423): naming update for RA report

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	Other

# 1 Introduction

This document contains a TP for SON BLCR for 36.423 to reflect the agreements in RAN3 #119bis-e meeting.

# 2 Annex – TP to TS 36.423 for SN RA report

Start of changes

#### 9.1.2.50	ACCESS AND MOBILITY INDICATION

This message is sent by the eNB to the en-gNB to transfer access and mobility related information.

Direction: eNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR RA Report List |  | 0..1 |  |  | YES | ignore |
| >NR RA Report List Item |  | 1 .. <maxnoofRAReports> |  |  | EACH | ignore |
| >>NR RA Report Container | M |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| >>UE Assistant Identifier | O |  | en-gNB UE X2AP ID 9.2.100 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |