3GPP TSG-RAN3 Meeting #119-bis-e	R3-23xxxx

Online, 17th April - 26th April 2023

3GPP TSG-RAN WG2 Meeting #121	R2-2302042

Athens, Greece, 27th of Feb – 3rd of Mar 2023

Title:	LS on buffer level threshold-based RVQoE reporting

Response to:

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	RAN2

To:	SA4

Cc:	RAN3

Contact Person:

#### Name:	Ping-Heng Wallace Kuo

E-mail Address:	pingheng_kuo@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 has agreed to introduce buffer level threshold-based triggering of RVQoE reporting in Rel-18, and RAN2 has further discussed whether triggering of RVQoE reporting based on buffer level should be handled by APP layer or AS layer.

From RAN2 perspective, buffer level threshold-based triggering of RVQoE reporting handled by either APP layer or AS layer is feasible, but RAN2 prefers APP layer handling.

- With APP layer triggering, APP layer provides RVQoE measurements reports to AS layer when the measured buffer level satisfies a buffer level threshold, and the AS layer reports to gNB the RVQoE measurements received from the APP layer.

- With AS layer triggering, APP layer provides RVQoE measurements reports to AS layer according to the configured reporting periodicity (i.e. Rel-17 RVQoE reporting mechanism is reused), and the AS layer reports to gNB the RVQoE measurements when the received buffer level measurements satisfy a buffer level threshold.

RAN2 would like to ask SA4 to confirm APP layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18.

2. Actions:

To SA4 group.

ACTION: 	RAN2 respectfully asks SA4 to confirm APP layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18.

3. Date of Next TSG-RAN2 Meeting:

3GPP RAN2#121bis	from 2023-04-17	to 2023-04-26		Electronic Meeting

3GPP RAN2#122	from 2023-05-22	to 2023-05-26		Incheon, KR