3GPP TSG-RAN WG3 Meeting #120	R3-232753

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TPs for SON BLCRs for TS 38.300, 38.423, 38.413) SPR

Source: 	Huawei

Agenda item:	10.2.1

Document Type:	discussion

# Introduction

In last meeting, there were some progresses on the scenarios to be studied for SPR. The agreements and FFSs are showed as the following:

SPR:

WA: The triggers for SPR should be represented in terms of percentage values (similar to SHR)

If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations.

FFS Which Node decides the triggers?

Option 1: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers

Option 2：For MN-initiated classic PScell change /CPC, the source SN node decides the T310/T312 triggers

Option 3：For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs

FFS: correlation between SPR and SCGFailureInformation

In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization.

To assist in the forwarding of SPR, UE may include the following in SPR

- CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed)

- WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

Irrespective of option 1/2/3, in case SPR is collected during MN-initiated PSCell change, SPR optimizations are done in both MN and source SN

- MN is responsible to optimize PSCell change configuration and associated mobility thresholds

- Source SN is responsible to optimize lower layer issues (e.g., optimize T310/T312 timer values)

In case of SN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Change Required message  (already supported by existing specifications)

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

In case of MN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Release Request Acknowledge

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

SPCR

Proposal 11: FFS whether to also include the following in Successful PSCell Change Report:

- PCell information, in case of MN initiated PSCell change/CPC

- Information that PSCell change was MN-initiated or SN-initiated

- Time between CPC execution and report retrieval

- C-RNTI (MN, target SN, source SN)

In this paper, for the agreed scenarios and FFSs, we will give some potential solutions.

# Discussion

## FFS on the configuration of T310/312 triggers for MN-initiated classic PSCell change/CPC

In previous RAN3 meetings, there were some progresses and open issues left to be discussed.

Irrespective of option 1/2/3, in case SPR is collected during MN-initiated PSCell change, SPR optimizations are done in both MN and source SN

- MN is responsible to optimize PSCell change configuration and associated mobility thresholds

- Source SN is responsible to optimize lower layer issues (e.g., optimize T310/T312 timer values)

FFS Which Node decides the triggers?

Option 1: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers

Option 2：For MN-initiated classic PScell change /CPC, the source SN node decides the T310/T312 triggers

Option 3：For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs

For MN-initiated classic PSCell change/CPC, as the above summary, there are three options. First of all, for the MN-initiated classic PSCell chang/CPC, it is a shared understanding that the MN should decide the T310/312 triggers fully by itself (option 1) or based on the T310/312 timer values sent by the source SN. With this in mind, we could firstly exclude or down-select option 2.

Observation 1: For MN-initiated classic PSCell cheng/CPC, MN node should be responsible to decide the final T310/312 triggers.

In option 1, the MN node can decide the T310/312 triggers itself. This solution can work well if the configuration of the T310/312 triggers are relative ratio compared the related timers. In this way, the MN doesn’t need to know the exact absolute or the current value of the configured timers by the source SN. The changes of the T310/312 configurations have no impact on the MN decision on the T310/312 triggers. This will avoid unnecessary interaction between the MN and source SN.

For the configuration of the SPR triggers, there was a WA:

WA: The triggers for SPR should be represented in terms of percentage values (similar to SHR)

It is worth noting that the principle of SPR is similar to SHR and the SPR triggers are also based on the ones in SHR. Technically speaking, it is an intuitive idea to represent the triggers of SPR in term of percentage values similar to SHR.

Proposal 1: Turn the WA into agreement: The triggers for SPR should be represented in terms of percentage values (similar to SHR)

For option 3, the MN will need the inputs from source SN. The inputs can be the configured values of the T310/312. When the source SN decides to cancel or change the configurations of T310/312, the source SN node should inform MN of the updated ones. This brings additional notification from source SN node to MN node.

Proposal 2: Option 1 is preferred and option 3 is acceptable for MN to decide the T310/312 triggers and performs root cause analysis for MN-initiated classic PSCell change/CPC.

## Information in the SPR

In RAN3#118 meeting, there were some discussions on the contents of the SPR and the FFS is showed as below:

Proposal 11: FFS whether to also include the following in Successful PSCell Change Report:

- PCell information, in case of MN initiated PSCell change/CPC

- Information that PSCell change was MN-initiated or SN-initiated

- Time between CPC execution and report retrieval

- C-RNTI (MN, target SN, source SN)

In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization.

To assist in the forwarding of SPR, UE may include the following in SPR

- CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed)

- WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

Irrespective of option 1/2/3, in case SPR is collected during MN-initiated PSCell change, SPR optimizations are done in both MN and source SN

- MN is responsible to optimize PSCell change configuration and associated mobility thresholds

- Source SN is responsible to optimize lower layer issues (e.g., optimize T310/T312 timer values)

In case of SN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Change Required message (already supported by existing specifications)

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

In case of MN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Release Request Acknowledge

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

Taking into consideration the above forwarding scheme of SPR, it is noticed that the SPR will firstly be forwarded to the old MN when fetched from the UE by the reception node. Therefore, it is straightforward that the CGI of PCell should be mandatory.

Proposal 3: CGI of PCell which sent the SPR configuration is mandatory.

For the MN-initiated PSCell change, the MN performs optimization of PSCell change configuration and associated mobility threshold, while source SN is responsible to optimize T310/312 timer values. For the SN-initiated PSCell change, the MN just forwards the SPR to the source SN for root cause analysis. That is, the MN behaviors are different for MN- and SN-initiated PSCell change. It is beneficial for the MN to be aware of whether the PSCell change was initiated by the MN or source SN.

Observation 2: Indication whether the PSCell change was MN-initiated or SN-initiated is useful for MN to decide whether to perform mobility related optimization.

For CPC case, there are different fields for the execution conditions from MN and source SN. In this way, the UE can realize whether the CPC is initiated by MN or SN.

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]]

}

For classic inter-SN PSCell change, irrespective of MN-initiated or SN-initiated cases, the target SN generates the configuration container and forwards it to the MN. The MN transparently delivers it to the UE. For UE point of view, we believe it is impossible to differ the two cases.

Observation 3: The UE can know the initiating node for CPC but cannot differ for classic inter-SN PSCell change.

Another possible way is that the network can send the indication to the UE to inform the initiating node in case of classic inter-SN PSCell change. Correspondingly, if SPR is triggered, the UE includes the indication into the SPR to indicate whether the PSCell change is initiated by the MN or source SN.

However, the above discussion is a bit outside RAN3 scope. An LS to RAN2 would be beneficial.

Proposal 4: Ask RAN2 to include the indication of the initiating node in the UE report.

To assist the MN node to identify the UE, the UE should provide the related MN C-RNTI. As for the time between CPAC execution and report retrieval, together with the agreed time elapsed between the CPAC execution and reception of CPAC configuration, the network can infer when the mobility decision was made and know the corresponding mobility configuration parameters. Based on the time information and the MN C-RNTI, they can be considered for MN to uniquely identify the UE to finally find the suboptimal configurations.

In addition, to assist the MN to identify the source SN and target SN for further SPR forwarding, it is also desirable for the UE to provide the source PSCell information in case of classic PSCell change/CPC and target PSCell information.

Then, like SHR, the SPR causes and latest measurement results should be provided into SPR for network related root cause analysis.

Based on the above consideration, we believe that:

Proposal 5: For SPR, the following information should be provided:

- MN C-RNTI

- Indication whether the PSCell change was MN-initiated or SN-initiated

- Time between CPC execution and report retrieval for CPAC

- Source PSCell information in case of classic PSCell change/CPC

- Target PSCell information

- SPR cause, including T304/310/312 triggers

- Latest measurement results

RAN3 should send LS to RAN2 for the above information.

## Correlation between SPR and SCGFailureInformation

FFS: correlation between SPR and SCGFailureInformation

As the discussion in section 2.4, the UE can store the SPR at most 48 hours and report it to the serving node different from the one where the SPR is generated. On the contrary, the SCGFailureInformation message is sent to the related node immediately upon detecting of the SCG failure. Both SPR and SCGFailureInformation will be delivered to the right node responsible for the PSCell related mobility.

Observation 4: Both SPR and SCGFailureInformation are sent to the right node responsible for the PSCell related mobility.

As indicated in potential contents in SPR from the UE, there will be enough information to assist the node to look up the UE context. By comparing the UE information with the one in the SCG failure case, the node will correlate the two reports. Thus, it is the network implementation to perform the correlation of SPR and SCGFailureInformation.

Proposal 6: Correlation between SPR and SCGFailureInformation depends on network implementation.

## The forwarding of source SN mobility information from MN

It is worth noting that for CHO, the source node may update the mobility information upon completion of the CHO procedures. When the target node receives the mobility information IE more than once, it only provides the most recent one.

Observation 5: For CHO, the mobility information IE may be provided more than once and the target node only provided the most recent one.

According to latest agreements, there are two possible procedures for source SN to forward the SN mobility information to the MN.

In case of SN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Change Required message (already supported by existing specifications)

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

In case of MN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Release Request Acknowledge

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

For SN-initiated CPC, similar as CHO, it is possible that the source SN provides the target specific SN mobility information into the SN Change Required message in step 1 and provides the complete CPC related SN mobility information via SN Release Request Acknowledge message in step 11b as showed in figure 1 quoted from TS37.340.

Figure 1: Conditional SN change procedure - SN initiated

For the case that the source SN provides the SN mobility information more than once, it is reasonable for the MN to only forward the most recent one to the source SN together with SPR as CHO related mobility information forwarding scheme. It is noticed that the related scenario is missing in the existing TP [2] on TS38.423 and should be captured.

Proposal 7: Capture the following updates in to TP on TS38.423:

- The MN includes the most recent SN Mobility Information IE to the source SN in the ACCESS AND MOBILITY message if the SN mobility information IE was sent more than once.

Besides, RAN3 agreed that the MN can forward the SN mobility information IE together with SPR. The SN mobility information IE is missing in current TP [3] on TS38.413.

Proposal 8: Updates TP on TS38.413 to include SN mobility information IE in the SON Information Report IE.

# Conclusion

In this paper, we discuss miscellaneous enhancements SPR, and we have the following observations and proposals:

Observation 1: For MN-initiated classic PSCell cheng/CPC, MN node should be responsible to decide the final T310/312 triggers.

Observation 2: Indication whether the PSCell change was MN-initiated or SN-initiated is useful for MN to decide whether to perform mobility related optimization.

Observation 3: The UE can know the initiating node for CPC but cannot differ for classic inter-SN PSCell change.

Observation 4: Both SPR and SCGFailureInformation are sent to the right node responsible for the PSCell related mobility.

Proposal 1: Turn the WA into agreement: The triggers for SPR should be represented in terms of percentage values (similar to SHR)

Proposal 2: Option 1 is preferred and option 3 is acceptable for MN to decide the T310/312 triggers and performs root cause analysis for MN-initiated classic PSCell change/CPC.

Proposal 3: CGI of PCell which sent the SPR configuration is mandatory.

Proposal 4: Ask RAN2 to include the indication of the initiating node in the UE report.

Proposal 5: For SPR, the following information should be provided:

- MN C-RNTI

- Indication whether the PSCell change was MN-initiated or SN-initiated

- Time between CPC execution and report retrieval for CPAC

- Source PSCell information in case of classic PSCell change/CPC

- Target PSCell information

- SPR cause, including T304/310/312 triggers

- Latest measurement results

Proposal 6: Correlation between SPR and SCGFailureInformation depends on network implementation.

Proposal 7: Capture the following updates in to TP on TS38.423:

- The MN includes the most recent SN Mobility Information IE to the source SN in the ACCESS AND MOBILITY message if the SN mobility information IE was sent more than once.

Proposal 8: Updates TP on TS38.413 to include SN mobility information IE in the SON Information Report IE.

A draft LS also including SPR aspects and related TPs are included in the annex.

# Reference

[1] R3-232140, LS on intra-system inter-RAT SHR and SPR

[2] R3-232002, (TPs for SON BLCRs for TS 38.413 ) Inter-RAT SHR and SPR

[3] R3-232021, (TPs for SON BLCRs for TS 38.413 ) Inter-RAT SHR and SPR

# Annex_draft LS to RAN2

Title:	LS on SPR

Response to:

Release:	Rel-18

Work Item:	         NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Henrik Olofsson

E-mail Address:	henrik.olofsson@huawei.com

- Overall Description:

RAN3 has discussed the SPR and identified the potential information for the UE provide in SPR:

  - MN C-RNTI

  - Indication whether the PSCell change was MN-initiated or SN-initiated

  - Source PSCell information, in case of PSCell change/CPC

  - target PSCell information

  - SPR cause, including T304/310/312 triggers

  - Latest measurement results

  - Time elapsed between the CPAC execution and the reception of the CPAC configuration in case of CPAC

## 2. Actions:

To RAN2:

ACTION: RAN3 kindly asks RAN2 to take the above into consideration, confirm the feasibility and provide feedback if any.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #121		21 – 25 Aug 2022			Toulouse, FR

# Annex – TP to TS38.300 for SPR

Start of the change

### 15.5.2	Support for Mobility Robustness Optimization

#### 15.5.2.1	General

Mobility Robustness Optimisation aims at detecting and enabling correction of following problems:

-	Connection failure due to intra-system or inter-system mobility;

-	Inter-system Unnecessary HO (too early inter-system HO from NR to E-UTRAN with no radio link failure);

-	Inter-system HO ping-pong;

-	PSCell change failure.

-	Inter-system voice fallback failure

-	Fast MCG recovery failure

MRO provides means to distinguish the above problems from NR coverage related problems and other problems, not related to mobility.

For detection of a sub-optimal successful handovers, MRO additionally enables observability of:

-	Successful HO due to intra-NR mobility.

-	Successful HO due to inter-RAT mobility

For detection of a sub-optimal successful PScell addition/change, MRO additionally enables observability of:

-	Successful PScell addition/change

Start of next change

#### 15.5.2.X	Successful PSCell Addition/Change Report

For the analysis of successful PSCell addition/change, the UE supports Successful PSCell Addition/Change Report (SPR) based on the configuration by network, if received, and makes the SPR available to the network as specified in TS 38.331 [12].

The UE stores the SPR until the SPR is fetched by the network or for 48 hours after the SPR is recorded.

The UE only indicates SPR availability and only provides the SPR to the network if the current RPLMN is a PLMN that was present in the UE's EPLMN List or was the RPLMN at the time the SPR was generated. The UE makes the SPR available to gNB(s).

When gNB fetches the SPR from UE, it forwards the SPR to the MN that was serving the UE at the time the SPR was generated by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG. The MN may then forward the SPR to the source SN and/or target SN for the mobility event the SPR was generated for. Upon retrieval of an SPR, the receiving node may analyse whether its mobility configuration needs adjustment.

End of the change

# Annex – TP to TS38.423 for SPR

Start of the change

### 8.4.12	Access And Mobility Indication

#### 8.4.12.1	General

The purpose of the Access and Mobility Indication procedure is to transfer Access and Mobility related information between NG-RAN nodes.

#### 8.4.12.2	Successful Operation

Figure 8.2.12.2-1: Access And Mobility Indication, successful operation

NG-RAN node1 initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to NG-RAN node2.

If the Successful HO Report IE is included in the ACCESS AND MOBILITY INDICATION message, NG-RAN node2 may use it to optimize handover configurations.

If the Successful PSCell Change Report List IE is included in the ACCESS AND MOBILITY INDICATION message, NG-RAN node2 may use it to optimize PSCell change and/or PSCell addition configurations. The SN mobility information may be included if the SN mobility information IE was sent for this PSCell change from NG-RAN node2. (in case the NG-RAN node2 provided it more than once, the most recent SN Mobility Information IE is included in the ACCESS AND MOBILITY INDICATION message).

Start of next change

#### 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report List Item |  | 1 .. <maxnoofRAReports> |  |  | EACH | ignore |
| >>RA Report Container | M |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | – |  |
| >>Successful HO Report Container | M |  | OCTET STRING | SuccessHO-Report-r17 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| Successful PSCell Change Report List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report List Item |  | 1 .. <maxnoofSuccessfulPSCellChangeReports> |  |  | – |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | FFS: pending on RAN2 | – |  |
| >>SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Most recent SN Mobility Information in the PSCell of the source SN in case this message is sent from the MN to the source SN; SN Mobility Information in the PSCell of the target SN in case this message is sent from the MN to the target SN. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports, the maximum value is FFS. |

End of the change

# Annex – TP to TS38.413 for SPR

Start of the change

#### 9.3.3.35	SON Information Report

This IE contains the configuration information to be transferred.

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  | - |  |
| >Failure Indication Information |  |  |  |  |  |  |
| >>Failure Indication | M |  | 9.3.3.37 |  | - |  |
| >HO Report Information |  |  |  |  |  |  |
| >>HO Report | M |  | 9.3.3.39 |  | - |  |
| >Successful HO Report Information |  |  |  |  |  |  |
| >>Successful HO Report List |  | 1 |  |  | YES | ignore |
| >>>Successful HO Report Item |  | 1..<maxnoofSuccessfulHOReports> |  |  | - |  |
| >>>>Successful HO Report Container | M |  | OCTET STRING | SuccessHO-Report-r17 IE as defined in TS 38.331 [18]. | - |  |
| >Successful PSCell Change Report Information |  |  |  |  | YES | ignore |
| >>Successful PSCell Change Report List |  | 1 |  |  |  |  |
| >>>Successful PSCell Change Report Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>>>Successful PSCell Change Report Container | M |  | OCTET STRING | FFS pending on RAN2 | - |  |
| >>>>SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Most recent SN Mobility Information in the PSCell of the source SN in case this message is sent from the MN to the source SN; SN Mobility Information in the PSCell of the target SN in case this message is sent from the MN to the target SN. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports. Value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is FFS. |

Start of next change

### 9.4.5	Information Element Definitions

<<< Skip unchanged>>>

SONInformationReport::= CHOICE {

failureIndicationInformation	FailureIndication,

hOReportInformation				HOReport,

choice-Extensions		ProtocolIE-SingleContainer { { SONInformationReport-ExtIEs} }

}

SONInformationReport-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-SuccessfulHandoverReportList			    CRITICALITY ignore	TYPE SuccessfulHandoverReportList			PRESENCE mandatory }|

{ ID  id-SuccessfulPSCellChangeReportList		    	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformation	              PRESENCE mandatory }

,

...

}

-- --------------------------------------------------------------------

-- SON Information Report

-- --------------------------------------------------------------------

SuccessfulHandoverReportList ::= SEQUENCE (SIZE(1..maxnoofSuccessfulHOReports)) OF SuccessfulHandoverReport-Item

SuccessfulHandoverReport-Item ::= SEQUENCE {

successfulHOReportContainer			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { SuccessfulHandoverReport-Item-ExtIEs} } OPTIONAL,

...

}

SuccessfulHandoverReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SuccessfulPSCellChangeReportList ::= SEQUENCE (SIZE(1..maxnoofSuccessfulPSCellChangeReports)) OF SuccessfulPSCellChangeReport-Item

SuccessfulPSCellChangeReport-Item ::= SEQUENCE {

successfulPSCellChangeReportContainer			OCTET STRING,

snMobilityInforamtion							BIT STRING (32)						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SuccessfulPSCellChangeReport-Item-ExtIEs} } OPTIONAL,

...

}

SuccessfulPSCellChangeReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

End of the change