3GPP TSG-RAN3 Meeting # 120	R3-233072 (revision of R3-231613)

# Incheon, Korea, May 22nd - 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1159 | rev | 1 | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on F1AP for L2 U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V., ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The Uu RLC Channel ID IE defined in TS 38.473 is not consistent with the Uu-RelayRLC-ChannelID IE defined in TS 38.331. Uu RLC Channel ID is defiend as BIT STRING (SIZE(16)), while Uu-RelayRLC-ChannelID is defined as INTEGER (1..maxLC-ID), where maxLC-ID is equal to 32. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the definition of Uu RLC Channel ID in clause 9.3.1.266, changing the IE type of Uu RLC Channel ID from BIT STRING (SIZE(16)) to INTEGER (1..32). The ASN.1 IE definition of UuRLCChannelID in clause 9.4.5 is also changed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent definition of Uu RLC Channel ID will cause value overflow when downcasting a value of Uu RLC Channel ID greater than 32 to Uu-RelayRLC-ChannelID in the RRC configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.266, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

#### 9.3.1.266	Uu RLC Channel ID

This IE uniquely identifies a Uu Relay RLC channel for a L2 U2N Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uu RLC Channel ID | M |  | INTEGER (1..32) |  |

Next Change

### 9.4.5	Information Element Definitions

UuRLCChannelID ::= INTEGER (1..32)

End of Change