3GPP TSG-RAN WG3 Meeting #120 	R3-233327

Incheon, Korea, 22th May – 26th May 2023

Agenda Item:	9.2.4

Source:	CATT (moderator)

Title:	Summary of Offline Discussion on CB # 13_MultipleTrace

Document for:	Approval

# Introduction

CB: # 13_MultipleTrace

- Check the multiple MDT configuration scenario and try to achieve the group common understanding based on current RAN specs

- Only the latest MDT configuration needs to be transfer to the target node?

- LS to SA5 if agreeable

(moderator - CATT)

Summary of offline disc R3-233327

# For the Chairman’s Notes

[TBD]

# Discussion

During the online discussion, on the scenario of multiple MDT configurations, it is common understanding that multiple MDT configurations may be configured to one UE. And the assumption on the behaviour of NG-RAN node when receiving multiple MDT configurations for active UE is as below:

Case 1: Multiple immediate MDT

All of the immediate MDTs are configured to UE

Case 2: Multiple log MDT

Always replace the previous log MDT with the new received log MDT.

Case 3: Combination of immediate MDT and log MDT

All of the immediate MDT and the latest log MDT are configured to UE

Then during handover procedure, since only one MDT configuration could be transferred during handover procedure, then there would be some problem. To avoid introducing Trace Activation list in the NG/Xn AP, one solution which was proposed during the online session is that source NG-RAN node only transfer one MDT configuration during handover procedure and inform AMF failure of other MDT configurations via Trace Failure Indication message.

Question1: Do you agree with solution above?

| Company | Yes/No | Comment |
| --- | --- | --- |

Then for RRC-Inactive UE, similarly,  if  NG-RAN node receives multiple immediate MDT and log MDT, it would keep all the immediate MDTs and the latest log MDT. When UE resume to another node, the source NG-RAN node would also only transfer one MDT configuration to the target node and inform AMF failure of other MDT configurations via Trace Failure Indication message.

Question2 : Do you agree with solution above?

| Company | Yes/No | Comment |
| --- | --- | --- |

For trace session, we think the similar mechanism as immediate MDT could be adopted i.e. source NG-RAN node only transfer one Trace session during handover procedure and inform AMF failure of other trace sessions via Trace Failure Indication message

Question: Do you agree with solution above?

| Company | Yes/No | Comment |
| --- | --- | --- |

If above proposal is agreeable, then an LS should be sent to SA5 on the common understanding of RAN3.The draft LS is also uploaded in the draft folder. Please provide your comments below or directly on the draft LS

| Company | Comment |
| --- | --- |

For Trace Failure indication, it is stated that “The purpose of the Trace Failure Indication procedure is to allow the NG-RAN node to inform the AMF that a Trace Start procedure or a Deactivate Trace procedure has failed due to an interaction with a handover procedure.” If we use this procedure for another purpose, some clarification is needed. The proposed sentence is as follow:

The procedure also allows the NG-RAN node to inform the AMF that a trace session for the indicated trace reference is stop due to mobility procedure.

And we maybe also need to introduce a new cause value as” trace reference is stop due to mobility procedure”

Please provide your views on the above two changes below

| Company | Comment |
| --- | --- |

# Conclusion

[TBD]

# References