3GPP TSG-RAN WG3#120                                             	                     R3-233344

Incheon, Korea, 22– 26 May 2023

Agenda item:	21.2

Source:	Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm Incorporated

Title: 	(TP for TS 38.413) Update of CN based communication handling

Document for:	Approval

- 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: MT Communication Handling procedure. Successful operation.

The NG-RAN node initiates the procedure by sending the MT COMMUNICATION HANDLING REQUEST message to the AMF.

If the 5GC Action IE is included in the MT COMMUNICATION HANDLING REQUEST message and set to 'HLCom Activate', the AMF shall activate MT communication handling as specified in TS 23.501 [9].

If the NR Paging eDRX Cycle for RRC INACTIVE IE is included in the MT COMMUNICATION HANDLING REQUEST message, the AMF shall, if supported, take it into account when applying MT communication handling as specified in TS 38.304 [12] and TS 23.502 [10].

If the 5GC Action IE is included in the MT COMMUNICATION HANDLING REQUEST message and set to 'HLCom Deactivate', the AMF shall deactivate MT communication handling as specified in TS 23.501 [9].

- 8.3.Y	DL Data Notification

- 8.3.Y.1	General

This procedure is initiated by the AMF to indicate that there is DL data or DL signaling buffered in 5GC for the UE.

The procedure uses UE associated signalling.

- 8.3.Y.2	Successful Operation

Figure 8.5.2.2-1: DL Data Notification

The AMF initiates the DL Data Notification procedure by sending the DL DATA NOTIFICATION message to the NG-RAN node.

Upon reception of the DL DATA NOTIFICATION message, the NG-RAN node shall perform RAN Paging for the UE in RRC_INACTIVE state.

If the Paging Policy Differentiation IE is included in the DL DATA NOTIFICATION message, the NG-RAN shall, if supported, take it into account when performing RAN Paging for the UE in RRC_INACTIVE state.

- 9.2.2.X1	MT COMMUNICATION HANDLING REQUEST

This message is sent by the NG-RAN node to the AMF to request activating or deactivating CN based MT communication handling for UEs in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as specified in TS 23.501 [9].

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| CHOICE 5GC Action | M |  |  |  | YES | reject |
| >HLCom Activate |  |  |  |  |  |  |
| >>NR Paging eDRX Cycle for RRC INACTIVE | M |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) |  | - |  |
| >>NR Paging Time Window for RRC_INACTIVE | FFS |  | ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32, ...) |  | - | - |
| >>NR PTW Start (FFS) | FFS |  | FFS |  | - | - |
| > HLcom Deactivate |  |  |  |  |  |  |
| >>HLCom Deactivated | M |  | ENUMERATED (true, …) |  | - |  |
| FFS |  |  |  |  |  |  |

Editor’s Note 1: It is FFS if the NR Paging Time Window should always be provided by RAN

Editor’s Note 2: It is FFS if the NR PTW start time should be provided by RAN

Editor’s Note 3: FFS on whether to include a separate UE reachability indication in the message.

- 9.2.3.X4	DL DATA NOTIFICATION

This message is sent by the AMF to indicate the there is MT data or signaling buffered in 5GC for the UE.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| Paging Policy Differentiation | O |  | 9.3.3.X |  | YES | ignore |
| FFS |  |  |  |  |  |  |

Editor’s Note 4: It is FFS if/how DL DATA NOTIFICATION message should indicate DL signalling

- 9.3.3.X	Paging Policy Differentiation

This IE provides paging policy differentiation information for a UE in RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Priority Indicator (PPI) | M |  | INTEGER (0.. 7, …) | The Paging Policy Indicator is used for paging policy differentiation (see details in TS 23.501 [9]). |
| Allocation and Retention Priority | O |  | 9.3.1.19 |  |
| 5QI (FFS) | O |  | INTEGER (0..255, …) | indicate the 5QI associated with the PDU session |
| QoS Flow Identifier | M |  | 9.3.1.51 |  |
| PDU Session ID | M |  | 9.3.1.50 | This IE identifies a PDU Session for a UE. The definition and use of the PDU Session ID is specified in TS 23.501 |

- 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

- -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

WarningSecurityInfo,

WarningType,

WUS-Assistance-Information,

RIMInformationTransfer,

5GCAction,

PagingPolicyDifferentiation

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

id-WarningType,

id-WUS-Assistance-Information,

id-RIMInformationTransfer,

id-5GCAction,

id-PagingPolicyDifferentiation

FROM NGAP-Constants;

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

- -- MT Communication Handling Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

- -- MT COMMUNICATION HANDLING REQUEST

--

-- **************************************************************

MTCommunicationHandlingRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MTCommunicationHandlingRequestIEs} },

...

}

MTCommunicationHandlingRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-5GCAction			CRITICALITY reject	TYPE 5GCAction				PRESENCE mandatory	}|

{ FFS },

...

}

-- **************************************************************

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

- -- DL Data Notification procedure

--

-- **************************************************************

-- **************************************************************

--

- -- DL DATA NOTIFICATION

--

-- **************************************************************

DLDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {  DLDataNotificationIEs} },

...

}

DLDataNotificationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY ignore	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY ignore	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-PagingPolicyDifferentiation		CRITICALITY ignore	TYPE PagingPolicyDifferentiation		PRESENCE optional	}|

{ FFS		},

...

}

Not modified

- 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Not modified

5GCAction ::= CHOICE {

hLComActivate		HLComActivate,

hLComDeactivate		HLComDeactivate,

choice-Extensions		ProtocolIE-SingleContainer { {5GCAction-ExtIEs} }

}

5GCAction-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

HLComActivate ::= SEQUENCE {

nR-Paging-eDRX-Cycle-RRC-INACTIVE			NR-Paging-eDRX-Cycle-RRC-INACTIVE,

nR-Paging-Time-Window						NR-Paging-Time-Window-Inactive FFS,

nR-PTW-Start								NR-PTW-Start FFS,

iE-Extensions		ProtocolExtensionContainer { { HLComActivate-ExtIEs} }	OPTIONAL,

...

}

HLComActivate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HLComDeactivate ::= SEQUENCE {

hLComDeactivated	ENUMERATED {true, ...},

iE-Extensions		ProtocolExtensionContainer { { HLComDeactivate-ExtIEs} }	OPTIONAL,

...

}

HLComDeactivate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingOrigin ::= ENUMERATED {

non-3gpp,

...

}

PagingPolicyDifferentiation ::= SEQUENCE {

pagingPolicyIndicator				PagingPolicyIndicator,

allocationAndRetentionPriority		AllocationAndRetentionPriority		OPTIONAL,

qosFlowIdentifier					QosFlowIdentifier,

pDUSessionID						PDUSessionID,

fiveQI								FiveQI		OPTIONAL  FFS,

iE-Extensions		ProtocolExtensionContainer { { PagingPolicyDifferentiation-ExtIEs} }	OPTIONAL,

...

}

PagingPolicyDifferentiation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingPolicyIndicator ::= INTEGER (0..7, ...)

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

-- N

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

NR-PagingeDRXInformation ::= SEQUENCE {

nR-paging-eDRX-Cycle		NR-Paging-eDRX-Cycle,

nR-paging-Time-Window		NR-Paging-Time-Window					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NR-PagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NR-PagingeDRXInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NR-Paging-eDRX-Cycle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16,

hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NR-Paging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...,

s17, s18, s19, s20, s21, s22, s23, s24,

s25, s26, s27, s28, s29, s30, s31, s32

}

NR-Paging-Time-Window-Inactive ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

s17, s18, s19, s20, s21, s22, s23, s24,

s25, s26, s27, s28, s29, s30, s31, s32,

...

}

NR-Paging-eDRX-Cycle-RRC-INACTIVE	::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16,

hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NR-PTW-Start ::= FFS

- 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

- -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

- -- Elementary Procedures

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-MTCommunicationHandling					ProcedureCode ::= x1

id-DLDataNotification						ProcedureCode ::= x2

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

- -- IEs

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<< Skipped text unchanged >>>>>>>>>>>>>>>>>>>>

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

is-CN-MT-communication-handling							ProtocolIE-ID ::= xx1

id-5GCAction			ProtocolIE-ID ::= xx2

id-PagingPolicyDifferentiation							ProtocolIE-ID ::= xx3

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>