3GPP TSG-RAN WG3 Meeting #120	R3-233363

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TP to eNPN_Ph2 BLCR for TS 38.413) Support of the enhanced NPN phase 2

Source: 	Huawei, Nokia, Nokia Shanghai Bell, Samsung, Ericsson, ZTE, NEC

Agenda Item:	26.1

Document Type:	Other

# Introduction

This paper provides the TP to implement the following agreement for Rel-18 eNPN_Ph2.

- Agree to add Selected NID IE in the top level of the INITIAL UE MESSAGE message.

# TP to TS 38.413 (based on BLCR R3-232784)

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | ignore |
| Selected PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the selected PLMN id for the non-3GPP access. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) | Indication of an IAB node | YES | reject |
| CE-mode-B Support Indicator | O |  | 9.3.1.156 |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.157 |  | YES | ignore |
| EDT Session | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Authenticated Indication | O |  | ENUMERATED (true, …) | Indicates the FN-RG has been authenticated by the access network. | YES | ignore |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |
| Selected NID | O |  | NID 9.3.3.42 | Indicates the selected SNPN Identity together with the Selected PLMN Identity IE for the non-3GPP access. | YES | ignore |

<Unchanged Text Omitted>

- 9.4.4	PDU Definitions

<Unchanged Text Omitted>

NewSecurityContextInd,

NGRAN-CGI,

NGRAN-TNLAssociationToRemoveList,

NGRANTraceID,

NID,

NotifySourceNGRANNode,

NPN-AccessInformation,

NR-CGI,

NR-PagingeDRXInformation,

NRPPa-PDU,

<Unchanged Text Omitted>

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-WUS-Assistance-Information,

id-RIMInformationTransfer,

id-SelectedNID

<Unchanged Text Omitted>

InitialUEMessage-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-NAS-PDU									CRITICALITY reject	TYPE NAS-PDU										PRESENCE mandatory	}|

{ ID id-UserLocationInformation					CRITICALITY reject	TYPE UserLocationInformation						PRESENCE mandatory	}|

{ ID id-RRCEstablishmentCause					CRITICALITY ignore	TYPE RRCEstablishmentCause						PRESENCE mandatory	}|

{ ID id-FiveG-S-TMSI							CRITICALITY reject	TYPE FiveG-S-TMSI									PRESENCE optional		}|

{ ID id-AMFSetID								CRITICALITY ignore	TYPE AMFSetID										PRESENCE optional		}|

{ ID id-UEContextRequest						CRITICALITY ignore	TYPE UEContextRequest								PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE optional		}|

{ ID id-SourceToTarget-AMFInformationReroute	CRITICALITY ignore	TYPE SourceToTarget-AMFInformationReroute	PRESENCE optional		}|

{ ID id-SelectedPLMNIdentity					CRITICALITY ignore	TYPE PLMNIdentity									PRESENCE optional		}|

{ ID id-IABNodeIndication						CRITICALITY reject	TYPE IABNodeIndication								PRESENCE optional		}|

{ ID id-CEmodeBSupport-Indicator				CRITICALITY reject	TYPE CEmodeBSupport-Indicator					PRESENCE optional		}|

{ ID id-LTEM-Indication							CRITICALITY ignore	TYPE LTEM-Indication								PRESENCE optional		}|

{ ID id-EDT-Session								CRITICALITY ignore	TYPE EDT-Session									PRESENCE optional		}|

{ ID id-AuthenticatedIndication					CRITICALITY ignore	TYPE AuthenticatedIndication						PRESENCE optional		}|

{ ID id-NPN-AccessInformation					CRITICALITY reject	TYPE NPN-AccessInformation						PRESENCE optional		}|

{ ID id-RedCapIndication						CRITICALITY ignore	TYPE RedCapIndication								PRESENCE optional		}|

{ ID id-SelectedNID				CRITICALITY ignore	TYPE NID								         PRESENCE optional		},

...

}

<Unchanged Text Omitted>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

<Unchanged Text Omitted>

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-Selected-SNPN-Identity							ProtocolIE-ID ::= XXX

id-EquivalentSNPNsList								ProtocolIE-ID ::= YYY

id-SelectedNID										ProtocolIE-ID ::= ZZZ