3GPP TSG-RAN WG3 #121-bis                                                                               R3-235217

Xiamen, China, 9th – 13th October, 2023

Source:	CATT

Title:	(TP for BL CR to TS 38.455, 38.305) on support of LPHAP

Agenda Item:	23.2.2

Document for:	Discussion and Decision

- Introduction

In the last RAN3 meeting, on support of LPHAP, some discussion on area specific SRS configuration, a working assumption was achieved, and still some open issue are to be further discussed [1], as below:

WA: LMF provides the validity area as a list of cells, and SRS recommendation to the serving gNB. The serving gNB replies with a single SRS configuration (as in legacy positioning information exchange procedure).

The details of the SRS recommendation are FFS e.g. SRS configuration or requested SRS transmission characteristics with additional information.

FFS on whether the serving gNB includes a list of cells identifying a modification of the validity area in the response message to the LMF.

In the RAN1 LS [2], they are considering the candidate values larger than 10240 ms for PRS and/or SRS periodicity, e.g., 20480 ms, details could be found below:

Agreement

From RAN1 perspective, candidate values larger than 10240 ms for PRS and/or SRS periodicity, e.g., 20480 ms, can be introduced.

- FFS: specification impact on PRS/SRS configuration.

- Send LS to RAN2 asking them to work on the higher layer signalling details (e.g., specific values of periodicity, hyper SFN information in the configuration, etc.)

In this contribution, we will further discuss on support of cross-cell SRS configuration, and the potential specification impact on PRS/SRS configuration to align with the RAN1 agreement. Base on the discussion, we will provide the observations and proposals accordingly.

- Discussion

2.1 Cross cell SRS configuration

### 2.1.1 Area specific SRS configuration

In the last RAN3 meeting, a working assumption was achieved:

- WA: LMF provides the validity area as a list of cells, and SRS recommendation to the serving gNB. The serving gNB replies with a single SRS configuration (as in legacy positioning information exchange procedure).

However, on the details of the SRS recommendation, it’s open for further discussion.

- The details of the SRS recommendation are FFS e.g. SRS configuration or requested SRS transmission characteristics with additional information.

From the previous discussion, there’re two options on the table:

- Option 1: The LMF reuses the exiting SRS characteristics to the serving gNB, and the serving gNB allocates the SRS configuration accordingly, just as the legacy Positioning Information Request procedure.

- Option 2: LMF provides (available) SRS configuration(s) to the serving gNB, the serving gNB further determines the SRS configuration and response to LMF.

For the option 1, the legacy procedure on SRS allocation are greatly reused, the serving gNB just allocate the SRS configuration based on the SRS characteristics received in Positioning Information Request message, the SRS configuration is provided to the UE and provided to the LMF.

Observation 1: If SRS recommendation equal to SRS characteristics, the gNB just decides the SRS configuration according to the SRS characteristics provided by the LMF, the legacy procedures on SRS allocation could be greatly reused.

For the option 2, how the LMF provides the SRS configuration(s) and the validity area, it may require the LMF to check with gNBs on the available SRS configurations, and sorts out of the available SRS configuration(s) among some gNBs or cells, and provides them to the serving gNB. More NRPPa exchange seems necessary, and we should consider the issue whether the available SRS configuration of each gNB/cell is changed or not.

Observation 2: If SRS recommendation equal to SRS configuration(s), which requires LMF to check with each of the gNBs the available SRS configuration(s) before cross-cell SRS configuration.

Some companies may argue that, even for the option 1, the SRS configuration decided by the serving gNB may be not available in the other cells within the validity area, but the serving gNB does not know about that, thus, coordination between LMF and the other gNBs is also needed to check whether the allocated SRS configuration is available or not.

Our understanding is, LMF should have the overall picture on the (positioning) SRS allocations for each of the gNBs, thus, the SRS confliction issue is not so essential, just similar to the legacy positioning procedures, where the LMF provides the SRS configuration allocated by the serving gNB towards the neighbor gNBs/TRPs and request for the SRS measurement, no need to coordinate with each of the gNBs.

With above understanding, the option 1 works, and not necessary to do the additional coordination between LMF and the other gNBs on the availability of the SRS configuration.

Proposal 1: On allocation of cross-cell SRS configuration, it’s preferred to reuse the legacy procedure, i.e. the serving gNB allocate the SRS configuration according to the SRS characteristics received from LMF. Not necessary to do the additional coordination between LMF and the other gNBs on the availability of the SRS configuration.

For the validity area, it’s agreed that LMF provides the validity area to the serving gNB, however, whether the gNB could further determine the validity area is not decided yet, as below:

- FFS on whether the serving gNB includes a list of cells identifying a modification of the validity area in the response message to the LMF.

Base on previous discussion, two potential options are provided below:

- Option 1: the serving gNB just follow and use the validity area provided by the LMF.

- Option 2: the serving gNB may further down-scope the recommended validity area provided by the LMF.

We understand it’s possible and more flexible to allow the serving gNB to do the further decision, e.g. considering the Xn reachability between the cells of gNBs with the recommended validity area. To make life easier, we prefer to go for the option 1 directly, which means the validity area provided by the LMF should be respected, no change from the gNB is allowed, how the LMF decide the validity area is up to implementation.

Proposal 2: The validity area is decided by the LMF, which should not be modified by the serving gNB.

Since the cross-cell SRS configuration is to be used in the validity area, a list of cells within or across the gNBs, the allocated SRS configuration should be reserved in the gNBs/cells within the validity area. The LMF should be able to notify the gNBs in the validity area to reserve the allocated SRS resources in a list of cells.

Proposal 3: For cross-cell SRS configuration, the LMF should notify the gNBs in the validity area to reserve the allocated SRS configuration in a list of cells.

Similarly, the reservation cancellation procedure should also be supported. But the SRS reservation is UE specific procedure or non-UE specific procedure, and how to identify and cancel the reservation should be further discussed.  Maybe some new identity, e.g. reservation id need to be introduced. For the signalling design, either way is fine, to reuse the Measurement procedure by extending the “TRP Measurement Type”, or define new NRPPa procedure(s),  details could be further discussed when the above issues are addressed.

Proposal 4: For cross-cell SRS configuration, the LMF should be able to notify the gNBs in the validity area the reservation is cancelled.

Proposal 5: the stage 3 details on SRS reservation to be further discussed, e.g. UE specific or non-UE specific signalling to be used, and how to identify the reservation should be further discussed.

For cross-cell SRS configuration, another issue is whether and how to define the validity time of the cross-cell SRS configuration, we understand this is needed, as the reserved SRS resource in NG-RAN nodes could not be there forever. However, the details is pending to RAN2 progress.

Proposal 6: Validity time of the cross-cell SRS configuration is pending to RAN2.

According to the discussion above, the overall procedure for cross-cell configuration and reservation could be illustrated with the figure below:

Figure 1: Overall procedure for cross-cell SRS configuration

Corresponding text proposal for the stage 2 is provided in section 5.

Proposal 7: Discuss and agree the overall procedures for cross-cell SRS configuration and reservation.

### 2.1.2 SRS configuration update when UE moves out of validity area

Some agreements made in RAN2, as below:

Agreements:

The SRS validity area configuration contains a list of cells in which it is valid.  FFS validity timer or if we would depend only on explicit release by the network.

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

Observation 3: RAN2 assumes when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

Following the RAN2 agreements, UE will request for the SRS resources from the new gNB (out of validity area) directly. The most straightforward way is to retrieve the full UE context from the last serving gNB, then the new gNB performs SRS resource allocation according to the SRS characteristics, the new allocated SRS configuration is provided to the UE via RRCRelease message.

To support that, a specific cause value or indicator in Xn Retrieve UE Context Request is needed, with that, the last serving gNB could decide whether to relocate the UE context.

Proposal 8: To support UE moves out of the validity area, a specific cause value or indicator in Xn Retrieve UE Context Request is needed, to indicate the last serving gNB to perform the UE context relocation.

From current spec, the Positioning Information (i.e. SRS characteristics, Routing ID, etc.) may be included in the Retrieve UE Context Response message, we should consider whether it could be used in this case. As the area-specific SRS is configured for a validity area, and UE moves out of the validity area, thus, it’s not appropriate to continue using the old SRS characteristics in the new gNB. The proper way is to involve LMF in the new SRS allocation procedure.

Proposal 9: the old SRS characteristics from the last serving gNB should not be used in the receiving gNB out of the validity area, LMF should be involved in the new SRS allocation procedure.

How to involve LMF, the straightforward way is to notify LMF a new Cell ID in Positioning Information Update message from the last serving gNB, then LMF knows the UE moves out of the validity area by comparing the received Cell ID and the validity area configured for the UE. For the case where the UE moves out the validity area, LMF may allocate the new area-specific SRS for the UE, may allocate the SRS for the UE as legacy.

Proposal 10: the last serving gNB could notify LMF the UE moves out of the validity area by sending the Positioning Information Update message with a new Cell ID where the UE request for SRS configuration.

Proposal 11: LMF could decide to allocate new area-specific SRS or the legacy SRS configuration for the UE.

As above, the overall procedures when UE moves out of SRS validity area could be illustrate with the figure below:

Figure 2: Overall procedure UE moves out of SRS validity area

Above all, we should discuss the overall procedures on handling of UE moves out of validity area, as shown in the Annex.

Proposal 12: Discuss and agree the overall procedure on handling of UE moves out of validity area.

2.2 Larger values for PRS/SRS periodicity

In the RAN1 LS [2], they are considering the candidate values larger than 10240 ms for PRS and/or SRS periodicity, e.g., 20480 ms, details could be found below:

Agreement

From RAN1 perspective, candidate values larger than 10240 ms for PRS and/or SRS periodicity, e.g., 20480 ms, can be introduced.

- FFS: specification impact on PRS/SRS configuration.

- Send LS to RAN2 asking them to work on the higher layer signalling details (e.g., specific values of periodicity, hyper SFN information in the configuration, etc.)

From RAN3 perspective, we just need to align with RAN1 agreement to extend the periodicity for PRS and SRS. Double checked the NRPPa, and we see the value range of the DL PRS periodicity already covers 20480,40960,81920, which could meet the RAN1 requirement. For the SRS, the maximum value of SRS periodicity is 10240 ms, which should be extended.

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ...) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, … 20480) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

### 9.2.61	Requested DL PRS Transmission Characteristics

This IE contains the requested PRS configuration for transmission by the LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested DL-PRS Resource Set List |  | 1 |  |  |
| >Requested DL-PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  |
| >>PRS bandwidth | O |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >>Comb Size | O |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >>Resource Set Periodicity | O |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…) |  |
| >>Resource Repetition Factor | O |  | ENUMERATED(rf1,rf2,rf4,rf6,rf8,rf16,rf32,…) |  |
| >>Resource Number of Symbols | O |  | ENUMERATED(n2,n4,n6,n12,…) |  |
| >>Requested DL-PRS Resource List | O |  | 9.2.62 |  |
| >>Resource Set Start Time and Duration | O |  | Start Time and Duration 9.2.63 | This IE is ignored if the Start Time and Duration IE is present |
| Number of Frequency Layers | O |  | INTEGER(1..4) |  |
| Start Time and Duration | O |  | 9.2.63 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |

PeriodicityItem ::= ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot625, ms1, ms1dot25, ms2, ms2dot5, ms4dot, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80m, ms160, ms320, ms640m, ms1280, ms2560, ms5120, ms10240, ..., ms20480}

Proposal 13: Following the RAN1 agreement, add new value 20480 ms for the PeriodicitySRS.

- Conclusion

Based on the discussion in section 2, the following observations and proposals are provided:

Observation 1: If SRS recommendation equal to SRS characteristics, the gNB just decides the SRS configuration according to the SRS characteristics provided by the LMF, the legacy procedures on SRS allocation could be greatly reused.

Observation 2: If SRS recommendation equal to SRS configuration(s), which requires LMF to check with each of the gNBs the available SRS configuration(s) before cross-cell SRS configuration.

Proposal 1: On allocation of cross-cell SRS configuration, it’s preferred to reuse the legacy procedure, i.e. the serving gNB allocate the SRS configuration according to the SRS characteristics received from LMF. Not necessary to do the additional coordination between LMF and the other gNBs on the availability of the SRS configuration.

Proposal 2: The validity area is decided by the LMF, which should not be modified by the serving gNB.

Proposal 3: For cross-cell SRS configuration, the LMF should notify the gNBs in the validity area to reserve the allocated SRS configuration in a list of cells.

Proposal 4: For cross-cell SRS configuration, the LMF should be able to notify the gNBs in the validity area the reservation is cancelled.

Proposal 5: the stage 3 details on SRS reservation to be further discussed, e.g. UE specific or non-UE specific signalling to be used, and how to identify the reservation should be further discussed.

Proposal 6: Validity time of the cross-cell SRS configuration is pending to RAN2.

Proposal 7: Discuss and agree the overall procedures for cross-cell SRS configuration and reservation.

Observation 3: RAN2 assumes when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

Proposal 8: To support UE moves out of the validity area, a specific cause value or indicator in Xn Retrieve UE Context Request is needed, to indicate the last serving gNB to perform the UE context relocation.

Proposal 9: the old SRS characteristics from the last serving gNB should not be used in the receiving gNB out of the validity area, LMF should be involved in the new SRS allocation procedure.

Proposal 10: the last serving gNB could notify LMF the UE moves out of the validity area by sending the Positioning Information Update message with a new Cell ID where the UE request for SRS configuration.

Proposal 11: LMF could decide to allocate new area-specific SRS or the legacy SRS configuration for the UE.

Proposal 12: Discuss and agree the overall procedure on handling of UE moves out of validity area.

Proposal 13: Following the RAN1 agreement, add new value 20480 ms for the PeriodicitySRS.

- Reference

- RAN3#121 Chair Notes

- R3-235007 LS on the longer PRS/SRS periodicity for LPHAP

- Annex

5.1 TP for NRPPa BL CR

#### 9.1.1.13	POSITIONING INFORMATION UPDATE

This message is sent by the NG-RAN node to indicate that a change in the SRS configuration or UE Tx TEG association has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| SRS Transmission Status | O |  | ENUMERATED (stopped, ...) |  | YES | ignore |
| NR CGI | O |  | 9.2.9 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ...) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …, 20480) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request Information[FFS] | O |  | ENUMERATED(true, …) [FFS] |  | YES | ignore |

Editor Notes: the detail information in the Bandwidth Aggregation Request Information is FFS.

Editor Notes: It is FFS if the Measurement procedure need to indicate explicit SRS aggregation ID

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- P

<<<<<<<<< unchanged texts omitted >>>>>>>>>>>

PosSRSResourceID-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF SRSPosResourceID

PosSRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourceSets)) OF PosSRSResourceSet-Item

PosSRSResourceIDPerSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourcePerSet)) OF SRSPosResourceID

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceIDPerSet-List		PosSRSResourceIDPerSet-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-PosSRSResource-Aggregation-ID	CRITICALITY ignore EXTENSION PosSRSResource-Aggregation-ID		PRESENCE optional },

...

}

PosSRSResource-Aggregation-ID ::= INTEGER(0..16) --FFS

PosResourceSetType  ::= CHOICE {

periodic			PosResourceSetTypePeriodic,

semi-persistent		PosResourceSetTypeSemi-persistent,

aperiodic			PosResourceSetTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ PosResourceSetType-ExtIEs }}

}

PosResourceSetType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

PeriodicityItem ::= ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot625, ms1, ms1dot25, ms2, ms2dot5, ms4dot, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80m, ms160, ms320, ms640m, ms1280, ms2560, ms5120, ms10240, ... , ms20480}

<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

  - TP for TS 38.305 BL CR

x.y Procedures for Cross-Cell SRS configuration

x.y.1 General

Editor’s Note: To be updated by RAN2.

x.y.2 Area specific SRS resource allocation procedure

Figure x.y.2-1 shows the area specific SRS resource allocation procedure.

Figure x.y.2-1: Overall procedure for area specific SRS resource allocation

- LMF initiates NRPPa Positioning Information Request towards the serving gNB, including a list of cells as the validity area.

- The serving gNB decides the area specific SRS configuration, and it moves the UE to RRC_INACTIVE by sending RRCRelease message, which includes the area specific SRS configuration, and corresponding validity area.

- The serving gNB responses the NRPPa Positioning Information Response to the LMF.

- The LMF reserves the SRS configuration in the gNBs and cells included in the validity area.

Editor’s Note: the detail design for the SRS reservation is FFS.

x.y.3 Area specific SRS resource update procedure

Figure x.y.3-1 shows the SRS validity area update procedure.

Figure x.y.3-1: Overall procedure area specific SRS resource update

- The UE in RRC_INACTIVE is configured with an area specific SRS configuration.

- When UE moves out of the SRS validity area, it request for new SRS resource.

- The gNB which receives the request from UE triggers the Retrieve UE Context procedure, and indicate new SRS is requested.

- The last serving gNB notifies the LMF the UE is moves out of the validity area by providing the new Cell ID the UE resumes.

- The last serving gNB relocate the full UE context to the new gNB.

- The receiving gNB triggers the Path Switch Request procedure towards the AMF.

- AMF response with the Path Switch Request Acknowledge.

- The LMF indicate the receiving gNB to allocate new SRS resource for the UE, new validity area may be included.

- The receiving gNB moves the UE to RRC_INACTIVE by sending RRCRelease message, which includes the new SRS configuration, and probably the new validity area.

- The receiving gNB indicates the last serving gNB to delete the UE context.