3GPP TSG-RAN WG3 Meeting #121bis	R3-235260

Xiamen, CN, 09-13 Oct, 2023

Agenda Item:	10.2.2

Source:	Huawei

Title:	(TPs for SON BLCRs for TS 38.423) MRO on fast MCG recovery

Document for:	Other

# 1	Introduction

MRO for fast MCG recovery related issues were discussed during RAN3 recent meetings and the following agreements were achieved:

MRO for the fast MCG recovery:

RAN3#121:

- Support of pre-R18 UEs?

RAN3#120:

Agree to define case c in R18. Solution can be further discussed.

RAN3#119bis-e:

Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed.

Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running).

RAN3#119:

Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure.

It is beneficial for the UE to report at least PSCell where SCG failure happened, the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure, SCG was deactivated or other cases that SCG is not available), and also if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

RAN3#118:

It is beneficial for the UE to report at least the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure) and also, if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

RAN3#117:

SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure)

the signalling delay is longer than the time the UE waits for the response (T316 expired);

other problems are not precluded if legacy MRO mechanism cannot cope with it.

In this paper, we address different aspects concerning MRO for fast MCG recovery.

# 2	Discussion

## 2.1		MRO for fast MCG recovery

Currently, the following cases for fast MCG recovery failure has been addressed both in RAN2 and RAN3:

- Case a: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running)

- Case f1: SCG fails or is deactivated yet before the UE sends the MCGFailureInformation

- Case b: the signalling delay is longer than the time the UE waits for the response (T316 expired)

- Case c: Fast recovery near failure case, i.e. UE receives the response message from MN via SN while T316 is running which almost expires but not yet.

RAN3 also reached an agreement for the required reporting from the UE

- It is beneficial for the UE to report at least the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure) and also, if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

The topic is also discussed in RAN2#123, and the following agreements have been reached:

Agreements:

1	UE reports the elapsed T316 between the transmission of MCGFailureInformation and receiving RRC reconfiguration or RRC release message.

2	No T316 related triggering threshold is introduced.

3	Reuse existing RLF report to capture fast MCG recovery related information.

RAN2#123 has agreed to resue RLF report without T316 related triggering condition to log fast MCG recovery related imformation. The receiving node fetching the RLF report needs to identify the previously agreed scenarios for further forwarding of RLF report. If it is the failure case of MCG recovery, the receiving node could decide to forward to the failed MN and/or failed SN according to the agreed the specific cause and failure type. For identifying the corresponding failed SN, the UE shall indicate the PSCell ID in which the MCG recovery was initiated in the RLF report. If it is a successful MCG recovery, the receiving node needs to differ the successful case from the failure case, even might need to further distinguish the near failure case from the plain successful case. The UE could not indicate whether the MCG recovery procedure succeeds in the RLF report. If it is a plain successful case, the recieving node only forwards the RLF report to the failed MN. If it is a near failure case, the receiving node might need to identify whether it is the failed MN and/or the SN causing the underlying issues. Therefore, we propose:

- In order for the receiving node to identify the node to forward the RLF report to, the UE needs to indicate the identity of the SN where recovery was initated and the possibility to indicate that the recovery was succesful. RAN2 is informed about this with a new LS.

Further, based on the above analysis, RAN3 need to discuss how to forward the information. Since we reuse the RLF reporting, it seems most straightforward to use forwarding between the last serving MN and the SN (i.e. not to send from the node receiving the RLF report directly to the corresponding SN) and reuse ACCESS AND MOBILITY INDICATION message.

Reuse the ACCESS AND MOBILITY INDICATION message to forward the RLF report from the last serving MN to the corresponding SN.

Besides, it is beneficial for the UE to indicate the Cell the UE executes HO towards or RRC Setup towards, according to the RAR in the successful MCG recovery. So the failed MN could optimize the mobility parameter, e.g handover the UE to the cell timely to avoid the MCG RLF.

It is beneficial for the UE to indicate the Cell UE executes HO towards or RRC Setup towards.

A FFS from last meeting is support of MRO for fast MCG recovery for pre-Rel.18 UEs, which is a network-based solution instead of UE reporting solution, which is intended for the timely release of SCG radio resource:

So far, detection of the failure events for MRO for the fast MCG recovery relies on the report from the UE. However, this reporting may be provided from UEs that are Rel.18 only. We consider that if the SN is aware of the T316 it can, based on the reception of the MCGFailureInformation detect at least some of the Rel.18 MRO events also for pre-Rel.18 UE (e.g. scenario ‘b’, where the signalling takes longer than the timer).

However, the network-based solution for support of pre-Rel.18 UEs has its limitations:

- It requires that SN receives the MCGFailureInformation successfully.

- The T316 timer maintained at the SN side is inaccurate for the purpose of estimating the elapsed T316 at the UE side.

In our understanding, for case a and case b, since the transmission of MCGFailureInformation might take so long, the SN receives the MCGFailureInformation and just starts the T316 timer, while the T316 timer at the UE side has already been running for a while, even close to expiry. Therefore, having SN maintain T316 timer is not practical since the gap between when to start T316 timer at the UE side and SN side could be huge.

Observation 1: The network-based solution for support of pre-Rel.18 UEs has its limitations:

- It requires that SN receives the MCGFailureInformation successfully.

- The T316 timer maintained at the SN side is inaccurate for the purpose of estimating the elapsed T316 at the UE side.

The main benefit of the network-based solution seems to be that this would enable a limited support for pre-Rel-18 UEs. This could be interesting to explore. We would discuss further whether the benefits are big enough to motivate the inclusion of this solution as well.

- The solution for pre Rel-18 UEs is suitable TEI-Rel 17 instead of SON topic.

# 3	Conclusion

This document proposes:

- In order for the receiving node to identify the node to forward the RLF report to, the UE needs to indicate the identity of the SN where recovery was initated and the possibility to indicate that the recovery was succesful RAN2 is informed about this with a new LS.

Reuse the ACCESS AND MOBILITY INDICATION message to forward the RLF report from the last serving MN to the corresponding SN.

It is beneficial for the UE to indicate the Cell UE executes HO towards or RRC Setup towards.

Observation 1: The network-based solution for support of pre-Rel.18 UEs has its limitations:

- It requires that SN receives the MCGFailureInformation successfully.

- The T316 timer maintained at the SN side is inaccurate for the purpose of estimating the elapsed T316 at the UE side.

The solution for pre Rel-18 UEs is suitable TEI-Rel 17 instead of SON topic.

# Annex 1

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report List Item |  | 1 .. < maxnoofRAReports > |  |  | EACH | ignore |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| >>PSCell List Container | O |  | OCTET STRING | [FFS – Name to be decided by RAN2] IE as defined in subclause [FFS – to be decided by RAN2] in TS 36.331 [14]. | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | – |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |

| Successful PSCell Change Report List |  |  |  |  |  |  |  | 0..1 |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| >Successful PSCell Change Report List Item |  | 1 .. <maxnoofSuccessfulPSCellChangeReports> |  |  | – |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | FFS: pending on RAN2 | – |  |
| >>SN Mobility Information |  |  | BIT STRING (SIZE (32)) | Mobility Information in the PSCell of the source SN in case this message is sent from the MN to the source SN; Mobility Information in the PSCell of the target SN in case this message is sent from the MN to the target SN. | – |  |
| RLF Report List |  | 0..1 |  |  | YES | ignore |
| >RLF Report List Item |  | 1 .. <maxnoofRLFReports> |  |  | – |  |
| >>UE RLF Report Container | M |  | 9.2.2.59 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports, the maximum value is FFS. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RAReport,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

HashedUEIdentityIndexValue,

RaReportIndicationList,

SuccessfulPSCellChangeReportInformation,

CPACConfiguration,

RLFReportInformation

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RAReport,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-HashedUEIdentityIndexValue,

id-RaReportIndicationList,

id-SuccessfulPSCellChangeReportInformation,

id-CPACConfiguration,

id-RLFReportInformation,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- ACCESS AND MOBILITY INDICATION

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ AccessAndMobilityIndication-IEs}},

...

}

AccessAndMobilityIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RAReport			CRITICALITY ignore		TYPE RAReport					PRESENCE optional}|

{ ID id-SuccessfulHOReportInformation		CRITICALITY ignore		TYPE SuccessfulHOReportInformation			PRESENCE optional}|

{ ID id-SuccessfulPSCellChangeReportInformation		CRITICALITY ignore		TYPE SuccessfulPSCellChangeReportInformation			PRESENCE optional}|

{ ID id-RLFReportInformation		CRITICALITY ignore		TYPE RLFReportInformation			PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofRBsetsPerCell1,

maxnoofTargetSNsMinusOne,

maxnoofThresholdsForExcessPacketDelay,

maxnoofSuccessfulPSCellChangeReports,

maxnoofRAReportIndications,

maxnoofPSCellsinCPAC,

maxnoofCPACexecutioncond,

maxnoofRLFReports

FROM XnAP-Constants

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- R

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

RLCDuplicationStateList	::= 	SEQUENCE (SIZE(1..maxnoofRLCDuplicationstate)) OF RLCDuplicationState-Item

RLCDuplicationState-Item ::=	SEQUENCE {

duplicationState			ENUMERATED {active,inactive, ...},

iE-Extensions	ProtocolExtensionContainer { {RLCDuplicationState-ItemExtIEs } }	OPTIONAL,

...

}

RLCDuplicationState-ItemExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLFReportInformation	::= SEQUENCE (SIZE(1.. maxnoofRLFReports)) OF RLFReportList-Item

RLFReportList-Item	::= SEQUENCE {

UERLFReport		UERLFReportContainer,

iE-Extensions						ProtocolExtensionContainer { { RLFReportList-Item-ExtIEs} }	OPTIONAL,

...

}

RLFReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

maxnoofNSAGs								INTEGER ::= 256

maxnoofTargetSNsMinusOne					INTEGER ::= 7

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofSuccessfulPSCellChangeReports		INTEGER ::= FFS

maxnoofRAReportIndications					INTEGER ::= FFS

maxnoofPSCellsinCPAC						INTEGER ::= 8

maxnoofCPACexecutioncond					INTEGER ::= 2

maxnoofRLFReports					INTEGER ::= 64

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-ChannelOccupancyTimePercentageUL																	ProtocolIE-ID ::= xx1

id-EnergyDetectionThresholdUL																		ProtocolIE-ID ::= xx2

id-SuccessfulPSCellChangeReportInformation															ProtocolIE-ID ::= xx3

id-PSCellListContainer																				ProtocolIE-ID ::= xx4

id-RadioResourceStatusNR-U																			ProtocolIE-ID ::= xx5

id-CPACConfiguration																				ProtocolIE-ID ::= xxx

id-RLFReportInformation																				ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex2	LS to RAN2 on MRO for fast MCG recovery

Title:	[draft] LS on MRO for fast MCG recovery

Response to:

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:

Contact person:	Henrik Olofsson

henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 has previously sent an LS on information from the UE that is beneficial for the MCG recovery case in R3-230908

RAN3 has identified additional information that is beneficial for the UE to report:

- Whether the MCG recovery is successful or not

- The identity of the cell in the SN where recovery is initiated.

- The identity of the cell the UE executes HO towards or RRC Setup towards at completion of the recovery.

RAN3 therefore kindly asks RAN2 to clarify the above agreement.

# 2	Actions

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above into account and provide feedback if necessary.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#122  	                            2023-11-13 -  2023-11-17 		         Chicago, US