3GPP TSG-RAN WG3 Meeting 121bis     	R3-235292

Xiamen, China, Oct 9-13, 2023

Agenda Item:	10.2.2

Source:	NEC

Title:	Discussion on MRO on fast MCG recovery

Document for:	Decision

1	Introduction

During the previous meeting, it has been agreed that

MRO for the fast MCG recovery: Case a: SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure) Case b: the signalling delay is longer than the time the UE waits for the response (T316 expired); other problems are not precluded if legacy MRO mechanism cannot cope with it. RAN3 #119 Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure. Case c/Case d/Case e/Case f would not be considered for MRO for fast MCG recovery failure? Additional information to be reported by UE? Agreements: Definitions of MRO events for CPAC will be introduced in TS 37.340 in a new chapter (based on [1226], CPA needs to be added). Agreements: During CPAC configuration, the value of the Time Stay IE for the source PSCell UHI, sent in S-NODE ADDITION REQUEST message, does not reflect the exact time the UE stayed in the source PSCell. In Rel.18, no extra signalling will be enabled to support CPAC UHI, stage2 text to be checked? A scenario when MCG fails before CPA/CPC is executed is a subject to regular MCG MRO correction to avoid possible resource waste. Agreements: Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed. Case (c), i.e. the “near-failure scenario” will be addressed. WA: In order to enable support for pre-Rel.18 UEs (and help avoid wasting Uu resources), the T316 will be delivered from the MN to the SN. Agreements: Stage-2 description of the detection mechanism will be introduced in TS 38.300 (based on R3-231585). Agreements: Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running). The document containing list of scenarios (last noted at #118 as R3-226422) is recommended to be updated to cover the scenario ‘f1’. To be continued: Transfer of the T316 from the MN to the SN to support the MRO for the fast MCG recovery Rel.18 scenarios for pre-Rel.18 UEs (or Uu resource usage optimisation). Agreements:Agree to define case c in R18. Solution can be further discussed. How to support pre-R18 UE? Whether the network based solution or UE based solution should be used for Case c? Whether the MRO for fast MCG recovery needs to support pre-R18 UEs? MRO for CPAC: Whether CPA/CPC candidate PSCell list can be sent from MN to the source/last serving SN? UHI for CPAC? RAN3 assumes the indication is needed and draft the LS to RAN2 to check the feasibility of the solution for MRO for voice fallback.

It could be found that some remaining issues should be resolved related to the newly introduced case C, as well as other information should be exchanged between MN and SN regarding to CPAC.

2	Discussion

During last meeting, the following case C is agreed as one additional scenario for the fast MCG recovery.

- Case c: Fast recovery near failure case, i.e., UE receives the response message from MN via SN while T316 is running which almost expires but not yet.

Furthermore, during online discussion, companies have proposed two different solutions to solve this scenario, which are network based solution and UE based solution:

- Network based solution: Introduce a new SHR-wise report

- UE based solution: Reuse the RLF report

For the network based solution, SN should report to MN the elapsed time between receiving the MCGFailureInformation and receiving confirmation from the UE of the reception of the reconfiguration from the MN. According to such report, the uplink delay will not be reflected. On the other hand, for the UE based solution, UE will send the elapsed value of T316 towards network, which is some additional information. In comparison, the network based solution will break some of the current function of SHR. Therefore, it is suggested to adopt UE based solution for the case C scenario. Also the elapsed time reported by UE should be more accurate compared with the evaluated time maintained at network side. Furthermore, during last meeting, RAN2 has already agreed the UE based solution.

It is suggested to adopt UE based solution for case C scenario for the fast MCG recovery.

During last meeting, one of the remaining issues left, which is whether to transfer T316 from the MN to the SN to support the MRO for the fast MCG recovery Rel-18 scenarios for pre-Rel 18 UEs or Uu resource usage optimization. From our view, at current stage, the motivation of such optimization is not clear due to it is not a must for Pre-Rel 18 UE to support the Rel-18 fast MCG recovery scenarios. Thus, such optimization may just involve unnecessary signalling overhead.

Therefore, it is not suggested to transfer T316 from the MN to the SN to support the MRO for the fast MCG recovery Rel-18 scenarios for pre-Rel-18 UE or Uu resource usage optimization.

It is not suggested to transfer T316 from the MN to the SN to support the MRO for the fast MCG recover Rel-18 scenarios for pre Rel-18 UE or Uu resource usage optimization.

3	Conclusion and Proposals

In this contribution, we have analysed other potential cases which may be included in the MRO for fast MCG recovery, as well as some necessary information should be reported for fast MCG recovery, and the following proposals has been given out:

Proposal1.	It is suggested to adopt UE based solution for case C scenario for the fast MCG recovery.

Proposal2.	It is not suggested to transfer T316 from the MN to the SN to support the MRO for the fast MCG recover Rel-18 scenarios for pre Rel-18 UE or Uu resource usage optimization.

4	References