3GPP TSG-RAN WG3 Meeting #121bis	R3-235560

Xiamen, China, 9 – 13 October 2023

Agenda Item:	11.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.423) QoE scenarios requiring support in Xn signalling for NR-DC

Document for:	Text Proposal

# 1	Introduction

In this paper we analyze basic QMC scenarios for NR-DC, based on the current state of the BLCR for XnAP.

# 2	Discussion

In current version of the XnAP BLCR [1], messages for NR-DC are enhanced with the following IEs:

- QMC Initial Coordination: QMC Initial Coordination Request, QMC Initial Coordination Response. Current semantics: used for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

- QMC Modification: QMC Modification Request, QMC Modification Response. Current semantics: used for requesting modification of one or more QoE and/or RAN visible QoE measurements.

The IEs are included in the following XnAP signalling procedures:

- S-NG-RAN node Addition Preparation: It is FFS which IE to be used in this procedure (same IE as in other procedures or different IE).

- M-NG-RAN node initiated S-NG-RAN node Modification Preparation

- S-NG-RAN node initiated S-NG-RAN node Modification

In our understanding, the main scenario to be handled under SN Addition is when it is triggered by MN with the aim to offload QoE reporting, and possibly also RVQoE reporting, to SRB5.

A corresponding agreement on how to handle this was taken at RAN3#119bis-e:

If the SN is asked by the MN to forward to the MCE the QoE reports pertaining to a measurement configured by the MN, the MN should indicate to the SN the QoE Reference, the MCE IP Address and the RRC ID.

Furthermore, RAN3#121 agreed:

For s-based QoE configuration received by MN

- MN sends the QoE configuration via SRB1

- QoE reports can be sent via SRB4 or SRB5

We believe that the same mechanism applies to m-based QoE, and therefore propose in alignment with the agreement above:

Proposal 1: For m-based QoE configuration received by MN directly from OAM, MN sends the QoE configuration via SRB1, and QoE reports can be sent via SRB4 or SRB5.

Based on this, we believe that the mentioned scenario is the only QoE scenario requiring Rel-18 support in the SN Addition procedure.

Proposal 2: Only one QoE scenario requires support in the SN Addition procedure: MN triggers SN Addition with the aim to offload QoE reporting, and possibly also RVQoE reporting, to SRB5.

How to support this scenario in stage 3 would first require some additional clarification of the use of QMC Initial Configuration vs. QMC Modification, beyond semantics currently captured in stage 3 and copied above. We believe that following RAN3#121’s agreement copied above, only a single scenario remains for QMC Initial Configuration, i.e. in case of m-based QMC received by the SN. The main purpose of QMC Initial Configuration is to request the MN to allocate the RRC ID.

Proposal 3: QMC Initial Configuration is only triggered by the SN, and is used in the case of m-based QMC received by the SN directly from the OAM with main purpose to request the MN to allocate RRC ID.

Our further understanding is that QMC Modification may be triggered by any node (MN or SN), independently of whether the triggering node initially configured the QoE session.

Proposal 4: QMC Modification may be triggered by any node (MN or SN), independently of whether the triggering node initially configured the QoE session.

Based on this understanding, it seems that QMC Modification should be used in the SN Addition scenario described in proposal 2, and not QMC Initial Configuration.

Proposal 5: Replace QMC Initial Configuration by QMC Modification in SN Addition.

The proposals above are reflected in the XnAP TP provided in annex of this paper. In the TP, we additionally provide updates as follows:

- add support of rejected QMC Modification: This is needed at least for QMC Modification in SN Addition because failed QMC Modification should not fail the SN Addition procedure, but still the SN may reject use of SRB5 as per earlier RAN3 agreement. We also believe that other procedures, like bearer setup in the SN may involve QMC Modification, however a rejected QMC Modification should in our understanding not require to fail the entire M-NG-RAN node initiated S-NG-RAN node Modification procedure. When it comes to QMC Initial Configuration, with the understanding expressed in proposal 3 above, it seems this request is only needed to be included in the S-NG-RAN node initiated S-NG-RAN node Modification procedure (SN Modification Required). And it can be ok to fail the entire SN Modification in case of rejected QMC Initial Configuration, so support for failure handling within the QMC Initial Configuration Response IE doesn’t seem to be needed.

- because QMC Modification may be received by a node that has not configured the QoE session, the QoE Reporting Modification Request IE and the RAN Visible QoE Reporting Modification Request IE should both explicitly indicate the requested configuration, i.e. whether reporting is requested to be performed over SRB4 or SRB5.

Proposal 6: Discuss and agree XnAP TP provided in annex of this paper.

# 3	Conclusion

We have made the following proposals:

Proposal 1: For m-based QoE configuration received by MN directly from OAM, MN sends the QoE configuration via SRB1, and QoE reports can be sent via SRB4 or SRB5.

Proposal 2: Only one QoE scenario requires support in the SN Addition procedure: MN triggers SN Addition with the aim to offload QoE reporting, and possibly also RVQoE reporting, to SRB5.

Proposal 3: QMC Initial Configuration is only triggered by the SN, and is used in the case of m-based QMC received by the SN directly from the OAM with main purpose to request the MN to allocate RRC ID.

Proposal 4: QMC Modification may be triggered by any node (MN or SN), independently of whether the triggering node initially configured the QoE session.

Proposal 5: Replace QMC Initial Configuration by QMC Modification in SN Addition.

Proposal 6: Discuss and agree XnAP TP provided in annex of this paper.

# References

[1]		R3-234811, (BL CR to 38.423) Introduction of R18 QoE measurement

# Annex	- TP for TS 38.423

The TP is based on [1].

<<< start of changes >>>

## 8.3	Procedures for Dual Connectivity

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

If the QMC Modification Request IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Modification Response IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

Editor’s Note: The procedure text could be further refined. This note applies to all the new procedure text in this TP.

Editor’s Note: Whether SN Addition procedure would be used for QMC modification could be further discussed. It is FFS which IE to be used in this procedure (same IE as in other procedures or different IE).

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the Management Based MDT PLMN Modification List IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, overwrite any previously stored Management Based MDT PLMN List information in the UE context and use the received information to determine subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the QMC Modification Request IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Modification Response IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

### 8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification

#### 8.3.4.1	General

This procedure is used by the S-NG-RAN node to modify the UE context in the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: S-NG-RAN node initiated S-NG-RAN node Modification, successful operation.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the SCG Reconfiguration Notification IE is included in the S-NODE MODIFICATION REQUIRED message the M-NG-RAN node shall, if supported, consider the request is sent to coordinate CHO or MN-initiated CPC with SCG reconfigurations:

-	If the SCG Reconfiguration Notification IE is set to "executed", the M-NG-RAN node shall, if supported, consider that a reconfiguration of the SCG resources using SRB3 has been executed. If the S-NG-RAN node to M-NG-RAN node Container IE is also included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, consider that the received SCG configuration has already been applied in the UE and should not be forwarded to the UE.

-	If the SCG Reconfiguration Notification IE is set to "executed-deleted", the M-NG-RAN node shall, if supported, consider that a reconfiguration with sync of the SCG resources has been executed and earlier CHO or MN-initiated CPC configuration has been deleted in the UE. If the S-NG-RAN node to M-NG-RAN node Container IE is also included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, consider that the received SCG configuration has already been applied in the UE and should not be forwarded to the UE.

-	If the SCG Reconfiguration Notification IE is set to "deleted", the M-NG-RAN node shall, if supported, consider that an earlier CHO or MN-initiated CPC configuration will be deleted in the UE when the SCG configuration provided in the S-NG-RAN node to M-NG-RAN node Container IE is delivered to the UE and executed.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the QMC Initial Coordination Request IE is contained in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node may use it to determine how to configure the management-based QoE as specified in TS 37.340 [8], and shall, if supported, include the QMC Initial Coordination Response IE in the S-NODE MODIFICATION CONFIRM message.

If the QMC Modification Request IE is contained in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Modification Response IE in the S-NODE MODIFICATION CONFIRM message.

-------------------------------------------Next change-------------------------------------------

# 9	Elements for XnAP Communication

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |  |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |  |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |  |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |  |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |  |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |  |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |  |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |  |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |  |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |  |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |  |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |  |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |  |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |  |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |  |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |  |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |  |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |  |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |  |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |  |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |  |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |  |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |  |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |  |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |  |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node or the source SN. | YES | ignore |  |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |  |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |  |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |  |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |  |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |  |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |  |
| CHO Information SN Addition | O |  |  |  | YES | reject |  |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |  |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |  |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |  |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |  |
| QMC Modification Request | O |  | 9.2.3.x3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

Editor’s Note: Whether QMC Modification Request IE could be included in SN addition Request message could be further discussed.

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |  |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |  |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |  |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |  |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted to be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |  |
| >>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |  |
| >>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |  |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |  |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |  |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |  |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |  |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |  |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |  |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source NG-RAN node for intra-system handover or between the target S-NG-RAN node and the source SN. | YES | ignore |  |
| SCG Activation Status | O |  | 9.2.3.155 |  | YES | ignore |  |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |  |
| >Candidate PSCell List |  | 1 |  |  | – |  |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |  |
| QMC Modification Response | O |  | 9.2.3.x4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

Editor’s Note: Whether QMC Modification Response IE could be included in SN addition Request Acknowledge message could be further discussed.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  |  | 9.2.3.1 |  |  |  | YES |  | reject |  |  |
| M-NG-RAN node UE XnAP ID | M |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the M-NG-RAN node |  | YES |  | reject |  |  |
| S-NG-RAN node UE XnAP ID | M |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the S-NG-RAN node |  | YES |  | reject |  |  |
| Cause | M |  |  |  | 9.2.3.2 |  |  |  | YES |  | ignore |  |  |
| PDCP Change Indication | O |  |  |  | 9.2.3.74 |  |  |  | YES |  | ignore |  |  |
| Selected PLMN | O |  |  |  | PLMN Identity 9.2.2.4 |  | The selected PLMN of the SCG in the S-NG-RAN node. |  | YES |  | ignore |  |  |
| Mobility Restriction List | O |  |  |  | 9.2.3.53 |  |  |  | YES |  | ignore |  |  |
| SCG Configuration Query | O |  |  |  | 9.2.3.27 |  |  |  | YES |  | ignore |  |  |
| UE Context Information |  |  | 0..1 |  |  |  |  |  | YES |  | reject |  |  |
| >UE Security Capabilities | O |  |  |  | 9.2.3.49 |  |  |  | – |  |  |  |  |
| >S-NG-RAN node Security Key | O |  |  |  | 9.2.3.51 |  |  |  | – |  |  |  |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  |  |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  |  |  | – |  |  |  |  |
| >Index to RAT/Frequency Selection Priority | O |  |  |  | 9.2.3.23 |  |  |  | – |  |  |  |  |
| >Lower Layer presence status change | O |  |  |  | 9.2.3.60 |  |  |  | – |  |  |  |  |
| >PDU Session Resources To Be Added List |  |  | 0..1 |  |  |  |  |  | – |  |  |  |  |
| >>PDU Session Resources To Be Added Item |  |  | 1 .. <maxnoofPDUSessions> |  |  |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |  |  |  |
| >>>PDU Session ID | M |  |  |  | 9.2.3.18 |  |  |  | – |  |  |  |  |
| >>>S-NSSAI | M |  |  |  | 9.2.3.21 |  |  |  | – |  |  |  |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  |  |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Setup Info – SN terminated | O |  |  |  | 9.2.1.5 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Setup Info – MN terminated | O |  |  |  | 9.2.1.7 |  |  |  | – |  |  |  |  |
| >>>PDU Session Expected UE Activity Behaviour | O |  |  |  | Expected UE Activity Behaviour 9.2.3.82 |  | Expected UE Activity Behaviour for the PDU Session. |  | YES |  | ignore |  |  |
| >PDU Session Resources To Be Modified List |  |  | 0..1 |  |  |  |  |  | – |  |  |  |  |
| >>PDU Session Resources To Be Modified Item |  |  | 1 .. <maxnoofPDUSessions> |  |  |  | NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE nor the PDU Session Resource Modification Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |  |  |  |
| >>>PDU Session ID | M |  |  |  | 9.2.3.18 |  |  |  | – |  |  |  |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  |  |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Modification Info – SN terminated | O |  |  |  | 9.2.1.9 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Modification Info – MN terminated | O |  |  |  | 9.2.1.11 |  |  |  | – |  |  |  |  |
| >>>S-NSSAI | O |  |  |  | 9.2.3.21 |  |  |  | YES |  | reject |  |  |
| >>>PDU Session Expected UE Activity Behaviour | O |  |  |  | Expected UE Activity Behaviour 9.2.3.82 |  | Expected UE Activity Behaviour for the PDU Session. |  | YES |  | ignore |  |  |
| >PDU Session Resources To Be Released List | O |  |  |  | PDU session List with Cause 9.2.1.26 |  |  |  | – |  |  |  |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  |  |  | OCTET STRING |  | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10]. |  | YES |  | ignore |  |  |
| Requested Split SRBs | O |  |  |  | ENUMERATED (srb1, srb2, srb1&2, ...) |  | Indicates that resources for Split SRBs are requested. |  | YES |  | ignore |  |  |
| Requested Split SRBs release | O |  |  |  | ENUMERATED (srb1, srb2, srb1&2, ...) |  | Indicates that resources for Split SRBs are requested to be released. |  | YES |  | ignore |  |  |
| Desired Activity Notification Level | O |  |  |  | 9.2.3.77 |  |  |  | YES |  | ignore |  |  |
| Additional DRB IDs | O |  |  |  | DRB List 9.2.1.29 |  | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. |  | YES |  | reject |  |  |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  |  |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. |  | YES |  | reject |  |  |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  |  |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. |  | YES |  | reject |  |  |
| Location Information at S-NODE reporting | O |  |  |  | ENUMERATED (pscell, ...) |  | Indicates that the user’s Location Information at S-NODE is to be provided. |  | YES |  | ignore |  |  |
| MR-DC Resource Coordination Information | O |  |  |  | 9.2.2.33 |  | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. |  | YES |  | ignore |  |  |
| PCell ID | O |  |  |  | Global NG-RAN Cell Identity 9.2.2.27 |  |  |  | YES |  | reject |  |  |
| NE-DC TDM Pattern | O |  |  |  | 9.2.2.38 |  |  |  | YES |  | ignore |  |  |
| Requested Fast MCG recovery via SRB3 | O |  |  |  | ENUMERATED (true, ...) |  | Indicates that the resources for fast MCG recovery via SRB3 are requested. |  | YES |  | ignore |  |  |
| Requested Fast MCG recovery via SRB3 Release | O |  |  |  | ENUMERATED (true, ...) |  | Indicates that resources for fast MCG recovery via SRB3 are requested to be released. |  | YES |  | ignore |  |  |
| SN triggered | O |  |  |  | ENUMERATED (TRUE ...) |  |  |  | YES |  | ignore |  |  |
| Target Node ID | O |  |  |  | Global NG-RAN Node ID 9.2.2.3 |  | Indicates the target node ID of the handover procedure decided by the M-NG-RAN node. |  | YES |  | ignore |  |  |
| PSCell History Information Retrieve | O |  |  |  | ENUMERATED (query, ...) |  | Indicates that the SN UE history information is requested. |  | YES |  | ignore |  |  |
| UE History Information from the UE | O |  |  |  | 9.2.3.110 |  |  |  | YES |  | ignore |  |  |
| CHO Information SN Modification | O |  |  |  |  |  |  |  | YES |  | ignore |  |  |
| >Conditional Reconfiguration | M |  |  |  | ENUMERATED (intra-MN-CHO, ...) |  |  |  | – |  |  |  |  |
| >Estimated Arrival Probability | O |  |  |  | INTEGER (1..100) |  |  |  | – |  |  |  |  |
| SCG Activation Request | O |  |  |  | 9.2.3.154 |  |  |  | YES |  | ignore |  |  |
| Conditional PSCell Addition Information Modification Request | O |  |  |  |  |  | This IE may be sent to the target SN. |  | YES |  | ignore |  |  |
| >Maximum Number of PSCells To Prepare | O |  |  |  | INTEGER (1..8, ...) |  | Indicates the maximum number of PSCells that the target SN may prepare. |  | – |  |  |  |  |
| >Estimated Arrival Probability | O |  |  |  | INTEGER (1..100) |  | Indicates the arrival probability for the UE towards the candidate target SN. |  |  |  |  |  |  |
| Conditional PSCell Change Information Update | O |  |  |  |  |  | This IE may be sent to the source SN. |  | YES |  | ignore |  |  |
| >Multiple Target S-NG-RAN Node List |  |  | 1 |  |  |  |  |  | – |  |  |  |  |
| >>Multiple Target S-NG-RAN Node Item |  |  | 1 .. <maxnoofTargetSNs> |  |  |  |  |  | – |  |  |  |  |
| >>>Target S-NG-RAN node ID | M |  |  |  | Global NG-RAN Node ID 9.2.2.3 |  |  |  | – |  |  |  |  |
| >>>Candidate PSCell List |  |  | 1 |  |  |  |  |  | – |  |  |  |  |
| >>>>Candidate PSCell Item |  |  | 1 .. <maxnoofPSCellCandidate> |  |  |  |  |  | – |  |  |  |  |
| >>>>>PSCell ID | M |  |  |  | NR CGI 9.2.2.7 |  |  |  | – |  |  |  |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  |  |  | UE Slice Maximum Bit Rate List 9.2.3.167 |  | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] |  | YES |  | ignore |  |  |
| Management Based MDT PLMN Modification List | O |  |  |  | MDT PLMN Modification List 9.2.3.169 |  |  |  | YES |  | ignore |  |  |
| QMC Modification Request | O |  |  |  | 9.2.3.x3 |  |  |  | YES |  | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  |  | 9.2.3.1 |  |  |  | YES |  | reject |  |  |
| M-NG-RAN node UE XnAP ID | M |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the M-NG-RAN node |  | YES |  | ignore |  |  |
| S-NG-RAN node UE XnAP ID | M |  |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | Allocated at the S-NG-RAN node |  | YES |  | ignore |  |  |
| PDU Session Resources Admitted List |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |  |
| >PDU Session Resources Admitted To Be Added List |  |  | 0..1 |  |  |  |  |  | – |  |  |  |  |
| >>PDU Session Resources Admitted To Be Added Item |  |  | 1 .. <maxnoofPDUSessions> |  |  |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |  |  |  |
| >>>PDU Session ID | M |  |  |  | 9.2.3.18 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Setup Response Info – SN terminated | O |  |  |  | 9.2.1.6 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Setup Response Info – MN terminated | O |  |  |  | 9.2.1.8 |  |  |  | – |  |  |  |  |
| >PDU Session Resources Admitted To Be Modified List |  |  | 0..1 |  |  |  |  |  | – |  |  |  |  |
| >>PDU Session Resources Admitted To Be Modified Item |  |  | 1 .. <maxnoofPDUSessions> |  |  |  | NOTE: If neither the PDU Session Resource Modification Response Info – SN terminated IE nor the PDU Session Resource Modification Response Info – MN terminated IE is present in a PDU Session Resources Admitted To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |  |  |  |
| >>>PDU Session ID | M |  |  |  | 9.2.3.18 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Modification Response Info – SN terminated | O |  |  |  | 9.2.1.10 |  |  |  | – |  |  |  |  |
| >>>PDU Session Resource Modification Response Info – MN terminated | O |  |  |  | 9.2.1.12 |  |  |  | – |  |  |  |  |
| >PDU Session Resources Admitted To Be Released List |  |  | 0..1 |  |  |  |  |  | – |  |  |  |  |
| >>PDU Session Resources admitted to be released List – SN terminated | O |  |  |  | PDU session List with data forwarding request info 9.2.1.24 |  |  |  | – |  |  |  |  |
| >>PDU Session Resources admitted to be released List – MN terminated | O |  |  |  | PDU session List with data Cause 9.2.1.26 |  |  |  | – |  |  |  |  |
| PDU Session Resources Not Admitted to be Added List | O |  |  |  | PDU session List 9.2.1.27 |  |  |  | YES |  | ignore |  |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  |  |  | OCTET STRING |  | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. |  | YES |  | ignore |  |  |
| Admitted Split SRBs | O |  |  |  | ENUMERATED (srb1, srb2, srb1&2, ...) |  | Indicates admitted SRBs |  | YES |  | ignore |  |  |
| Admitted Split SRBs release | O |  |  |  | ENUMERATED (srb1, srb2, srb1&2, ...) |  | Indicates admitted SRBs release |  | YES |  | ignore |  |  |
| Criticality Diagnostics | O |  |  |  | 9.2.3.3 |  |  |  | YES |  | ignore |  |  |
| Location Information at S-NODE | O |  |  |  | Target Cell Global ID 9.2.3.25 |  | Contains information to support localisation of the UE |  | YES |  | ignore |  |  |
| MR-DC Resource Coordination Information | O |  |  |  | 9.2.2.33 |  | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. |  | YES |  | ignore |  |  |
| PDU Session Resources with Data Forwarding List |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |  |
| >PDU Session Resources with Data Forwarding List – SN terminated | M |  |  |  | PDU session List with data forwarding request info 9.2.1.24 |  |  |  | – |  |  |  |  |
| RRC Config Indication | O |  |  |  | 9.2.3.72 |  |  |  | YES |  | reject |  |  |
| Available fast MCG recovery via SRB3 | O |  |  |  | ENUMERATED (true, ...) |  | Indicates the fast MCG recovery via SRB3 isenabled. |  | YES |  | ignore |  |  |
| Release fast MCG recovery via SRB3 | O |  |  |  | ENUMERATED (true, ...) |  | Indicates the fast MCG recovery via SRB3 is released. |  | YES |  | ignore |  |  |
| Direct Forwarding Path Availability | O |  |  |  | ENUMERATED (direct path available,…) |  | Indicates direct path is available between the S-NG-RAN node and the target NG-RAN node. |  | YES |  | ignore |  |  |
| SCG UE History Information | O |  |  |  | 9.2.3.151 |  |  |  | YES |  | ignore |  |  |
| SCG Activation Status | O |  |  |  | 9.2.3.155 |  |  |  | YES |  | ignore |  |  |
| Conditional PSCell Addition Information Modification Acknowledge | O |  |  |  |  |  | This IE may be sent from the target SN. |  | YES |  | ignore |  |  |
| >Candidate PSCell List |  |  | 1 |  |  |  |  |  | – |  |  |  |  |
| >>Candidate PSCell Item |  |  | 1 .. <maxnoofPSCellCandidate> |  |  |  |  |  | – |  |  |  |  |
| >>>PSCell ID | M |  |  |  | NR CGI 9.2.2.7 |  |  |  | – |  |  |  |  |
| QMC Modification Response | O |  |  |  | 9.2.3.x4 |  |  |  | YES |  | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |  |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |  |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |  |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |  |
| PDCP Change Indication | O |  | 9.2.3.74 |  | YES | ignore |  |
| PDU Session Resources To Be Modified List |  | 0..1 |  |  | YES | ignore |  |
| >PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Required Info – SN terminated IE nor the PDU Session Resource Modification Required Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.4.4 apply. | – |  |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |  |
| >>PDU Session Resource Modification Required Info – SN terminated | O |  | 9.2.1.20 |  | – |  |  |
| >>PDU Session Resource Modification Required Info – MN terminated | O |  | 9.2.1.22 |  | – |  |  |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |  |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUSessions> |  |  | – |  |  |
| >PDU sessions to be released List – SN terminated | O |  | PDU session List with data forwarding request info 9.2.1.24 |  | – |  |  |
| >PDU sessions to be released List – MN terminated | O |  | PDU session List with Cause 9.2.1.26 |  | – |  |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |  |
| Spare DRB IDs | O |  | DRB List 9.2.1.29 | Indicates the list of unnecessary DRB IDs that had been used by the S-NG-RAN node. | YES | ignore |  |
| Required Number of DRB IDs | O |  | Number of DRBs 9.2.3.78 | Indicates the number of DRB IDs that the S-NG-RAN node requests more. | YES | ignore |  |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |  |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |  |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |  |
| SCG Indicator | O |  | ENUMERATED(released,...) |  | YES | ignore |  |
| SCG UE History Information | O |  | 9.2.3.151 |  | Yes | ignore |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |  |
| CPAC Information Required | O |  |  | This IE may be sent from the target SN. | YES | ignore |  |
| >Candidate PSCell List |  | 1 |  | Indicates the full list of candidate PSCells prepared at the target S-NG-RAN node. | – |  |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |  |
| SCG Reconfiguration Notification | O |  | ENUMERATED (executed, ..., executed-deleted, deleted) |  | YES | ignore |  |
| QMC Initial Coordination Request | O |  | 9.2.3.x1 |  | YES | ignore |
| QMC Modification Request | O |  | 9.2.3.x3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

#### 9.1.2.9	S-NODE MODIFICATION CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node about the successful modification.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |  |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |  |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |  |
| PDU sessions Admitted To Be Modified List |  | 0..1 |  |  | YES | ignore |  |
| >PDU sessions Admitted To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If neither the PDU Session Resource Modification Confirm Info – SN terminated IE nor the PDU Session Resource Modification Confirm Info – MN terminated IE is present in a PDU Session Resources Admitted To Be Modified Item IE, abnormal conditions as specified in clause 8.3.4.4 apply. | – |  |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |  |
| >>PDU Session Resource Modification Confirm Info – SN terminated | O |  | 9.2.1.21 |  | – |  |  |
| >>PDU Session Resource Modification Confirm Info – MN terminated | O |  | 9.2.1.23 |  | – |  |  |
| PDU sessions Released List |  | 0..1 |  |  | YES | ignore |  |
| >PDU sessions released List – SN terminated | O |  | PDU Session List with data forwarding info from the target node 9.2.1.25 |  | – |  |  |
| >PDU sessions released List – MN terminated | O |  | PDU session List 9.2.1.27 |  | – |  |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10] or the RRCConnectionReconfigurationComplete message as defined in subclause 6.2.2 of TS 36.331 [14]. | YES | ignore |  |
| Additional DRB IDs | O |  | DRB List 9.2.1.29 | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |  |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |  |
| QMC Initial Coordination Response | O |  | 9.2.3.x2 |  | YES | ignore |
| QMC Modification Response | O |  | 9.2.3.x4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.20	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message or from the S-NG-RAN-NODE to the M-NG-RAN-NODE to report the DL RRC message delivery status.

This message is also sent by the new NG-RAN-NODE to the old NG-RAN-NODE or from the old NG-RAN-NODE to the new NG-RAN-NODE to transfer an RRC message containing the SDT SRB in case of RACH based SDT without UE context relocation.

This message is also sent by the M-NG-RAN-NODE to the S-NG-RAN node or from the S-NG-RAN node to the M-NG-RAN node to forward the RAN visible QoE measurement results received from the UE.

Editor’s Note: The use of this message is to be confirmed.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node (Dual Connectivity).

Direction: new NG-RAN node  old NG-RAN node or old NG-RAN node  new NG-RAN node (SDT).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |  |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |  |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |  |
| Split SRB |  | 0..1 |  |  | YES | reject |  |
| >RRC Container | O |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 38.331 [10] or TS 36.331 [14] and ciphered with the key of the M-NG-RAN node | – |  |  |
| >SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type to be used | – |  |  |
| >Delivery Status | O |  | 9.2.3.45 | DL RRC delivery status of split SRB | – |  |  |
| UE Report |  | 0..1 |  |  | YES | reject |  |
| >RRC Container | M |  | OCTET STRING | For NGEN-DC and NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the MeasurementReport message or the RRCReconfigurationComplete message or the FailureInformation message or the UEAssistanceInformation message. For NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the IABOtherInformation message. For NE-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the MeasurementReport message. | – |  |  |
| Fast MCG Recovery via SRB3 from SN to MN |  | 0..1 |  |  | YES | ignore |  |
| >RRC Container | M |  | OCTET STRING | For NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the MCGFailureInformation, message. For NGEN-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the MCGFailureInformation message. | – |  |  |
| Fast MCG Recovery via SRB3 from MN to SN |  | 0..1 |  |  | YES | ignore |  |
| >RRC Container | M |  | OCTET STRING | For NR-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the RRCReconfiguration message, or the RRCRelease message, or the MobilityFromNRCommand message. For NGEN-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the RRCConnectionReconfiguration message, or the RRCConnectionRelease message, or the MobilityFromEUTRACommandmessage. | – |  |  |
| SDT SRB between New NG-RAN node and Old NG-RAN node |  | 0..1 |  |  | YES | ignore |  |
| >RRC Container | M |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 38.331 [10]. | – |  |  |
| >SRB ID | M |  | 9.2.3.165 | In this version of the specification, values "0", "1", "3", and "4" are not set by the sender and ignored by the receiver. | – |  |  |
| RVQoE measurement report |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | Contains RVQoE measurement results in the RAN-VisibleMeasurements IE of the MeasurementReportAppLayer message as defined in 38.331 [10]. [FFS] | – |  |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 9.2.3	General IE definitions

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.2.3.x1	QMC Initial Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QMC Configuration List |  |  |  |  |  |  |  |  |  |
| >QMC Configuration List Item |  |  |  | 1..<maxnoofUEAppLayerMeas> |  |  |  |  |  |
| >>QoE Reference |  | M |  |  |  | OCTET STRING (SIZE(6)) |  |  |  |
| >>Measurement Collection Entity IP Address |  | O |  |  |  | Transport Layer Address 9.2.3.29 |  | The IP address of the entity receiving the QoE measurement report. |  |
| >>QoE Configuration Sending Request |  | O |  |  |  | ENUMERATED (true, …) |  |  |  |
| >>QoE Reporting Option Preference |  | O |  |  |  | ENUMERATED (SRB4, SRB5[FFS], ...) |  |  |  |
| >>RVQoE Reporting Option Request |  | O |  |  |  | ENUMERATED (SRB4,…) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

Editor’s Note: Whether to also include QoE configuration container in the IE is FFS.

#### 9.2.3.x2	QMC Initial Coordination Response

Editor’s Note: The messages used to convey this IE are FFS.

This IE contains the information that the M-NG-RAN node needs to provide in response to the S-NG-RAN node for the QMC initial coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Response List |  |  |  |  |
| >QMC Response List Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | M |  | INTEGER (0..15, ...) |  |
| >>QoE Configuration Sending Option Response | M |  | ENUMERATED (MN, SN, …). |  |
| >>QoE Reporting Option | O |  | ENUMERATED (SRB4, SRB5, …) |  |
| >>RVQoE Reporting Option Response | O |  | ENUMERATED (SRB4, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x3	QMC Modification Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for requesting modification of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Modification Request List |  |  |  |  |
| >QMC Modification Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE could only be sent by the MN. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 |  |
| >>QoE Reporting Option Request | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the request to switch the SRB used for reception of QoE reports. |
| >>RAN Visible QoE Reporting Option Request | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE indicates the request to switch the SRB used for reception of RAN Visible QoE reports. |
| >>Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x4	QMC Modification Response

This IE contains the information that the S-NG-RAN node or the information that the M-NG-RAN node needs to provide in the response to the QMC modification request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Modification Response List |  |  |  |  |
| >QMC Modification Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>CHOICE QMC Initial Coordination Outcome | M |  |  |  |
| >>>Successful outcome |  |  |  |  |
| >>>>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>>>QoE Reporting Modification response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>>>RVQoE Reporting Modification response | O |  | ENUMERATED (accepted, rejected, …) | FFS whether the code points should be refined, to reflect the agreement about the response from non-RVQoE configuring node. |
| >>>>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 |  |
| >>>Unsuccessful outcome |  |  | NULL |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x5	RAN Visible QoE Configuration Preference

This IE provides information of RAN visible QoE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available RAN visible QoE Metrics | O |  | 9.2.3.158 |  |
| Reporting periodicity | O |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024) |  |
| Event trigger [FFS] | O |  |  |  |

Editor’s Note: The naming and coding of all the new defined IEs in the above tables could be further refined.

<<< end of changes >>>