3GPP TSG-RAN WG3 #121bis							R3-235731

Xiamen, CN, 9th Oct – 13th Oct 2023

Agenda Item:	10.2.3

Source:	Ericsson

Title:	(TPs for SON BLCRs for TS 38.473) RACH Optimization enhancement

Document for:	Discussion, Decision

# 1	TP for 38.473 BL CR

#### 9.2.1.x1	RACH INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about one or more random access procedures performed at the gNB-DU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RA Report Indication List |  | 1 |  |  | YES | reject |
| >RA Report Indication List Item |  | 1..<maxnoofUEsforRAReport Indications> |  |  | - | - |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEsforRAReportIndications | Maximum number of UEs from which gNB-DU is interested to collect RA report. Value is 64. |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

[snip]

maxnoofPosSITypes,

maxnoofMRBs,

maxNrofBWPs,

maxnoofUEsforRAReportIndications,

maxnoofSuccessfulPSCellChangeReports

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

[snip]

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

maxnoofUEsforRAReportIndications				INTEGER ::= 64

maxnoofSuccessfulPSCellChangeReports	INTEGER ::= 64  –- value is FFS

[snip]