3GPP TSG-RAN WG3 Meeting #121	R3-235831

Xiamen, China, 9 - 13 October 2023

Agenda Item:	25.2.2

Source:	ZTE, CMCC, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, 	Samsung

Title:	(TP to BL CR TS 38.415) support for ECN Marking

Document for:	Discussions & Approval

1	Introduction

This paper provides the corresponding TP for TS 38.415 to support ECN Marking for XR.

2	Text Proposal for BL CR TS 38.415

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 |  | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  | QMP | DL Delay Ind. |  | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag |  | QoS Flow Identifier |  |  |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) |  | New IE Flag 6 |  | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 |  | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Information |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| DL Information |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0).

Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Information is present (1) or not (0)

Bit 2 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Information is present (1) or not (0)

Editor’s notes: FFS on where the two new flags are.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

#### 5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU session UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SESSION INFORMATION, 1=UL PDU SESSION INFORMATION, 2-15=reserved for future PDU type extensions}.

Field length: 4 bits.

#### 5.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

*// skip unchanged part //*

#### 5.5.3.23	MBS Sequence Number Presence (MSNP)

Description: This parameter indicates the presence of the DL MBS QFI Sequence Number in the DL PDU Session Information frame.

Value range: {0= DL MBS QFI Sequence Number not present, 1= DL MBS QFI Sequence Number present}.

Field length: 1 bit.

#### 5.5.3.24	DL MBS QFI Sequence Number

Description: This parameter indicates the sequence number as assigned by the MB-UPF associated with a given MBS QoS Flow.

Value range: {0..232-1}.

#### 5.5.3.x1	DL Information (FFS)

Description: FFS.

Value range: FFS.

Field length: 2 octets.

#### 5.5.3.x2	UL Information (FFS)

Description: FFS.

Value range: FFS.

Field length: 2 octets.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>