3GPP TSG-RAN WG3 Meeting #121	R3-233815

Toulouse, France, 21 - 25 August 2023

Agenda item:	26.2

Source:	Nokia, Nokia Shanghai Bell, Samsung, Qualcomm, CATT

Title:	(TP for TS 38.401 BL CR) Stage 2 for timing resiliency and URLLC

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#120, RAN3 continued discussing the overall signalling framework to support RAN timing synchronization status (TSS) reporting and reached the following agreements:

In NGAP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB to report RAN TSS to the AMF. In F1AP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB-DU to report RAN TSS to the gNB-CU.

However, RAN3 was unable to conclude on whether the new NGAP Timing Synchronisation Report procedure is controlled by the AMF, i.e. whether to introduce a new AMF-initiated class 1 procedure.  It is critical for RAN3 to conclude on the Stage 2 aspects of RAN TSS reporting at this meeting, in order to complete the remaining stage 3 work in Q4.

In this paper, we further discuss the overall signalling framework for RAN TSS reporting and provide a Stage 2 text proposal.

# 2	Discussion

To determine when the gNB time synchronization is under normal or abnormal operation, TS 23.502 states “the RAN node is “pre-configured for the thresholds for each timing synchronization status attribute.”. When the gNB timing synchronization status exceeds any of the pre-configured thresholds (i.e. status degradation), or the gNB timing synchronization status meets the thresholds again (i.e. status improvement), the gNB generates a RAN Timing Synchronization Status (TSS) report and notifies it to the consumer(s) of the information. There are two potential consumers of RAN TSS information:

1)	The CN: The TSCTSF is the NF that controls time synchronization as a service if the AF requests it via exposure framework for a target UE. If the AF provided clock quality reporting control information (i.e., clock quality detail level and clock quality acceptance criteria), the TSCTSF will subscribe to RAN TSS updates at the AMF. Based on the updates the TSCTSF receives, it can determine the status of the time synchronization service for the targeted UEs. This corresponds to objective 1d in the WID.

2)	The UE: As a consumer of the 5G clock the gNB provides, RAN TSS updates provided to the UE help the UE deciding if the service is ok or not (e.g., to determine if it should change to another time source if available). The reporting to the UE requires clock quality reporting control information to be provided to the gNB from the AMF. The trigger for this configuration may be an AF request or the UE subscription. This corresponds to objectives 1a to 1c in the WID.

Despite the two different consumers, the triggers for reporting are the same for both, i.e. the pre-configured thresholds at the gNB. SA2 did not discuss whether the thresholds are pre-configured to the gNB-CU or to the gNB-DU, since disaggregated architecture is transparent outside of RAN3. However, given the common understanding in RAN3 that the RAN TSS attributes are available at the gNB-DU, it can be concluded that the thresholds are pre-configured at the gNB-DU.

Observation-1:	RAN3 can assume that gNB-DUs are pre-configured with thresholds for each RAN TSS attribute, which are used to evaluate RAN TSS degradation or improvement.

Further details for RAN TSS reporting to CN and UE are discussed in the subclauses below.

## 2.1	RAN TSS reporting towards the CN

As previously described, there are scenarios where the TSCTSF is subscribed to RAN TSS reports. The TSCTSF may be interested in this information due to time synchronization service provided to a UE. When a gNB reports a RAN TSS update, the TSCTSF is responsible for correlating the timing synchronization status with the UE location to determine if the time synchronization service is acceptable or not for the UE.

The TSCTSF subscribes for RAN TSS updates by sending the Namf_NonUeN2InfoSubscribe to the AMF as described in TS 23.502 clause 4.15.9.5.1. The AMF must then initiate RAN TSS reporting from the gNB. Since a RAN TSS report contains gNB or cell level information that impacts all the UEs served by the gNB/cell, the signaling required to provide this information via control plane to the CN should be non-UE associated.

At RAN#120, the RAN TSS reporting over NG was discussed and there was down-selection to the following two alternative solutions [1]:

Solution 1: New AMF-initiated class 1 procedure + new gNB-initiated class 2 procedure

-	Common understanding is that class 1 procedure is not used to for gNB to indicate its capabilities

Solution 2A: New gNB-initiated class 2 procedure

The main difference is whether the RAN TSS reporting is enabled/disabled via NGAP/F1AP signalling (solution 1) or via OAM (solution 2A).  Solution 1 is preferable:

-	It is aligned with SA2 agreements captured in:

-	clause 5.27.1.12 of TS 23.501: “TSCTSF may receive node-level information about timing synchronization status from gNB […], if supported by a node, using control plane signalling at node level. […] the AMF controls the gNB node level reporting and subscription using NGAP messages (see TS 38.413 [34]).”; and

-	clause 4.15.9.5.1 of TS 23.502: step 2 of figure 4.15.9.5.1-1.

-	RAN3 has commonly introduced a “control” procedure for functionality that uses reporting. There is no apparent reason to diverge from this good protocol design principle.

Therefore, the following is proposed:

Proposal 1:	Introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1, non-UE associated) in NGAP/F1AP to enable the AMF to initiate RAN TSS reporting by the gNB-DU.

RAN3 has previously agreed to introduce a new Timing Synchronization Status Report procedure (class 2, non-UE associated) in NGAP/F1AP.  The resulting overall signaling flow for RAN TSS reporting towards the CN is shown in Figure 1.

Figure 1:		RAN TSS reporting towards the CN

The steps for RAN TSS reporting towards the CN are as follows:

0.	The gNB-DU is pre-configured with thresholds for each RAN TSS attribute it supports.

NOTE 1:	It is assumed the pre-configured thresholds in the gNB-DU are sufficient to meet UE time sync performance requirement which are configured by the operator. The gNB-CU may also be pre-configured with the thresholds.

1.	The AMF requests RAN TSS reporting by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-CU. This may be triggered by a Namf_NonUeN2InfoSubscribe from the TSCTSF.

2.	Upon receiving the request from the AMF, the gNB-CU requests RAN TSS reporting from at least one gNB-DU by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-DU.

NOTE 2:	It is up to gNB-CU implementation whether to send the request to all its gNB-DUs or to a particular subset of its gNB-DUs, depending on e.g. network topology.

3.	The gNB-DU replies to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS RESPONSE message.

4.	Upon receiving the response from the gNB-DU, the gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS RESPONSE message. If the gNB-CU does not receive a successful response from at least one gNB-DU, the gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS FAILURE message and the flow stops at this step.

5. 	Upon sending the response to the gNB-CU, the gNB-DU provides a first RAN TSS report to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 3:	The list of RAN TSS attributes included in the report is up to gNB-DU implementation.

6.	Upon receiving the report from the gNB-DU, the gNB-CU sends the TIMING SYNCHRONISATION STATUS REPORT message to the AMF. The message contains the RAN TSS attributes received from the gNB-DU and the RAN TSS Scope IE to indicate whether the scope of the RAN TSS report is “all cells within the gNB” or “list of Cell IDs within the gNB”.

NOTE 4:	The CN may use the received RAN TSS report for determining the RAN TSS reporting capabilities of the gNB or list of cells.

7.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

8.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

9. 	Same as step 6.

Regarding step 2, RAN3 agreed that “topologies can exist where clock quality information is different between “groups of cells within a single gNB” (e.g. cells served by different gNB-DUs)” (see [2]). Furthermore, there was a common understanding that the gNB-CU is aware (through means outside the scope of 3GPP specifications) which gNB-DUs share the same clock quality. Therefore, the gNB-CU does not necessarily need to request RAN TSS reporting from all gNB-DUs; it could instead request from only one gNB-DU of the group that share the same clock quality. Alternatively, the gNB-CU could request RAN TSS reporting from all gNB-DUs and then aggregate the reporting towards the AMF.  This can be left to gNB-CU implementation.

Observation-2:	RAN3 can assume that the gNB-CU is aware of which gNB-DUs share the same clock quality. How the gNB-CU knows this is outside the scope of 3GPP specifications.

Regarding steps 5-6, it seems beneficial for the TSCTSF to receive feedback regarding the RAN TSS attributes that it can expect from the NG-RAN. Without such feedback, the TSCTSF may erroneously assume that it will receive notifications of primary source events even when they are not supported by the NG-RAN. Although the same thresholds are pre-configured to all gNB-DUs, the capabilities of the gNB-DUs may be different. Also, without receiving the current values of the RAN TSS attributes at the time reporting is initiated, the TSCTSF would be unaware whether the current status is degraded or not (it would have to wait for the next primary source event).

Observation-3:	RAN TSS reporting capability for the gNB (or group of cells) can be inferred by the content of the TIMING SYNCHRONISATION STATUS REPORT message. No explicit capability signalling is needed in the TIMING SYNCHRONISATION RESPONSE message.

## 2.2	RAN TSS reporting towards the UE

In addition to the pre-configured thresholds at the gNB-DU, which are node level parameters, the time synchronization as a service may include clock quality reporting control information containing UE level parameters. If the per-UE clock quality detail level is set to “acceptable/not acceptable indication”, the gNB is also provided with the clock quality acceptance criteria for the UE which describes the conditions for acceptable/not acceptable time synchronization service operation for a UE.

The evaluation of whether the clock quality is acceptable or not could conceivably be performed by either the gNB-DU (if the clock quality acceptance criteria is transferred from gNB-CU to gNB-DU) or by the gNB-CU (if the clock quality metrics are passed from the gNB-DU to the gNB-CU). For both cases, the gNB-CU is required to know the result of the evaluation (i.e., acceptable/not acceptable), to provide the result to the UE using unicast RRC, and the metrics to report them to the AMF, if subscribed. Additionally, keeping the clock quality acceptance criteria at the gNB-CU enables F1AP signaling to be node level, reducing the amount of timing synchronization status reports required between gNB-DU and gNB-CU.

For RAN TSS reporting towards the UE, the gNB requires clock quality reporting control information to be stored in the UE context. To provide this information to the gNB, the Time Synchronisation Assistance Information IE has been extended to include clock quality reporting control information. Then, when the serving gNB detects a new RAN TSS report is available and it has clock quality information stored within the UE context, the gNB-CU provides the latest report towards the UE via dedicated RRC signaling (once the UE is in RRC_CONNECTED state). The signaling is UE specific, and while the UE is in RRC_CONNECTED the gNB can continue providing updates to the UE as needed.

Proposal 2:	The gNB-CU stores the clock quality reporting control information in the UE context and determines the clock quality information to provide to the UE.

The resulting signaling flow for RAN TSS reporting towards the UE in RRC_CONNECTED state is shown in Figure 2.

Figure 2: RAN TSS reporting towards the UE in RRC_CONNECTED state

The steps for RAN TSS reporting towards the UE are as follows:

0.	The gNB-DU is pre-configured with thresholds for each RAN TSS attribute it supports.

NOTE 1: 	It is assumed the pre-configured thresholds in the gNB-DU are sufficient to meet UE time sync performance requirement which are configured by the operator. The gNB-CU may also be pre-configured with the thresholds.

1.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU, containing the Clock Quality Reporting Control Information IE within the Time Synchronization Assistance Information IE. The clock quality reporting control information indicates the clock quality detail level to provide to the UE, i.e. “metrics” or “acceptable/not acceptable indication”. If clock quality detail level equals “acceptable/not acceptable indication”, the clock quality reporting control information also contains the clock quality acceptance criteria.

NOTE 2:	The clock quality reporting control information can also be provided in the UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, or PATH SWITCH REQUEST ACKNOWLEDGE messages.

2.	The gNB-CU requests RAN TSS reporting by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-DU.

3.	The gNB-DU replies to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS RESPONSE message.

NOTE 3:	Steps 2, 3 and 5 may be skipped in case RAN TSS reporting is already enabled at the gNB-DU by an earlier request.

4.	The gNB-CU replies to the AMF by sending the INITIAL CONTEXT SETUP RESPONSE message.

5.	Upon sending the response to the gNB-CU, the gNB-DU provides a first RAN TSS report to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 4:	The list of RAN TSS attributes included in the report is up to gNB-DU implementation. The gNB-CU may use the received RAN TSS report for determining the RAN TSS reporting capabilities of the gNB-DU.

6.	The gNB-CU sends the latest clock quality information to the UE by sending the DLInformationTransfer message. The clock quality information provided to the UE depends on the clock quality detail level received in step 1 (i.e., “metrics” or “acceptable/not acceptable indication”).

7.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

8.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

9.	Same as step 6.

# 3	Conclusions

In this paper, we discuss the overall signalling framework for RAN TSS reporting and made the following observations and proposals:

Observation-1:	RAN3 can assume that gNB-DUs are pre-configured with thresholds for each RAN TSS attribute, which are used to evaluate RAN TSS degradation or improvement.

Observation-2:	RAN3 can assume that the gNB-CU is aware of which gNB-DUs share the same clock quality. How the gNB-CU knows this is outside the scope of 3GPP specifications.

Observation-3:	RAN TSS reporting capability for the gNB (or group of cells) can be inferred by the content of the TIMING SYNCHRONISATION STATUS REPORT message. No explicit capability signaling is needed in the TIMING SYNCHRONISATION RESPONSE message.

Proposal 1:	Introduce a new Timing Synchronisation Status Reporting Initiation procedure (class 1, non-UE associated) in NGAP/F1AP to enable the AMF to initiate RAN TSS reporting by the gNB-DU.

Proposal 2:	The gNB-CU stores the clock quality reporting control information in the UE context and determines the clock quality information to provide to the UE.

Proposal 3:	Agree to the Stage 2 text proposal for TS 38.401 in Annex A.

# References

- R3-233352 29_R18URLLC Summary of Offline, Nokia (moderator)

- R3-230811 Reply LS on proposed method for time synchronization status reporting to UE(s), RAN3

# Annex	A: Text Proposal for TS 38.401

start of modifications

## 8.x	Timing resiliency service

The following clauses describe the overall procedures for RAN Timing Synchronisation Status (TSS) reporting involving gNB-CU/gNB-DU.

### 8.x.1	RAN TSS reporting towards the CN

The signaling flow for RAN TSS reporting towards the CN is shown in Figure 8.x.1-1. This procedure is used when the TSCTSF subscribes to RAN TSS reporting at the AMF as described in TS 23.502 [x].

Figure 8.x.1-1	RAN TSS reporting towards the CN

0.	The gNB-DU is pre-configured with thresholds for each RAN TSS attribute it supports.

NOTE 1: 	It is assumed the pre-configured thresholds in the gNB-DU are sufficient to meet UE time sync performance requirement which are configured by the operator. The gNB-CU may also be pre-configured with the thresholds.

1.	The AMF requests RAN TSS reporting by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-CU.

2.	Upon receiving the request from the AMF, the gNB-CU requests RAN TSS reporting from at least one gNB-DU by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-DU.

NOTE 2:	It is up to gNB-CU implementation whether to send the request to all its gNB-DUs or to a particular subset of its gNB-DUs, depending on e.g. network topology.

3.	The gNB-DU replies to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS RESPONSE message.

4.	Upon receiving the response from the gNB-DU, the gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS RESPONSE message. If the gNB-CU does not receive a successful response from at least one gNB-DU, the gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS FAILURE message and the flow stops at this step.

5. 	Upon sending the response to the gNB-CU, the gNB-DU provides a first RAN TSS report to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 3:	The list of RAN TSS attributes included in the report is up to gNB-DU implementation.

6.	Upon receiving the report from the gNB-DU, the gNB-CU sends the TIMING SYNCHRONISATION STATUS REPORT message to the AMF. The message contains the RAN TSS attributes received from the gNB-DU and the RAN TSS Scope IE to indicate whether the scope of the RAN TSS report is “all cells within the gNB” or “list of Cell IDs within the gNB”.

NOTE 4:	The CN may use the received RAN TSS report for determining the RAN TSS reporting capabilities of the gNB or list of cells.

7.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

8.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

9. 	Same as step 6.

### 8.x.2	RAN TSS reporting towards the UE

The signaling flow for RAN TSS reporting towards the UE in RRC_CONNECTED state is shown in Figure 8.x.2-1.

Figure 8.x.2-1	RAN TSS reporting towards the UE in RRC_CONNECTED state

0.	The gNB-DU is pre-configured with thresholds for each RAN TSS attribute it supports.

NOTE 1: 	It is assumed the pre-configured thresholds in the gNB-DU are sufficient to meet UE time sync performance requirement which are configured by the operator. The gNB-CU may also be pre-configured with the thresholds.

1.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU, containing the Clock Quality Reporting Control Information IE within the Time Synchronization Assistance Information IE. The clock quality reporting control information indicates the clock quality detail level to provide to the UE, i.e. “metrics” or “acceptable/not acceptable indication”. If clock quality detail level equals “acceptable/not acceptable indication”, the clock quality reporting control information also contains the clock quality acceptance criteria.

NOTE 2:	The clock quality reporting control information can also be provided in the UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, or PATH SWITCH REQUEST ACKNOWLEDGE messages.

2.	The gNB-CU requests RAN TSS reporting by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-DU.

3.	The gNB-DU replies to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS RESPONSE message.

NOTE 3:	Steps 2, 3 and 5 may be skipped in case RAN TSS reporting is already enabled at the gNB-DU by an earlier request.

4.	The gNB-CU replies to the AMF by sending the INITIAL CONTEXT SETUP RESPONSE message.

5.	Upon sending the response to the gNB-CU, the gNB-DU provides a first RAN TSS report to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 4:	The list of RAN TSS attributes included in the report is up to gNB-DU implementation. The gNB-CU may use the received RAN TSS report for determining the RAN TSS reporting capabilities of the gNB-DU.

6.	The gNB-CU sends the latest clock quality information to the UE by sending the DLInformationTransfer message. The clock quality information provided to the UE depends on the clock quality detail level received in step 1 (i.e., “metrics” or “acceptable/not acceptable indication”).

7.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

8.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

9.	Same as step 6.

end of modifications