3GPP TSG-RAN WG3 Meeting #121	R3-233901

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	10.2.2

Source:	Huawei

Title:	(TPs for SON BLCRs for TS 38.413) MRO on fast MCG recovery and voice fallback

Document for:	other

# Introduction

In RAN3#119bis meeting, the following agreements were made for the purpose of MRO:

MRO for CPA and CPC:

Whether CPA/CPC candidate PSCell list can be sent from MN to the source/last serving SN?

UHI for CPAC?

MRO for the fast MCG recovery:

Agree to define case c in R18. Solution can be further discussed.

Whether the network based solution or UE based solution should be used for Case c?

Whether the MRO for fast MCG recovery needs to support pre-R18 UEs?

MRO for the voice fall-back:

RAN3 assumes the indication is needed and draft the LS to RAN2 to check the feasibility of the solution for MRO for voice fallback.

RAN2 has decided to introduce the indication of voice fallback in the LTE RLF report.

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

In this paper, for the agreed scenarios and FFSs, we will give further consideration on the potential impact on the specification, and give potential solution to support these above three objectives.

# 2	Discussion

## 2.1	MRO for fast MCG recovery

Further study for the optimization of the near failure scenario is required in the aspects including the definition of near failure (i.e. triggering condition), information reported for the UE, and the utilized report type.

In general, although the fast MCG recovery succeeds, there is underlying issues which might cause an actual fast MCG recovery failure. Besides, the consensus has been reached that the potential issue of fast MCG recovery failure could be lying both in the T316 setting and SCG RLF. Discussion on T316 related triggering condition has been going on for a while. On one hand, the too short T316 may lead to actual MCG recovery failure or near MCG recovery failure frequently, and on the other hand, the too long T316 may take UE wait too long before reestablishment upon detecting fast MCG recovery failure, which cause long service interruption. It is beneficial for the network to adjust the setting of T316. Except for T316 related triggering condition, the following potential issues regarding SCG failure detected by UE should be discussed:

- UE detect the radio link failure for the SCG due to T310 expiry

- UE detect the radio link failure for the SCG due to T312 expiry

- UE receives random access problem indication from SCG MAC, e.g., maximum number of preamble transmissions has been reached.

- UE receives indication form SCG RLC that the maximum number of retransmissions has been reached.

Therefore, in order to make proper adjustment in near failure case, the potential issues regarding SCG RLF need to be identified and solved. So we recommend the definition of near failure of fast MCG recovery should consider the triggering condition regarding SCG near failure as below:

- the ratio of SCG T310

- the ratio of SCG T312

- the number of preamble transmissions towards SCG has reached a threshold

- the number of SCG RLC retransmission has reached a threshold

- Proposal 1:  Introduce the T316 related triggering condition for defining the near failure scenario of fast MCG recovery.

- Proposal 2:   Introduce the following triggering conditions regarding SCG near failure for defining the near failure scenario of fast MCG recovery：

      - the ratio of SCG T310

      - the ratio of SCG T312

      - the number of preamble transmissions towards SCG has reached a threshold

      - the number of SCG RLC retransmission has reached a threshold

A FFS from last meeting is a network-based solution for the timely release of SCG radio resource:

So far, detection of the failure events for MRO for the fast MCG recovery relies on the report from the UE. However, this reporting may be provided from UEs that are Rel.18 only. We consider that if the SN is aware of the T316 it can, based on the reception of the MCGFailureInformation detect at least some of the Rel.18 MRO events also for pre-Rel.18 UE (e.g. scenario ‘b’, where the signalling takes longer than the timer).

However, the network-based solution has its advantages:

- It requires that SN receives the MCGFailureInformation successfully.

- The T316 timer maintained at the SN side is inaccurate for the purpose of estimating the elapsed T316 at the UE side.

In our understanding, for case a and case b, since the transmission of MCGFailureInformation might take so long, the SN receives the MCGFailureInformation and just starts the T316 timer, while the T316 timer at the UE side has already been running for a while, even close to expiry. Therefore, having SN maintain T316 timer is not practical since the gap between when to start T316 timer at the UE side and SN side could be huge.

Observation 1: The network-based solution has its advantage:

- It requires that SN receives the MCGFailureInformation successfully.

- The T316 timer maintained at the SN side is inaccurate for the purpose of estimating the elapsed T316 at the UE side.

Proposal 8: Agree on a  UE based solution for option (c) and send an LS to RAN2.

The main benefit of the network-based solution seems to be that this would enable a limited support for pre-Rel-18 UEs. This could be interesting to explore. We would discuss further whether the benefits are big enough to motivate the inclusion of this solution as well.

## 2.2	MRO for voice fallback

For the support of shortly RLF case, there is stage 3 impact to be captured. We intend to illustrate the detailed procedure of RLF report forwarding in case of shortly RLF, as shown in the fig.1. The UE can report LTE RLF report to the E-UTRA node. The E-UTRA node fetching the RLF report should forward it to the failed E-UTRA node via failure information kind of IE. When the failed E-UTRA node identifies the source NR node as the responsible one for the failure, it should forward the handover report to the source NR node. Currently, there is only the RLF report container delivery for the too early inter-system HO for forwarding procedures between E-TURA and NR listed as below:

9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |
| >Too early Inter-system HO |  |  |  |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |
| >Inter-system Unnecessary HO |  |  |  |  |
| [……] |  |  |  |  |

For shortly RLF case of case1, it is proposed to support the forwarding of the HO report from the failed E-UTRA node to the source NR node. The possible enhancements of the forwarding scheme are showed as below, including the potential impact on the messages in the network interface. Specifically, the E-UTRA node forwards the HO report within the eNB Configuration transfer message via S1 interface to MME, and the AMF further forwards the HO report within the Downlink RAN Configuration transfer message via NG interface to the source NR node.

Fig1. forwarding in case of reporting to the E-UTRA node

According to TS 36.413, eNB Configuration transfer message uses the IE as defined in TS 38.413 to transfer inter-system SON configuration. So there is only specification impact on TS 38.413.

### 9.1.16	eNB CONFIGURATION TRANSFER

This message is sent by the eNB in order to transfer RAN configuration information.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.2.3.26 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | 9.2.3.26a |  | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | OCTET STRING | Contains the Inter-system SON Configuration Transfer IE as defined in TS 38.413 [44]. | YES | ignore |

Proposal 4:  RAN3 introduces related inter-system HO Report in the eNB Configuration transfer in S1 interface and Downlink RAN configuration transfer in NG interface.

The corresponding TP in the stage 3 38.413 are attached in the Annex B of section 4.

# 3	Conclusion

This document proposes:

- Proposal 1:  Introduce the T316 related triggering condition for defining the near failure scenario of fast MCG recovery.

- Proposal 2:   Introduce the following triggering conditions regarding SCG near failure for defining the near failure scenario of fast MCG recovery：

- the ratio of SCG T310

- the ratio of SCG T312

- the number of preamble transmissions towards SCG has reached a threshold

- the number of SCG RLC retransmission has reached a threshold

Observation 1: The network-based solution has its advantage:

- It requires that SN receives the MCGFailureInformation successfully.

- The T316 timer maintained at the SN side is inaccurate for the purpose of estimating the elapsed T316 at the UE side.

Besides, whether the Rel-18 MRO features suppot pre-Rel.18 UEs is not in the scope of WID.

Proposal 3: UE reported solution is preferred than network-based solution.

Proposal 4:  RAN3 introduces related inter-system HO Report in the eNB Configuration transfer in S1 interface and Downlink RAN configuration transfer in NG interface.

The corresponding TP in the stage 3 38.413 are attached in the Annex 1 of section 4.

# Annex 1 TP to SON BL CR to TS 38.413

Start of the change

#### 9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |
| >Too early Inter-system HO |  |  |  |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |
| >Inter-system Unnecessary HO |  |  |  |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |
| >>Candidate Cell List |  | 1 |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |
| >>>>>Candidate PCI |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |
| >Inter-system Voice Fallback HO |  |  |  |  |
| >>Source Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for the HO. |
| >>Failure Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | M |  | 9.3.3.41 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |

Next change

### 9.4.5	Information Element Definitions

//skip unchanged part

id-TAI,

id-HFCNode-ID-new,

id-GlobalCable-ID-new,

id-SuccessfulPSCellChangeReportList,

id-IntersystemVoiceFallbackHO,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

//skip unchanged part

InterSystemHOReport ::= SEQUENCE {

handoverReportType		InterSystemHandoverReportType,

iE-Extensions			ProtocolExtensionContainer { { InterSystemHOReport-ExtIEs} }			OPTIONAL,

...

}

InterSystemHOReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InterSystemHandoverReportType ::= CHOICE {

tooearlyIntersystemHO				TooearlyIntersystemHO,

intersystemUnnecessaryHO			IntersystemUnnecessaryHO,

choice-Extensions		ProtocolIE-SingleContainer { { InterSystemHandoverReportType-ExtIEs} }

}

InterSystemHandoverReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-IntersystemVoiceFallbackHO CRITICALITY ignore TYPE IntersystemVoiceFallbackHO PRESENCE mandatory},

...

}

IntersystemUnnecessaryHO ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

earlyIRATHO				ENUMERATED {true, false, ...},

candidateCellList		CandidateCellList,

iE-Extensions			ProtocolExtensionContainer { { IntersystemUnnecessaryHO-ExtIEs} }			OPTIONAL,

...

}

IntersystemUnnecessaryHO-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemVoiceFallbackHO ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

uERLFReportContainer	UERLFReportContainer,

iE-Extensions			ProtocolExtensionContainer { { IntersystemVoiceFallbackHO-ExtIEs} }		OPTIONAL,

...

}

IntersystemVoiceFallbackHO-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {
...
}

### 9.4.7	Constant Definitions

//skip unchanged part

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-SuccessfulPSCellChangeReportList						ProtocolIE-ID ::= 3xx

id-IntersystemVoiceFallbackHO 						ProtocolIE-ID ::= 3xx

END

-- ASN1STOP

End of the  change