3GPP TSG-RAN WG3 Meeting #121	R3-233904

Toulouse, France, 21 - 25 August 2023

Agenda item:	26.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Interworking with TSN network and RAN feedback

Document for:	Discussion and Decision

# 1	Introduction

The RAN3-lead work item NR Timing Resiliency and URLLC enhancements includes the following two objectives [1]:

2. Interworking with TSN network deployed in the transport network [RAN3]: a. RAN impacts due to 5G System integration with TSN Transport network Note 2: This objective has dependency on the progress of SA2 and CT4. 3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a. RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication.

Interworking with TSN network has not been discussed yet in RAN3, so in this paper we provide a high-level overview of the functionality relevant to RAN3 and evaluate the RAN3 impacts.

Regarding RAN feedback for downstream/upstream scheduling adaptation, this topic has been discussed in RAN3 for several meetings with good progress. In this paper, we resolve remaining open issues.

# 2	Discussion

## 2.1	Interworking with TSN network

In a Time Sensitive Networking (TSN) enabled Transport Network (TN), the NG-RAN nodes and the UPFs act as End Stations and may optionally support the TSN Talker and Listener (TL) functionality (i.e. AN-TL in the NG-RAN node and CN-TL in the UPF). The AN-TL and CN-TL may extend each End Station with a set of IEEE 802.1 functionalities, which allows to perform the stream transformation according to IEEE Std 802.1Q-2022 clause 46.1.4, the scheduling of the stream according to IEEE Std 802.1Q-2022 clause 8.6.8.4 and clause 46.2.3.5, and the exchange of topology information according to IEEE Std 802.1AB-2016. The SMF/CUC can communicate with the AN-TL and CN-TL to:

-	retrieve the End Station capabilities (list of interfaces and interface capabilities) of the AN-TL and CN-TL; and

-	configure TSN features in the AN-TL/CN-TL (e.g. for stream identification, stream association with TSN resources, stream transformation, hold and buffer functionality).

From RAN3 perspective the extended IEEE 802.1 functionalities of the AN-TL can be seen as out-of-scope. Only the presence of AN-TL in the NG-RAN node and the transparent information exchange between SMF/CUC and AN-TL needs to be considered.

The communication between AN-TL and SMF/CUC is based on TL-Containers sent within NGAP signaling to be specified by RAN3 in TS 38.413, while the communication between CN-TL and SMF/CUC is based on TL-Containers sent within PFCP Session related signaling specified by CT4 in TS 29.244. The related stage 2 call flows are specified in 3GPP TS 23.502.

The SMF/CUC must know whether the NG-RAN/UPF supports AN-TL/CN-TL functionality. SA2 has not specified in detail how the support of AN-TL/CN-TL is known to the SMF/CUC and leaves it up to stage 3 to define the best solution (Editor’s note in TS 23.501, Annex M.1). The preferred solution in CN is to enable the UPF to signal its support of the CN-TL functionality to the SMF/CUC by setting a CN-TL flag in the UPF features IE to avoid the need for pre-configuration of CN-TL capabilities at the SMF/CUC or the need for SMF/CUC initiating a timer-based CN-TL capability query at the UPF. It would be natural to adopt a similar approach to enable the NG-RAN node to signal its support of the AN-TL functionality to the SMF/CUC via NGAP (e.g. in the NG SETUP REQUEST and RAN CONFIGURATION UPDATE messages). Alternatively, the support of AN-TL in the NG-RAN node can be pre-configured at the SMF/CUC.

Proposal 1.1:	RAN3 should decide whether AN-TL support at the NG-RAN node is known by the SMF/CUC via control plane signalling or via OAM pre-configuration.

If the NG-RAN node does not support the stream transformation according to IEEE Std 802.1Q-2022 clause 46.1.4 (e.g. AN-TL is not supported or stream transformation is performed in TN), the SMF/CUC may instruct the NG-RAN node to assign a separate N3 tunnel end point address for each QoS flow that may carry a stream so that the TN can distinguish the QoS flows based on the N3 tunnel destination IP addresses (TS 23.501, Annex M.1). This means that TSN information is optionally needed per QoS flow in the PDU Session Resource Setup Request Transfer IE.

Proposal 1.2:	Introduce an optional TSN Information IE in the QoS Flow Setup Request Item IE of the PDU Session Resource Setup Request Transfer IE. Details of the IE are FFS pending CT4 progress.

If AN-TL is supported in the NG-RAN node, the TSN specific functions shall be processed in the AN-TL and the get/set procedure with the TL-container transfer shall be supported as specified in TS23.501 clause 5.28a.2 and Annex M.1.

Proposal 1.3:	Introduce a new AMF-initiated class 2 procedure and a new RAN-initiated class 2 procedure in NGAP to enable a TL Container to be transparently transferred between the SMF and NG-RAN node. Details of the new procedures, including whether they are UE-associated or non-UE associated, are pending CT4 progress.

## 2.2	RAN feedback for scheduling adaptation

### 2.2.1	Proactive RAN feedback after establishment of the TSC QoS flow

At RAN3#120, the following open issue was captured in the Chair’s Minutes:

RAN3#120 Chair’s Minutes: Is proactive RAN feedback applicable/relevant after the initial establishment of the TSC QoS flow?

RAN3 has discussed whether proactive feedback should be allowed within the Handover Request Acknowledge Transfer IE of the HANDOVER REQUEST ACKNOWLEDGE message, and/or the Path Switch Request Transfer IE of the PATH SWITCH REQUEST message. We note that TS 23.501 defines proactive feedback as follows:

“Proactive RAN feedback for adaptation of Burst Arrival Time and Periodicity: NG-RAN may provide a Burst Arrival Time offset and an adjusted Periodicity as part of QoS flow establishment or modification as illustrated in clause 5.27.2.5.2;”

This indicates that proactive RAN feedback is provided only in the session setup signaling before the application starts sending periodic data bursts, therefore the target gNB does not provide feedback after handover. The proactive RAN feedback is to be used when the application is not able to change its transmission schedule in the middle of the session.

Observation 1: Enhancements are not needed in handover signaling for proactive RAN feedback.

As agreed at RAN3#119bis, the extended TSC Assistance Information IE can be included in multiple messages including the HANDOVER REQUEST message. When receiving the TSC Assistance Information IE in the HANDOVER REQUEST message, the target gNB shall ignore the BAT window and Periodicity Range if included.

Proposal 2.1:	In NGAP, clarify (e.g. in semantics description) that target gNB shall ignore BAT Window and Periodicity Range IEs if received in a HANDOVER REQUEST message.

Proposal 2.2:	In XnAP, the BAT Window and Periodicity Range IEs are not needed in the TSC Assistance Information IE.

### 2.2.2	Additional stage 3 encoding details

For reactive RAN feedback, the encoding of the BAT offset is described in TS 23.501 as follows:

-	NG-RAN determines BAT offset value in reference to the current arrival time of the bursts experienced by RAN in DL and by UE in UL.

The legacy Burst Arrival Time IE is encoded as defined in 9.3.1.133 of TS 38.413, with 1us granularity. Therefore, it should be possible to indicate a BAT offset and BAT window with 1us granularity. Regarding the maximum range, it seems reasonable to allow a BAT window centered around the maximum periodicity value (640000us), i.e. an offset of plus or minus 320000us.

This leads to the following encoding for the Burst Arrival Time Window IE and the Burst Arrival Time Offset IE:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time Window Start | M |  | INTEGER (0..320000, …) | Start of the burst arrival time window calculated with reference to the Burst Arrival Time IE, expressed in units of 1 us. Integer values are negative. |
| Burst Arrival Time Window End | M |  | INTEGER (0..320000, …) | End of the burst arrival time window calculated with reference to the Burst Arrival Time IE, expressed in units of 1 us. Integer values are positive. |

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time Offset | M |  | INTEGER (-320000..320000, …) | Burst arrival time offset expressed in units of 1 us. |

According to TS 23.501, the BAT offset is calculated with reference to the Burst Arrival Time IE in case of proactive RAN feedback, and calculated with reference to the current arrival time of the bursts experienced by the RAN in DL and by the UE in UL in case of reactive RAN feedback.

Regarding the encoding of adjusted periodicity, TS 23.501 states that it “shall be within the Periodicity Range”. Therefore, the Adjusted Periodicity IE can be encoded the same as the legacy Periodicity IE.

The Capability for BAT Adaptation IE is a simple indicator, and therefore can be encoded as ENUMERATED (true, …).

Proposal 2.3:	The burst arrival time window is encoded as a Burst Arrival Time Window Start IE and Burst Arrival Time Window End IE, expressed in units of 1us (i.e. same granularity as BAT) and range -320ms to 320ms.

Proposal 2.4:	The Burst Arrival Time Offset IE is between -320ms to 320ms, with granularity of 1us.

Proposal 2.5:	The Adjusted Periodicity IE reuses the same encoding as the existing Periodicity IE.

Proposal 2.6:	The Capability for BAT Adaptation IE is encoded as ENUMERATED (true, …)

### 2.2.3	F1AP impacts

Regarding proactive RAN feedback, the gNB provides the feedback during PDU session resource setup/modification when the TSC QoS flow is being established, i.e. before the application starts transmitting periodic data bursts. The gNB determines the feedback based on TSC traffic characteristics and application adaptation capabilities, QoS profile of the TSC QoS flow and radio configuration. Radio configuration information that can be taken into account includes TDD configuration and scheduling.

The gNB itself can adapt to the traffic pattern of application stream by means of scheduling and dynamic TDD, however radio configuration may impose constraints on when gNB’s adaptation mechanisms can be applied, main use case being a common TDD configuration.  Proactive feedback and feedback in general are supposed to be used to avoid burst arrival times which are beyond the adaptation capabilities of the gNB.

The gNB-CU is aware of the TDD configuration, and it also configures uplink grants for type 1 CGs (Configured Scheduling). For the type 2 CGs and SPS (Semi Persistent Scheduling) the gNB-CU configures higher layer parameters to the UE, but the gNB-DU activates/deactivates UL grants by L1 DCI signaling. The DCI signaling provides a flexible and fast way to modify allocated resources, therefore it does need to be taken into account for proactive feedback determination.

Observation 2.	For proactive feedback, the gNB-CU has all needed information to determine the proactive feedback for both downlink and uplink.

Currently, the TSC Assistance Information IE (Burst Arrival Time and Periodicity) can be sent to the gNB-DU in the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages. If the gNB-CU determines RAN feedback in proactive method, the gNB-CU is able to derive the “new” BAT and periodicity from the BAT Window and Periodicity Range received in the TSCAI from the 5GC. Therefore, when constructing the TSC Assistance Information IE to be sent to the gNB-DU over F1AP, the existing IEs can be reused, but the gNB-CU includes the “new” BAT and periodicity values in the TSC Assistance Information IE sent to the gNB-DU.

Observation 3: Proactive feedback does not require any changes to F1AP messages.

Regarding Reactive RAN feedback, RAN2 has decided in RAN2#122 -meeting that [2]:

“No new UE report is needed for objective 3 on BAT offset derivation.  The XR mechanism can be used if the network configures it, otherwise we can use legacy BSR.”

The XR mechanisms and legacy BSR is based on MAC layer signaling which is not visible at the gNB-CU. Therefore, support for reactive UL RAN feedback seems needed over F1AP.

Observation 4: Reactive feedback requires F1AP support.

Therefore, the following is proposed:

Proposal 2.7:	To support reactive feedback over F1AP, the TSC Assistance Information IE and the NOTIFY message should be enhanced to mirror relevant IEs from NGAP.

# 3	Conclusions

In this paper we evaluated the RAN3 impacts of interworking with TSN network and proposed the following:

Proposal 1.1:	RAN3 should decide whether AN-TL support at the NG-RAN node is known by the SMF/CUC via control plane signalling or via OAM pre-configuration.

Proposal 1.2:	Introduce an optional TSN Information IE in the QoS Flow Setup Request Item IE of the PDU Session Resource Setup Request Transfer IE. Details of the IE are FFS pending CT4 progress.

Proposal 1.3:	Introduce a new AMF-initiated class 2 procedure and a new RAN-initiated class 2 procedure in NGAP to enable a TL Container to be transparently transferred between the SMF and NG-RAN node. Details of the new procedures, including whether they are UE-associated or non-UE associated, are pending CT4 progress.

In addition, we analysed the remaining open issues related to RAN feedback for downstream/upstream scheduling adaptation and proposed the following.

Proposal 2.1:	In NGAP, clarify (e.g. in semantics description) that target gNB shall ignore BAT Window and Periodicity Range IEs if received in a HANDOVER REQUEST message.

Proposal 2.2:	In XnAP, the BAT Window and Periodicity Range IEs are not needed in the TSC Assistance Information IE.

Proposal 2.3:	The burst arrival time window is encoded as a Burst Arrival Time Window Start IE and Burst Arrival Time Window End IE, expressed in units of 1us (i.e. same granularity as BAT) and range -320ms to 320ms.

Proposal 2.4:	The Burst Arrival Time Offset IE is between -320ms to 320ms, with granularity of 1us.

Proposal 2.5:	The Adjusted Periodicity IE reuses the same encoding as the existing Periodicity IE.

Proposal 2.6:	The Capability for BAT Adaptation IE is encoded as ENUMERATED (true, …)

Proposal 2.7:	To support reactive feedback over F1AP, the TSC Assistance Information IE and the NOTIFY message should be enhanced to mirror relevant IEs from NGAP.

Stage 3 text proposals for NGAP, XnAP, and F1AP are provided below in Annex A, Annex B, and Annex C respectively.

# References

- RP-231106 Revised WID on NR Timing Resiliency and URLLC enhancements, Nokia, Nokia Shanghai Bell

- Draft Report of 3GPP TSG RAN WG2 meeting #122 Incheon, Korea

# Annex	A: Text Proposal for TS 38.413

first modification

### 9.2.x		TL Container Transport Messages

#### 9.2.x.1	DOWNLINK TL CONTAINER TRANSPORT

This message is sent by the AMF to transfer a TL Container to the NG-RAN node.

Direction: AMF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| [FFS] |  |  |  |  |  |  |

#### 9.2.x.2	UPLINK RAN TL CONTAINER TRANSPORT

This message is sent by the NG-RAN node to transfer a TL Container to the AMF.

Direction: NG-RAN node ® AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| [FFS] |  |  |  |  |  |  |

next modification

#### 9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  | - |  |
| Survival Time | O |  | 9.3.1.221 |  | YES | ignore |
| CHOICE RAN feedback type |  | 0..1 |  |  | YES | ignore |
| >proactive |  |  |  | Ignored if received in the HANDOVER REQUEST message |  |  |
| >>Burst Arrival Time Window | M |  | 9.3.1.z1 |  | - |  |
| >>Periodicity Range | O |  | 9.3.1.z2 |  | - |  |
| >reactive |  |  |  |  |  |  |
| >>Capability for BAT Adaptation | M |  | ENUMERATED (true, …) |  | - |  |

next modification

#### 9.3.1.z1	Burst Arrival Time Window

This IE indicates the burst arrival time window of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time Window Start | M |  | INTEGER (0..320000, …) | Start of the burst arrival time window calculated with reference to the Burst Arrival Time IE, expressed in units of 1 us. Integer values are negative. |
| Burst Arrival Time Window End | M |  | INTEGER (0..320000, …) | End of the burst arrival time window calculated with reference to the Burst Arrival Time IE, expressed in units of 1 us. Integer values are positive. |

next modification

next modification

#### 9.3.1.z5	TSC Feedback Information

This IE provides the TSC feedback information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time Offset | M |  | INTEGER (-320000..320000, …) | Burst arrival time offset expressed in units of 1 us. |
| Adjusted Periodicity | O |  | Periodicity 9.3.1.132 | Not applicable to reactive RAN feedback. |

next modification

#### 9.3.1.y1	TSN Information

This IE provides the TSN related information for a QoS flow (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |  |  |

last modification

#### 9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 | This IE shall be present when at least one Non-GBR QoS flow is being setup and is ignored otherwise. | YES | reject |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of UL PDUs for split PDU session. | YES | reject |
| Data Forwarding Not Possible | O |  | 9.3.1.63 | This IE may be present in case of HANDOVER REQUEST message and is ignored otherwise. | YES | reject |
| PDU Session Type | M |  | 9.3.1.52 |  | YES | reject |
| Security Indication | O |  | 9.3.1.27 |  | YES | reject |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Setup Request List |  | 1 |  |  | YES | reject |
| >QoS Flow Setup Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| >>TSN Information | O |  | 9.3.1.y1 |  | YES | ignore |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.64 | This IE may be present in case of inter-system handover and intra-system handover. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. | YES | ignore |
| Additional Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of redundant UL PDUs for split PDU session. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.3.1.136 |  | YES | ignore |
| MBS Session Setup Request List | O |  | 9.3.1.211 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

# Annex	B: Text Proposal for TS 38.423

first modification

#### 9.2.3.115	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.2.3.116 | Periodicity as specified in TS 23.501 [7]. | – |  |
| Burst Arrival Time | O |  | 9.2.3.117 | Burst Arrival Time as specified in TS 23.501 [7]. | – |  |
| Survival Time | O |  | 9.2.3.152 |  | YES | Ignore |
| CHOICE RAN feedback type |  | 0..1 |  |  | YES | ignore |
| >reactive |  |  |  |  |  |  |
| >>Capability for BAT Adaptation | M |  | ENUMERATED (true, …) |  | - |  |

last modification

# Annex	C: Text Proposal for TS 38.473

first modification

#### 9.2.2.13		NOTIFY

This message is sent by the gNB-DU to notify the gNB-CU that the QoS for already established DRBs associated with notification control is not fulfilled any longer or it is fulfilled again.

Direction: gNB-DU ® gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Notify List |  | 1 |  |  | YES | reject |
| >DRB Notify Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Notification Cause | M |  | ENUMERATED(Fulfilled, Not-Fulfilled, ...) |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters set Notify Index 9.3.1.124 | Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameter set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.z4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

next modification

#### 9.3.1.142	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [21]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.143 | Periodicity as specified in TS 23.501 [21]. | - |  |
| Burst Arrival Time | O |  | 9.3.1.144 | Burst Arrival Time as specified in TS 23.501 [21]. | - |  |
| Survival Time | O |  | 9.3.1.231 |  | YES | ignore |
| CHOICE RAN feedback type |  | 0..1 |  |  | YES | ignore |
| >reactive |  |  |  |  |  |  |
| >>Capability for BAT Adaptation | M |  | ENUMERATED (true, …) |  | - |  |

last modification

#### 9.3.1.z4	TSC Traffic Characteristics Feedback

This IE provides the TSC traffic characteristics feedback of a TSC QoS flow (see TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Feedback Information Downlink | O |  | TSC Feedback Information 9.3.1.z5 |  |
| TSC Feedback Information Uplink | O |  | TSC Feedback Information 9.3.1.z5 |  |

#### 9.3.1.z5	TSC Feedback Information

This IE provides the TSC feedback information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time Offset | M |  | INTEGER (-320000..320000, …) | Burst arrival time offset expressed in units of 1 us. |