3GPP TSG-RAN WG3 Meeting #121	R3-234086

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	14.4

Source:	Huawei

Title:	(TP to TS 38.423 BL CR) Consideration on Subsequent CPAC

Document for:	Other

# 1	Introduction

In the RAN3#120 meeting, the following agreements were made for selective activation of SCG (Subsequent CPAC):

Add a new IE as a sub IE of the Conditional PSCell Change Information Required IE in the S-NODE CHANGE REQUIRED message to indicate that the request is for SCG Selective Activation.

It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container).

Note: This can be revisited based on RAN2 progress.

Add a new IE as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message to indicate that the request is for SCG Selective Activation.

It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container).

Note: This can be revisited based on RAN2 progress.

Introduce a new indicator for selective activation in the S-NODE MODIFICATION REQUEST message to indicate that the request is for SCG selective activation. FFS on the scenarios.

Introduce a new indicator for selective activation in the Xn-U address indication message to indicate that the request is for SCG selective activation. FFS on the scenarios.

In the meanwhile, some agreements have been achieved in RAN2#122 meeting:

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

The UE shall skip the condition evaluation for a candidate which is a current PScell.

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

Rapporteur take initiative on naming offline

Terminology is “Subsequent CPAC”

Taking the RAN2 and RAN3 agreements into account, for the agreed solutions and FFSs, this paper presents further analysis and corresponding proposals.

# 2	Discussion

## 2.1	Indicators/Information exchanged

RAN2 agreed to use the terminology “Subsequent CPAC”, therefore we sould update the title/IE name in the existing 38.423 BL CR.

Proposal 0: Rename the title/IE name in the existing TS38.423 BL CR to use the new terminology “Subsequent CPAC”.

To support MN/SN initiated inter-SN subsequent CPAC, the candidate SN need to indicate to MN whether the candidate SCG configuration is a delta with respect to the reference SCG configuration or not. Besides that, RAN2 also agreed that the reference SCG configuration could be empty. Hence, the candidate SN generates the candidate complete SCG configuration with respect to empty referecence SCG configuration. The possible type of RRC configuration associated to candidate SCG configuration could be: full config, delta config, and complete config. Extension of existing RRC Config Indication IE is needed to include a new code point to indicate complete config.

In legacy Rel-17 CPAC, the RRC Config Indication IE to indicate the type of RRC configuration is provided per candidate SN. However, since the SN may comprise of one or more DUs, the DUs has difference capability in recognizing the reference config provided by MN, the cells from different DUs should be able to use different type of RRC confirnations, therefore, it is preferred to provide the RRC Config Indication IE for each candidate PSCell.

Proposal 1: Enhance the existing RRC Config Indication IE to include a new code point to indicate complete config.

Proposal 2: Provide the RRC Config Indication IE from Candidate SN to the MN in a per PSCell granularity in SN Addition Request ACK.

In SN initiated inter-SN subsequent CPAC, when source SN intends to trigger an update of subsequent CPAC execution condition and/or corresponding SCG measurement configuration for subsequent CPAC, the source SN can trigger SN Modification Required procedure, and if the candidate SN intends to update the list of PSCells prepared at the candidate SN, the SN Modification Required procedure could also be triggered. But there seems no strong need to introduce new IE in the messages in this procedure, since the CPAC information exchanged in the related messages introduced in Rel-17 CPAC can be reused by the Rel-18 subseuqent CPAC.

Proposal 3: There is no clear motivation to introduce new IE in the messages of SN initiated SN Modificaiton procedure.

To support SN initiated inter-SN subsequent CPAC, it was agreed last meeting to introduce a sub IE of the Conditional PSCell Change Information Required IE in the S-NODE CHANGE REQUIRED message, a sub IE for indicate the subseuqnet CPAC, but there seems no strong motidcation to introduce new IE in the SN CHANGE CONFIRM message as the existing IEs could be reused.

Proposal 4: there is no clear motivation to introduce new IE in the S-Node Change CONFIRM message.

## 2.2 Support of data forwarding in Subsequent CPAC

By default, both indirect and direct early data forwarding should be supported in subsequent CPAC. During the preparation of candidate SNs, the MN gets the data forwarding addresses from each candidate SN via SN addition request Acknowledge message. To support early data forwarding after configuration of subsequent CPAC and each time of CPAC execution, the new SN needs to get the early data forwarding addresses for the other candidate SNs.

There are two approaches listed as below on how the MN notifies the source SN about the data forwarding addresses of the other candidate SNs:

- Option1: the MN provides the data forwarding addresses of all the candidate SNs to all candidate SNs after Subsequent CPAC preparation

  - Via Xn-U Address Indication procedures, after initial subsequent CPAC configuration.

  - all candidate SNs get the data forwarding addresses of other candidate SNs at the very beginning, and when a candidate SN becomes to the source SN, this new source SN is able to use the forwarding addresses without further XnAP signalling exchanging.

  - But in case some candidate SNs cancels the subsequent CPAC preparation or would like to change the data forwarding addresses, or new candidate SN added, the MN needs to update the information to all other candidiate SNs.

- Option2: the MN provides the data forwarding address of the other candidate SNs only to the current source SN

  - via the Xn-U Address Indication, or via the SN RRC Reconfiguration complete

  - the up to date data forwarding addresses are provided to the new source SN when it needs to start early data forwarding

Comparing with these two solutions, we slightly prefer option 2 as it is simpler and each candidate SNs does not need to store the data forwarding addresses for all other candidate SNs and to be updated when something is changed.

For option 2, we need futher discuss which procedure to be used by the MN to inform the Source SN:

- Alt 1: single enhanced Xn-U Address Indication procedure which carries the data forwarding addresses of all other candidate SNs

- Alt 2: multiple Xn-U Address Indication procedures, and each procedure carries data forwarding addresses of one candidate SN

- Alt 3: single SN RRC Reconfiguration Complete procedure which carries the data forwarding addresses of all other candidate SNs

Considering that after each time of CPC execution, the MN needs to inform the new source SN about the selected candidate PSCell and forward the SN RRCReconfigurationComplete message, Alt 3 is more efficient and straight forward.

Proposal 5: Enhance SN RRC Reconfiguration Complete procedure to carry the data forwarding addresses of all the other candidate SNs, to be triggered by the MN towards the new source SN after each time of subsequent CPAC execution.

Once the UE select a candidate SN (new source SN) to access to, the old source SN should stop providing data to the UE and start late data forwarding to the new source SN. It should be pointed out that the legacy Xn-U address indication message could be reused for MN to inform the old source SN (the current SN) to stop providing user date to UE and start performing late data forwarding towards the new source SN.

Proposal 6: Reuse legacy Xn-U Address Indication procedure for MN to inform the old source SN (the current SN) to stop providing user date to UE and start performing late data forwarding towards the new source SN.

## 2.3 Reference configuration provided from MN to candidate SN

RAN2 has assumed that there is a single reference configuration. The MN needs to provide the reference configuration to candidate SNs for generating the candidate PSCell configuration. One possibility is to include the reference configuration in the existing M-NG-RAN node to S-NG-RAN node Container IE, which means RAN2 should include the reference configuration in the CG-ConfigInfo message, similar to the current sourceConfigSCG IE for Rel-17 delta candidate SCG configuration. Another possibility is to include the reference configuration as a separate IE. Currently, RAN2 is discussing whether the reference configuration is a separate RRC message, and whether the reference configuration includes the MCG part of reference configuration. The details are pending to RAN2 decision.

From our view, it is likely that the reference configuration includes the MCG part because different PSCell requires difference MCG configuration accordingly. In the RAN3 perspective, MN only needs to provide the SCG referene configuration. Hence, a separate XnAP IE for SCG reference configuration is preferred.

Proposal 7: include a Reference SCG Configuration IE as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message.

# 3	Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 0: Rename the title/IE name in the existing TS38.423 BL CR to use the new terminology “Subsequent CPAC”.

Proposal 1: Enhance the existing RRC Config Indication IE to include a new code point to indicate complete config.

Proposal 2: Provide the RRC Config Indication IE from Candidate SN to the MN in a per PSCell granularity in SN Addition Request ACK.

Proposal 3: There is no clear motivation to introduce new IE in the messages of SN initiated SN Modificaiton procedure.

Proposal 4: there is no clear motivation to introduce new IE in the S-Node Change CONFIRM message.

Proposal 5: Enhance SN RRC Reconfiguration Complete procedure to carry the data forwarding addresses of all the other candidate SNs, to be triggered by the MN towards the new source SN after each time of subsequent CPAC execution.

Proposal 6: Reuse legacy Xn-U Address Indication procedure for MN to inform the old source SN (the current SN) to stop providing user date to UE and start performing late data forwarding towards the new source SN.

Proposal 7: include a Reference SCG Configuration IE as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message.

# 5. TP to TS 38.423 BL CR

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >Subsequent CPAC Information Request | O |  | 9.2.3.xxx | This information is used for Subsequent CPAC. FFS on the details. Editor’s Note: This may be revisited based on RAN2 progress. | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

--------------Next Change---------------

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| // Skip unchanged part |  |  |  |  |  |  |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

---------------Next Change---------------

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  | – |  |
| >>Configuration successfully applied |  |  |  |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10] or the RRCConnectionReconfigurationComplete message as defined in subclause 6.2.2 of TS 36.331 [14]. | – |  |
| >>>PDU Session SN Change Inform List | O |  | 9.2.1.xxx | this IE would be present in case of Subsequent CPAC execution. | YES | ignore |
| >>Configuration rejected by the M-NG-RAN node |  |  |  |  | – |  |
| >>>Cause | M |  | 9.2.3.2 |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |

---------------Next Change---------------

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional PSCell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>CPC Indicator | M |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |
| >Subsequent CPAC Information Required | O |  | FFS | This information is used for subsequent CPAC. FFS on the details. Editor’s Note: This may be revisited based on RAN2 progress. | YES | ignore |

---------------Next Change---------------

#### 9.1.2.12	S-NODE CHANGE CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node change was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session SN Change Confirm List | O |  | 9.2.1.xxx |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Conditional PSCell Change Information Confirm | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |
| maxnoofTargetSNsMinusOne | Maximum no. of the target S-NG-RAN nodes minus 1. Value is 7 |

---------------Next Change---------------

#### 9.2.1.xxx	PDU Session SN Change Inform List

This IE indicates the PDU Session SN Change Inform List.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| >PDU Session SN Change Confirm Item |  | 1 .. <maxnoof PDUsessions> |  | NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present in a PDU Session SN Change Confirm Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Confirm Info – SN terminated | O |  | 9.2.1.19 |  | – |  |
| >>Additional List of PDU Session Resource Change Confirm Info – SN Terminated |  | 0..1 |  | This IE would be present only if multiple candidate target SNs are prepared in case of SN initiated inter-SN CPC. | YES | ignore |
| >>>Additional list of PDU Session Resource Change Confirm Info – SN Terminated-Item |  | 1 .. <maxnoofTargetSNsMinusOne> |  |  | – |  |
| >>>>PDU Session Resource Change Confirm Info – SN terminated | M |  | 9.2.1.19 |  | – |  |

---------------Next Change---------------

#### 9.2.3.72	RRC Config Indication

This IE indicates the type of RRC configuration used at the S-NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RRC Config Indication | M |  | ENUMERATED (full config, delta config, complete config, ...) |  |

---------------Next Change---------------

#### 9.2.3.xxx	Subsequent CPAC Information Request

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SCG Reference Config | O |  | OCTET STRING | Includes the [FFS – to be decided by RAN2] as defined in subclause [FFS – to be decided by RAN2] of TS 38.331 [10]. |

Editor’s note: The name and the encoding of the Subsequent CPAC Information Request IE are FFS

---------------Next Change---------------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

//skip unchanged part

-- **************************************************************

--

#### -- S-NODE RECONFIGURATION COMPLETE

--

-- **************************************************************

SNodeReconfigurationComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReconfigurationComplete-IEs}},

...

}

SNodeReconfigurationComplete-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-ResponseInfo-ReconfCompl				CRITICALITY ignore		TYPE ResponseInfo-ReconfCompl					PRESENCE mandatory},

...

}

ResponseInfo-ReconfCompl ::= SEQUENCE {

responseType-ReconfComplete		ResponseType-ReconfComplete,

iE-Extensions						ProtocolExtensionContainer { {ResponseInfo-ReconfCompl-ExtIEs} } OPTIONAL,

...

}

ResponseInfo-ReconfCompl-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResponseType-ReconfComplete ::= CHOICE {

configuration-successfully-applied			Configuration-successfully-applied,

configuration-rejected-by-M-NG-RANNode		Configuration-rejected-by-M-NG-RANNode,

choice-extension					ProtocolIE-Single-Container { {ResponseType-ReconfComplete-ExtIEs} }

}

ResponseType-ReconfComplete-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Configuration-successfully-applied ::= SEQUENCE {

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-successfully-applied-ExtIEs} } OPTIONAL,

...

}

Configuration-successfully-applied-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSession-SNChangeConfirm-List		CRITICALITY ignore	EXTENSION	PDUSession-SNChangeConfirm-List		PRESENCE optional},

...

}

Configuration-rejected-by-M-NG-RANNode ::= SEQUENCE {

cause											Cause,

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-rejected-by-M-NG-RANNode-ExtIEs} } OPTIONAL,

...

}

Configuration-rejected-by-M-NG-RANNode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

---------------Next Change---------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

//skip unchanged part

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-SubsequentCPACInfoRequest,

id-RRCConfigIndication,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

//skip unchanged part

CPACcandidatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCells-item

CPACcandidatePSCells-item ::= SEQUENCE {

pscell-id						NR-CGI,

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCells-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-RRCConfigIndication	CRITICALITY reject	EXTENSION RRCConfigIndication		PRESENCE optional},

...

}

CPCindicator ::= ENUMERATED {cpc-initiation, cpc-modification, cpc-cancellation, ...}

CPAInformationRequest ::= SEQUENCE {

max-no-of-pscells					INTEGER (1..maxnoofPSCellCandidates, ...),

cpac-EstimatedArrivalProbability	CHO-Probability 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CPAInformationRequest-ExtIEs} } OPTIONAL,

...

}

CPAInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SubsequentCPACInfoRequest	CRITICALITY ignore	EXTENSION SubsequentCPACInfoRequest	PRESENCE optional},

...

}

CPAInformationAck ::= SEQUENCE {

candidate-pscells					CPACcandidatePSCells-list,

iE-Extensions		ProtocolExtensionContainer { { CPAInformationAck-ExtIEs} } OPTIONAL,

...

}

CPAInformationAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

Reestablishment-Indication ::= ENUMERATED {

reestablished,

...

}

RFSP-Index ::= INTEGER (1..256)

RRCConfigIndication ::= ENUMERATED {

full-config,

delta-config,

complete-config,

...

}

RRCConnections::= SEQUENCE {

noofRRCConnections							NoofRRCConnections,

availableRRCConnectionCapacityValue		AvailableRRCConnectionCapacityValue,

iE-Extensions				ProtocolExtensionContainer { { RRCConnections-ExtIEs} }	OPTIONAL,

...

}

RRCConnections-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

SecondaryRATUsageInformation ::= SEQUENCE {

pDUSessionUsageReport		PDUSessionUsageReport				OPTIONAL,

qosFlowsUsageReportList		QoSFlowsUsageReportList				OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {SecondaryRATUsageInformation-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATUsageInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			ENUMERATED {required, preferred, not-needed, ...},

confidentialityProtectionIndication		ENUMERATED {required, preferred, not-needed, ...},

maximumIPdatarate						MaximumIPdatarate												OPTIONAL,

-- This IE shall be present if the Integrity Protection IE within the Security Indication IE is present and set to "required" or "preferred". --

iE-Extensions							ProtocolExtensionContainer { {SecurityIndication-ExtIEs} } OPTIONAL,

...

}

SecurityIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityResult ::= SEQUENCE {

integrityProtectionResult				ENUMERATED {performed, not-performed, ...},

confidentialityProtectionResult			ENUMERATED {performed, not-performed, ...},

iE-Extensions							ProtocolExtensionContainer { {SecurityResult-ExtIEs} } OPTIONAL,

...

}

SecurityResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SubsequentCPACInfoRequest ::= SEQUENCE {

sCG-ReferenceConfig						OCTET STRING 				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {SubsequentCPACInfoRequest-ExtIEs} } OPTIONAL,

...

}

SubsequentCPACInfoRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasurementConfiguration ::= SEQUENCE {

sensorMeasConfig 				SensorMeasConfig,

sensorMeasConfigNameList		SensorMeasConfigNameList			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SensorMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

SensorMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofSensorName)) OF SensorName

SensorMeasConfig::= ENUMERATED {setup,...}

SensorName ::= SEQUENCE {

uncompensatedBarometricConfig	ENUMERATED {true, ...}         OPTIONAL,

ueSpeedConfig					ENUMERATED {true, ...}         OPTIONAL,

ueOrientationConfig				ENUMERATED {true, ...}         OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SensorNameConfig-ExtIEs} } OPTIONAL,

...

}

SensorNameConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

------Start of the Next Change--------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

//skip unchanged part

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-SubsequentCPACInfoRequest																		ProtocolIE-ID ::= xx1

END

-- ASN1STOP

------End of the Changes--------