3GPP TSG-RAN WG3 Meeting #121	R3-234089

Toulouse, France, 21 – 25 Aug, 2023

Title: 	(TPs to MBS BL CRs) MBS reception in RAN sharing scenario

Source: 	Huawei, CBN

Agenda item:	15.2

Document Type:	other

# 1. Introduction

In RAN3#120 meeting, the following agreements/WAs/FFSs were achieved:

Add Associated Session ID IE in the same level of MBS Session ID IE in related NG/F1 messages. FFS on E1.

Include in the MBS Session Setup or Modification Response Transfer IE an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session in case of unicast transport.

Define a new class 1 procedure to allow the gNB/CU-CP requesting the setup of shared NG-U resources during an ongoing broadcast MBS session for unicast transport.

For the new procedure, FFS on the name of the message

Option 1:Broadcast Session Modification Required/Confirm/failure

Option 2:Broadcast Distribution Request/Response/failure or Broadcast session transport request/response/failure

Turn the following WA to agreement with removal of the text in the bracket:

In case of location dependent broadcast services, the gNB deduces identical broadcast content from the MBS Associated Session ID and the MBS Service Area information provided by the participating 5GCs.

The CU-CP sends the MBS Service Area to DU and DU determines whether to apply RAN sharing optimization over the radio. FFS on the determination between CU and DU for MOCN case.

FFS on where to apply the RAN sharing optimization.

FFS on how DU determines whether RAN sharing enhancement applies to the corresponding cells, i.e. based on which information and based on which behavior.

PDCP configuration selection in gNB-DU for multiple cell-ID case

Open issue 1: Which option should be used by DU to decide the MRB-PDCP-ConfigBroadcast?

- Option 1: OAM configure the MRB-PDCP-ConfigBroadcast of each MRB to CU or DU.

- Option 2: DU applied the MRB-PDCP-ConfigBroadcast of the first CU or the second CU. FFS whether DU needs to provide the used MRB-PDCP-ConfigBroadcast to CU.

FFS whether an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session also applies to multicast transport.

FFS on how many MBS session context procedure instances should be triggered in gNB-DU to establish the related MBS session contexts. It could be further discussed based on the whole picture of Broadcast context setup procedure which includes NG,F1 and E1.

FFS on how many F1-U tunnels should be established for multiple cell-ID broadcast cases.

In this contribution, we will further analysis on the support of MBS reception in RAN sharing efficiency optimization for the broadcast MBS service from different perspectives and provide the corresponding TPs to the BL CRs.

# 2. Discussion

## 2.1 Details of Associated Session ID

In the last meeting, it has been agreed to add Associated Session ID IE in the same level of MBS Session ID IE in the related NG/F1 messages, but the detailed content of the Associated Session ID IE is still FFS.

As defined in TS23.247, the Associated Session ID includes two IP addresses:

6.5.5 Associated Session ID In the case of network sharing, an Associated Session ID may be used as specified in clause 6.17. When the AF creates multiple broadcast MBS Sessions via different CNs to deliver the same content, it may provide the Associated Session ID which enables the NG-RAN to identify the multiple MBS Sessions delivering the same content. Source Specific IP Multicast Address specified in clause 6.5.3 may be used as Associated Session ID.

### 6.5.3 Source Specific IP Multicast Address The source specific IP multicast address is used to identify an Multicast MBS session and consists of two IP addresses, one is an IP unicast address used as source address in IP packets for identifying the source of the multicast service (e.g. AF/AS), the other is an IP multicast address used as destination address in related IP packets for identifying a multicast communication service associated with the source.

Proposal 1: Define the Associated Session ID IE as a sequence type including two IP addresses, i.e. source address (IP unicast address) and destination address (IP multicast address).

It is noticed that in the NGAP BL CR asn.1 part, the Associated Session ID is wrongly added in the BroadcastSessionModificationRequestIEs instead of BroadcastSessionSetupRequestIEs, from our understanding, the Associated Session ID should not be updated therefore we need to remove it in the modification procedure and add it in the setup procedure.

Proposal 1a: update the asn.1 part of NGAP BLCR, to remove the Associated Session ID in the BroadcastSessionModificationRequestIEs and add it in the BroadcastSessionSetupRequestIEs.

## 2.2 RAN triggered shared NG-U tunnel establishment

In the previous RAN3 meeting, it was agreed that “Define a new class 1 procedure to allow the gNB/CU-CP requesting the setup of shared NG-U resources during an ongoing broadcast MBS session for unicast transport.” To support this agreement, the new class 1 procedure has been proposed, whose name has two options as follows:’

Option 1: Broadcast Session Modification Required/Confirm/failure

Option 2: Broadcast Distribution Request/Response/failure or Broadcast session transport request/response/failure

From our view, the second option is more appropriate from the functionality point of view, and it is similar with the Distribution Setup procedure introduced for multicast. On the other hand, due to the different establishment procedures for multicast and broadcast, this new Broadcast Distribution Setup procedure should be able to establish multiple shared NG-U tunnels for location dependent service, which is different with the existing Distribution Setup procedure defined for multicast.

Proposal 2: Introduce a new Broadcast Distribution Setup procedure with REQUEST/RESPONSE/FAILURE messages, and this new procedure can establish multiple shared NG-U tunnels in case of location dependent service.

## 2.3 F1AP impact

### Single or multiple context

There was an to be continued issue marked last meeting:

To be continued: Whether, for MOCN, F1 supports establishment of a single Broadcast Context for multiple MBS sessions at the DU.

In MOCN scenario, both the gNB-CU and gNB-DU are shared by operators, although different TMGIs are provided by different operators, but they are actually linked to the same service and same radio resources, therefore we do not see strong need to establish multiple broadcast context. And it is noticed that in case the gNB-CU receives multiple requests from different core networks at the same time, it can send a single message with a list of TMGIs with the Associated Session ID, and by default the single set of F1-U tunnel(s) for the MRB(s) will be established. But in case of multiple context, as the UL and DL shared F1-U tunnel addresses are mandatory IEs, it will be a little bit complex to support single F1-U tunnel establishment.

Figure 1 illustrate the overall Broadcast context setup procedure which includes NG, F1 and E1, with the solution of single context:

Figure 1, Broadcast context setup procedure with single context solution

Proposal 3-1: In case of MOCN, single broadcast context and single set of F1-U tunnel(s) for the MRB(s) are established, the gNB-CU sends in a single F1AP: BROADCAST CONTEXT SETUP/MODIFICATION REQUEST message includes a list of MBS Session IDs.

In Rel-17, the configuration of broadcast is provided in MCCH, which is generated by the gNB-DU, in the meanwhile, the MRB PDCP configuration and mtch-neighbour cell are provided in the MBS CU to DU RRC Information IE. Considering that the shared DU needs to broadcast MRB Configuration over radio for different TMGIs.

For MOCN, assuming single Broadcast Context as proposed above, by default, the shared gNB-CU will provide one set of per MRB PDCP Configuration, and a list of {MBS Session ID + per Cell mtch-neighbourCell}, to the shared gNB-DU.

Proposal 3-2: In case of MOCN, the gNB-CU needs to provide one set of per MRB PDCP Configuration, and a list of {MBS Session ID + MBS Service Area+ per Cell mtch-neighbourCell}, to the shared gNB-DU.

But in case if only gNB-DU is shared (Network Sharing with multiple cell-ID broadcast), the gNB-DU (each logical gNB-DU) will receive different request messages from the non-shared gNB-CUs. Therefore, it’s quite possible for the gNB-DU receiving different PDCP configurations from different gNB-CUs.

In RAN3#119 meeting, there was an WA achieved as:

WA: In case of RAN sharing with multiple Cell ID broadcast, the entity controlling the logical DUs decides which MRB-PDCP-ConfigBroadcast to provide on MCCH. Details are FFS.

If the gNB-DU receives different PDCP configurations from different gNB-CUs, and gNB-DU can determine the ultimate PDCP configurations needed to be broadcasted. Although some companies have also proposed that gNB-DU can determine the ultimate PDCP configurations, gNB-DU needs to coordinate different gNB-CUs to achieve the same PDCP configurations, which needs more signaling interactions. However, in our opinion, the gNB-DU only needs to choose one of the PDCP configuration and broadcast it, which need no interaction among gNB-CUs. For example, the gNB-DU can broadcast the PDCP configuration provided by the first gNB-CU and use the data received via the shared F1-U tunnel with the first gNB-CU. Therefore, it’s proposed to turn the WA to agreement.

Proposal 3-3: In case of RAN sharing with multiple Cell ID broadcast, the entity controlling the logical DUs decides which MRB-PDCP-ConfigBroadcast to provide on MCCH.

Proposal 3-4: In case of RAN sharing with multiple Cell ID broadcast, the gNB-DU only needs to choose one of PDCP configurations from different gNB-CUs, and transmit the DL data received from the related gNB-CU to the UEs.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: Define the Associated Session ID IE as a sequence type including two IP addresses, i.e. source address (IP unicast address) and destination address (IP multicast address).

Proposal 1a: update the asn.1 part of NGAP BLCR, to remove the Associated Session ID in the BroadcastSessionModificationRequestIEs and add it in the BroadcastSessionSetupRequestIEs.

Proposal 2: Introduce a new Broadcast Distribution Setup procedure with REQUEST/RESPONSE/FAILURE messages, and this new procedure can establish multiple shared NG-U tunnels in case of location dependent service.

Proposal 3-1: In case of MOCN, single broadcast context and single set of F1-U tunnel(s) for the MRB(s) are established, the gNB-CU sends in a single F1AP: BROADCAST CONTEXT SETUP/MODIFICATION REQUEST message includes a list of MBS Session IDs.

Proposal 3-2: In case of MOCN, the gNB-CU needs to provide one set of per MRB PDCP Configuration, and a list of {MBS Session ID + MBS Service Area+ per Cell mtch-neighbourCell}, to the shared gNB-DU.

Proposal 3-3: In case of RAN sharing with multiple Cell ID broadcast, the entity controlling the logical DUs decides which MRB-PDCP-ConfigBroadcast to provide on MCCH.

Proposal 3-4: In case of RAN sharing with multiple Cell ID broadcast, the gNB-DU only needs to choose one of PDCP configurations from different gNB-CUs, and transmit the DL data received from the related gNB-CU to the UEs.

Based on these proposals and the previous agreements, the TPs to BL CRs are provided in the following sections.

# 4. TP to TS 38.401

## 6.5	MBS Session associations in NG-RAN Node

The following MBS Session associations are defined in the NG-RAN node to support NR MBS:

NG-RAN MBS session resource context: Encompasses CP and UP, transport and radio resources to support an MBS Session or multiple MBS sessions with the same associated session ID. For multicast it also encompasses the MBS Session state (active, de-activated) and information about joined UEs. If an MBS session resource within a gNB serves multiple MBS service areas, as specified in TS 23.247 [27] the same NG RAN MBS session resource context may be associated with multiple NG-U resources. For a multicast MBS session, NG-U resources are setup or released by the gNB upon UE mobility or UEs leaving or joining the multicast MBS session.

MBS Session context in a gNB-DU:

The definition of an MBS Session context in a gNB-DU applicable for broadcast and multicast.

An MBS Session context in a gNB-DU

-	is a block of information associated to an MBS Session, or associated to multiple MBS sessions with the same associated session ID, which may consist of one or several MRB Contexts;

-	corresponds to either one or several F1-U tunnels.

MRB Context in a gNB-DU:

An MRB Context is a block of information in a gNB-DU associated to an MRB (MRB “instance”). The gNB-DU sets up resources for each MRB Context in a gNB-DU associated to an MBS Session context

-	based on information provided within MBS Session Context related information as received by the gNB-DU (e.g. MRB QoS, MBS service area information, etc.), and,

-	for multicast, based on the UE Contexts established for RRC_CONNECTED UEs within the gNB-DU containing joining information of the UE for the respective multicast session.

-	for broadcast, the gNB-DU determines whether F1-U tunnels are setup per gNB-DU or per MBS Area Session ID served by the gNB-DU.

-	for multicast, the gNB-DU determines whether F1-U tunnels are setup per gNB-DU or per cell served by the DU or per MBS Area Session ID served by the gNB-DU or for ptp restransmissions or for a ptp-only MRB leg.

For multicast, for each MRB, the gNB-DU provides the MRB specific Uu configuration to the gNB-CU to configure the UE.

Multicast F1-U Context:

A Multicast F1-U Context is a block of information in a gNB-DU to control the F1-U tunnels associated to the MRB Contexts established for a multicast MBS session. A Multicast F1-U Context is either established per gNB-DU or per cell served by the gNB-DU or per MBS Area Session ID served by the gNB-DU or for ptp restransmissions or for ptp forwarding or for a ptp-only MRB leg. Several Multicast F1-U contexts may exist in parallel in a gNB-DU for the same multicast MBS session.

Allocation and usage of MRB ID values on NG-RAN interfaces for multicast MBS sessions:

-	F1 interface: an MRB ID signalled on an F1 interface instance identifies uniquely an MRB among all MRB contexts in an gNB-DU, allocated for all active multicast MBS sessions served by that gNB-DU. The value of each MRB ID is the same value as communicated to UEs served by that gNB-DU.

-	E1 interface: an MRB ID signalled on an E1 interface instance identifies uniquely an MRB among all MRBs allocated for a multicast MBS session.

-	Xn interface, NG interface: MRB IDs are signalled on Xn/NG interfaces for providing MBS QoS flow to MRB mapping information and data forwarding information from the source gNB.  The value of the MRB ID signalled on the Xn/NG interface is the same value as communicated to UEs at the source cell.

Allocation and usage of MRB ID values on NG-RAN interfaces for broadcast MBS sessions:

-	An MRB ID signalled on NG-RAN interfaces identifies uniquely an MRB among all MRBs allocated for a broadcast MBS sesssion, or multiple MBS sessions with the same associated session ID.

MBS-associated logical F1/E1-connection:

F1AP and E1AP provide means to exchange control plane messages associated with an MBS session, or multiple MBS sessions with the same associated session ID, over the respective F1/E1 interface.

An MBS-associated logical connection is established during the first F1AP/E1AP message exchange between the F1/E1 peer nodes.

The connection is maintained as long as MBS associated F1AP/E1AP messages need to be exchanged over the F1/E1 interface.

The MBS-associated logical F1-connection uses the identities gNB-CU MBS F1AP ID and gNB-DU MBS F1AP ID.

The MBS-associated logical E1-connection uses the identities gNB-CU-CP MBS E1AP ID and gNB-CU-UP MBS E1AP ID.

When a node (DU or CU or CU-CP and CU-UP) receives an MBS associated F1AP/E1AP message the node retrieves the associated MBS session based on the F1AP/E1AP ID.

MBS-associated signalling:

MBS-associated signalling is an exchange of F1AP/E1AP messages associated with one MBS session, or multiple MBS sessions with the same associated session ID, over the MBS-associated logical F1/E1-connection.

//skip unchanged part

### 7.7.x	Support of resource efficiency for RAN Sharing

### 7.7.x1	General

Editor’s Note:	TBD

### 7.7.x2	Support of resource efficiency for MOCN

F1AP supports resource efficiency in case both the gNB-CU and the gNB-DU are shared by multiple PLMNs for broadcast.

The gNB-CU and the gNB-DU manage an MBS Session context for multiple MBS sessions with the same associated session ID.

### 7.7.x3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

F1AP supports resource efficiency in RAN Sharing with multiple cell-ID broadcast.

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content, and decide which MRB-PDCP-ConfigBroadcast to provide on MCCH.

Applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	resolve different QoS requirements received from the participating 5GCs in an implementation specific way.

# 5. TP to TS 38.410

-----------------Start of the Changes-------------------

## 6.23	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Modification;

-	Broadcast Session Resource Release;

-	Broadcast Session Resource Release Required;

-	Broadcast Session Distribution;

-	Multicast Session Activation;

-	Multicast Session Deactivation;

-	Multicast Session Update;

-	Distribution Setup;

-	Distribution Release;

-----------------End of the Changes-------------------

# 6. TP to TS 38.413

-----------------Start of the Changes-------------------

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Broadcast Session Distribution | BROADCAST SESSION DISTRIBUTION REQUEST | BROADCAST SESSION DISTRIBUTION RESPONSE | BROADCAST SESSION DISTRIBUTION FAILURE |

-----------------Next of the Change-------------------

### 8.17.X Broadcast Session Distribution

#### 8.17.X.1 General

The purpose of the Broadcast Session Distribution procedure is to assign NG-U resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.X.2 Successful Operation

Figure 8.17.X.2-1: Broadcast Session Distribution, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST SESSION DISTRIBUTION REQUEST message to AMF. The AMF responds with a BROADCAST SESSION DISTRIBUTION RESPONSE message.

If the Shared NG-U Unicast TNL Information IE is included in the MBS Session TNL Information NG-RAN IE within the MBS Session Distribution Request Transfer IE in the BROADCAST SESSION DISTRIBUTION REQUEST message, the MB-SMF shall use the included information as the downlink termination point for the shared NG-U transport.

If the Shared NG-U Unicast TNL Information IE is not included in the MBS Session TNL Information NG-RAN IE within the MBS Session Distribution Request Transfer IE in the BROADCAST SESSION DISTRIBUTION REQUEST message, the MB-SMF shall interpret that the IP multicast is used for this shared NG-U transport, and include the Shared NG-U Multicast TNL Information IE in the MBS Session TNL Information 5GC IE in the MBS Session Distribution Response Transfer IE in the BROADCAST SESSION DISTRIBUTION RESPONSE message.

#### 8.17.X.3 Unsuccessful Operation

Figure 8.17.X.2-1: Broadcast Session Distribution, unsuccessful operation.

In case the shared NG-U transport cannot be setup successfully, the AMF shall respond with the BROADCAST SESSION DISTRIBUTION FAILURE message to the NG-RAN node with an appropriate cause value.

#### 8.17.X.4	Abnormal Conditions

Void.

-----------------Next of the Change-------------------

#### 9.2.16.X	BROADCAST SESSION DISTRIBUTION REQUEST

This message is sent by the NG-RAN node to request the setup of the NG-U transport for a broadcast MBS session.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| BC Distribution Request Transfer | M |  | OCTET STRING | Containing the BC Distribution Request Transfer IE specified in subclause 9.3.5.x | YES | reject |

#### 9.2.16.Y	BROADCAST SESSION DISTRIBUTION RESPONSE

This message is sent by the AMF to confirm the setup of the NG-U transport for the broadcast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| BC Distribution Response Transfer | M |  | OCTET STRING | Containing the BC Distribution Response Transfer IE specified in subclause 9.3.5.y | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.16.Z	BROADCAST SESSION DISTRIBUTION FAILURE

This message is sent by the AMF to indicate that the setup of the NG-U transport was unsuccessful.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| BC Distribution Failure Transfer | M |  | OCTET STRING | Containing the BC Distribution Failure Transfer IE specified in subclause 9.3.5.z | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------Next of the Change-------------------

#### 9.3.1.y	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |
| IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |

-----------------Next of the Change-------------------

### 9.3.5	MB-SMF Related IEs

#### 9.3.5.x	BC Distribution Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  |
| MBS Session TNL Information NG-RAN | O |  | 9.3.2.17 |  |

#### 9.3.5.y	BC Distribution Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  |
| MBS Session TNL Information 5GC | O |  | 9.3.2.15 |  |

#### 9.3.5.z	BC Distribution Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

-----------------Next of the Change-------------------

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

BroadcastSessionDistributionFailure,

BroadcastSessionDistributionRequest,

BroadcastSessionDistributionResponse,

BroadcastSessionModificationFailure,

BroadcastSessionModificationRequest,

BroadcastSessionModificationResponse,

BroadcastSessionReleaseRequest,

BroadcastSessionReleaseRequired,

BroadcastSessionReleaseResponse,

BroadcastSessionSetupFailure,

BroadcastSessionSetupRequest,

BroadcastSessionSetupResponse,

CellTrafficTrace,

ConnectionEstablishmentIndication,

DeactivateTrace,

DistributionReleaseRequest,

DistributionReleaseResponse,

// skip unchanged part

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

id-BroadcastSessionDistribution,

id-BroadcastSessionModification,

id-BroadcastSessionRelease,

id-BroadcastSessionReleaseRequired,

id-BroadcastSessionSetup,

id-CellTrafficTrace,

id-ConnectionEstablishmentIndication,

id-DeactivateTrace,

id-DistributionSetup,

// skip unchanged part

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionDistribution	|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning,

...

}

// skip unchanged part

aMFStatusIndication NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		AMFStatusIndication

PROCEDURE CODE			id-AMFStatusIndication

CRITICALITY				ignore

}

broadcastSessionDistribution NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionDistributionRequest

SUCCESSFUL OUTCOME		BroadcastSessionDistributionResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionDistributionFailure

PROCEDURE CODE			id-BroadcastSessionDistribution

CRITICALITY				reject

}

broadcastSessionModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionModificationRequest

SUCCESSFUL OUTCOME		BroadcastSessionModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionModificationFailure

PROCEDURE CODE			id-BroadcastSessionModification

CRITICALITY				reject

}

broadcastSessionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastSessionReleaseResponse

PROCEDURE CODE			id-BroadcastSessionRelease

CRITICALITY				reject

-----------------Next of the Change-------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

AuthenticatedIndication,

BC-DistributionFailureTransfer,

BC-DistributionRequestTransfer,

BC-DistributionResponseTransfer,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

CEmodeBrestricted,

CEmodeBSupport-Indicator,

CNAssistedRANTuning,

ConcurrentWarningMessageInd,

// skip unchanged part

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-AuthenticatedIndication,

id-BC-DistributionFailureTransfer,

id-BC-DistributionRequestTransfer,

id-BC-DistributionResponseTransfer,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-CEmodeBrestricted,

id-CEmodeBSupport-Indicator,

id-CNAssistedRANTuning,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformationForInactive,

// skip unchanged part

-- **************************************************************

--

#### -- MBS SESSION MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Broadcast Session Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION SETUP REQUEST

--

-- **************************************************************

BroadcastSessionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionSetupRequestIEs} },

...

}

BroadcastSessionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-S-NSSAI									CRITICALITY reject	TYPE S-NSSAI											PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea							CRITICALITY reject	TYPE MBS-ServiceArea									PRESENCE mandatory	}|

{ ID id-MBSSessionSetupRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModRequestTransfer)		PRESENCE 		mandatory	}|

{ ID id-AssociatedSessionID					CRITICALITY ignore	TYPE AssociatedSessionID							PRESENCE optional		},

...

}

// skip unchanged part

-- **************************************************************

--

##### -- Broadcast Session Modification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION REQUEST

--

-- **************************************************************

BroadcastSessionModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationRequestIEs} },

...

}

BroadcastSessionModificationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea									CRITICALITY reject	TYPE MBS-ServiceArea									PRESENCE optional		}|

{ ID id-MBSSessionModificationRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModRequestTransfer)		PRESENCE optional		}

...

}

// skip unchanged part

-- **************************************************************

--

##### -- BROADCAST SESSION RELEASE RESPONSE

--

-- **************************************************************

BroadcastSessionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseResponseIEs} },

...

}

BroadcastSessionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-MBSSessionReleaseResponseTransfer		CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBSSessionReleaseResponseTransfer)		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Session Distribution Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION DISTRIBUTION REQUEST

--

-- **************************************************************

BroadcastSessionDistributiontRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionDistributionRequestIEs} },

...

}

BroadcastSessionDistributionRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-BC-DistributionRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING BC-DistributionRequestTransfer)		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION DISTRIBUTION RESPONSE

--

-- **************************************************************

BroadcastSessionDistributionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionDistributiontResponseIEs} },

...

}

BroadcastSessionDistributionResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-BC-DistributionResponseTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING BC-DistributionResponseTransfer)	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION DISTRIBUTION  FAILURE

--

-- **************************************************************

BroadcastSessionDistributionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionDistributionFailureIEs} },

...

}

BroadcastSessionDistributionFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID						PRESENCE mandatory	}|

{ ID id-BC-DistributionFailureTransfer	CRITICALITY ignore	TYPE OCTET STRING (CONTAINING BC-DistributionFailureTransfer)	PRESENCE 	mandatory	}|

{ ID id-Cause												CRITICALITY ignore	TYPE Cause									PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Distribution Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

DistributionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionSetupRequestIEs} },

...

}

DistributionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupRequestTransfer			CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupRequestTransfer)			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DISTRIBUTION SETUP RESPONSE

--

-- **************************************************************

DistributionSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionSetupResponseIEs} },

...

}

DistributionSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupResponseTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupResponseTransfer)			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-----------------Next of the Change(in case of per MBS session)-------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

// skip unchanged part

AssociatedQosFlowList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF AssociatedQosFlowItem

AssociatedQosFlowItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowMappingIndication		ENUMERATED {ul, dl, ...}							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssociatedQosFlowItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedQosFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex	PRESENCE optional	},

...

}

AssociatedSessionID ::= SEQUENCE {

iP-MulticastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

iE-Extensions				ProtocolExtensionContainer { {AssociatedSessionID-ExtIEs} } 	OPTIONAL,

...

}

AssociatedSessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AuthenticatedIndication ::= ENUMERATED {true, ...}

// skip unchanged part

AvailableRANVisibleQoEMetrics-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- B

BC-DistributionFailureTransfer ::= SEQUENCE {

mBS-SessionID 				MBS-SessionID,

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BC-DistributionFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

BC-DistributionFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BC-DistributionRequestTransfer ::= SEQUENCE {

mBS-SessionID 				MBS-SessionID,

mBS-SessionTNLInfoNGRAN 	MBS-SessionTNLInfoNGRAN																	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {BC-DistributionRequestTransfer-ExtIEs} }		OPTIONAL,

...

}

BC-DistributionRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BC-DistributionResponseTransfer ::= SEQUENCE {

mBS-SessionID 			MBS-SessionID,

mBS-SessionTNLInfo5GC 	MBS-SessionTNLInfo5GC																	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {BC-DistributionResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

BC-DistributionResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BeamMeasurementsReportConfiguration ::= SEQUENCE {

beamMeasurementsReportQuantity			BeamMeasurementsReportQuantity 	OPTIONAL,

maxNrofRS-IndexesToReport               MaxNrofRS-IndexesToReport		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { BeamMeasurementsReportConfiguration-ExtIEs} } OPTIONAL,

...

}

BeamMeasurementsReportConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BeamMeasurementsReportQuantity ::= SEQUENCE {

rSRP						ENUMERATED {true, ...},

rSRQ						ENUMERATED {true, ...},

sINR						ENUMERATED {true, ...},

iE-Extensions			ProtocolExtensionContainer { { BeamMeasurementsReportQuantity-ExtIEs} } OPTIONAL,

...

}

BeamMeasurementsReportQuantity-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-----------------Next of the Change-------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

// skip unchanged part

id-ConnectionEstablishmentIndication		ProcedureCode ::= 65

id-BroadcastSessionModification				ProcedureCode ::= 66

id-BroadcastSessionRelease					ProcedureCode ::= 67

id-BroadcastSessionSetup					ProcedureCode ::= 68

id-DistributionSetup 						ProcedureCode ::= 69

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-BroadcastSessionDistribution				ProcedureCode ::= FFS

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

// skip unchanged part

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-AssociatedSessionID									ProtocolIE-ID ::= 998 -- to be allocated

id-SharedNGUNotEstablished								ProtocolIE-ID ::= 999 -- to be allocated

id-MBS-AssistanceInformation							ProtocolIE-ID ::= 1000 -- to be allocated

id-BC-DistributionFailureTransfer						ProtocolIE-ID ::= FFS

id-BC-DistributionRequestTransfer						ProtocolIE-ID ::= FFS

id-BC-DistributionResponseTransfer						ProtocolIE-ID ::= FFS

END

-----------------End of the Change-------------------

# 7. TP to TS 38.473

-----------------Start of the Changes-------------------

### 8.14.1	Broadcast Context Setup

#### 8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

If the Associated Session ID IE is contained in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.401 [4].

If the MBS RAN Sharing Information IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take this information into account to determine MBS session resource sharing efficient scheme for broadcast delivery, as specified in TS 38.401 [4].

#### 8.14.1.3	Unsuccessful Operation

Figure 8.14.1.3-1: Broadcast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the requested MBS session context it shall consider the procedure as failed and reply with the BROADCAST CONTEXT SETUP FAILURE message.

#### 8.14.1.4	Abnormal Conditions

Not applicable.

-----------------Start of the Changes-------------------

### 8.14.4	Broadcast Context Modification

#### 8.14.4.1	General

The purpose of the Broadcast Context Modification procedure is to modify an established MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.4.2	Successful Operation

Figure 8.14.4.2-1: Broadcast Context Modification procedure. Successful operation

The BROADCAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the BROADCAST CONTEXT MODIFICATION RESPONSE message.

If the Broadcast MRB To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicate the reason for the failure with an appropriate cause value for each MRB failed to be setup.

If the Broadcast MRB To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-DU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Broadcast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Broadcast MRB Failed To Be Modified List IE;

If the Broadcast MRB Failed To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be modified with an appropriate cause value for each Broadcast MRB failed to modify.

If the MBS Service Area IE is included in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall

-	release MBS Session Resources within cells not contained in the MBS Service Area IE, if any;

-	establish MBS Session Resources within cells which have not been contained in MBS Service Area information previously received;

-	replace MBS Service Area information previously received with information received in the MBS Service Area IE included in the BROADCAST CONTEXT MODIFICATION REQUEST message;

-	include the Broadcast Area Scope IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message to indicate the cells where MBS Session resources are currently established in the gNB-DU.

If the the MBS Service Area IE was not included in the BROADCAST CONTEXT MODIFICATION REQUEST message and the gNB-DU has released MBS Session Resources within at least one cell or has established MBS Session Resources within at least one cell the gNB-DU shall include the Broadcast Area Scope IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message to indicate the cells where MBS Session resources are currently established in the gNB-DU.

If the MBS RAN Sharing Information IE is included in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that the request provides the update for the list of MBS Sessions providing identical content.

#### 8.14.4.3	Unsuccessful Operation

Figure 8.14.4.3-1: Broadcast Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the broadcast context can be successfully performed, the gNB-DU shall respond with the BROADCAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

-----------------Next of the Change-------------------

### 9.2.13	Broadcast Context Management messages

#### 9.2.13.1	BROADCAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a broadcast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Associated Session ID | O |  | 9.3.1.xxx |  | YES | ignore |
| MBS Session ID List | O |  | 9.3.1.yyy | Provides the full list of MBS Session ID providing identical content. If included, the MBS Session ID IE shall be ignored. | YES | reject |

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Associated Session ID | O |  | 9.3.1.x |  | YES | ignore |
| MBS RAN Sharing Information | O |  | 9.3.1.yyy | Provides the full list of MBS Session ID providing identical content. If included, the MBS Session ID IE and the MBS Service Area IE shall be ignored. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

-----------------Next of the Change-------------------

#### 9.2.13.6	BROADCAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify broadcast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 | Overwrites any previously received MBS Service Area information | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Broadcast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | Updated gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Broadcast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| MBS RAN Sharing Information | O |  | 9.3.1.yyy | Provides the full list of MBS Session ID providing identical content. If included, the MBS Session ID IE and the MBS Service Area IE shall be ignored. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

-----------------Next of the Change-------------------

#### 9.3.1.225	MBS CU to DU RRC Information

This IE indicates the MBS CU to DU RRC Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Broadcast Cell List | M |  |  |  | - |  |
| >MBS Broadcast Cell Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>mtch-neighbourCell | O |  | OCTET STRING | Includes the mtch-neighbourCell contained in the MBS-SessionInfoList IE, as defined in TS 38.331[8] If mtch-neighbourCell List IE is included, this IE shall be ignored. | - |  |
| >>mtch-neighbourCell List |  | 0..1 |  |  | YES | ignore |
| >>>mtch-neighbourCell Item |  | 1..<maxnoofMBSSessionIDinMOCN> |  |  | - |  |
| >>>>MBS Session ID | M |  | 9.3.1.218 |  | - |  |
| >>>>mtch-neighbourCell | O |  | OCTET STRING | Includes the mtch-neighbourCell contained in the MBS-SessionInfoList IE, as defined in TS 38.331[8]. | - |  |
| MBS Broadcast MRB List |  | 1 |  |  | - |  |
| >MBS Broadcast MRB Item |  | 1 .. <maxnoofMRBs> |  |  | - |  |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB PDCP Config Broadcast | M |  | OCTET STRING | Includes the MRB-PDCP-ConfigBroadcast IE, as defined in TS 38.331[8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessionIDinMOCN | Maximum number of MBS Sessions providing identical content in MOCN. Value is FFS. |

-----------------Next of the Change-------------------

#### 9.3.1.x Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Multicast Address | M |  | Transport Layer Address 9.3.2.3 |  |
| IP Source Address | M |  | Transport Layer Address 9.3.2.3 |  |

#### 9.3.1.yyy	MBS RAN Sharing Information

This IE provides the list of MBS Session IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS RAN Sharing Information Item IEs |  | 1..<maxnoofMBSSessionIDinMOCN> |  |  |
| >MBS Session ID | M |  | 9.3.1.218 |  |
| >MBS Service Area | O |  | 9.3.1.222 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessionIDinMOCN | Maximum number of MBS Sessions providing identical content in MOCN. Value is FFS. |

-----------------Next of the Change-------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

// skip unchanged part

MBS-Area-Session-ID,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBSRANSharingInformation,

MBS-Session-ID,

MBS-ServiceArea,

MulticastF1UContextReferenceCU,

// skip unchanged part

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

// skip unchanged part

id-MBS-Area-Session-ID,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MBSRANSharingInformation,

id-MC-PagingCell-Item,

// skip unchanged part

-- **************************************************************

--

#### -- BROADCAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP REQUEST

--

-- **************************************************************

BroadcastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupRequestIEs} },

...

}

BroadcastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE	GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID					PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea					CRITICALITY reject 	TYPE	MBS-ServiceArea					PRESENCE optional   }|

{ ID id-MBS-CUtoDURRCInformation		CRITICALITY reject	TYPE	MBS-CUtoDURRCInformation		PRESENCE mandatory	}|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI							PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	BroadcastMRBs-ToBeSetup-List	PRESENCE mandatory	}|

{ ID id-AssociatedSessionID				CRITICALITY ignore	TYPE	AssociatedSessionID				PRESENCE optional	}|

{ ID id-MBSRANSharingInformation		CRITICALITY reject	TYPE	MBSRANSharingInformation		PRESENCE optional	},

...

}

BroadcastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetup-ItemIEs} }

BroadcastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	BroadcastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

// skip unchanged part

-- **************************************************************

--

#### -- BROADCAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

BroadcastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationRequestIEs} },

...

}

BroadcastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional		}|

{ ID id-MBS-CUtoDURRCInformation			CRITICALITY reject	TYPE MBS-CUtoDURRCInformation				PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeSetupMod-List	PRESENCE optional	}|

{ ID id-BroadcastMRBs-ToBeModified-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeModified-List	PRESENCE optional	}|

{ ID id-BroadcastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeReleased-List	PRESENCE optional	}|

{ ID id-MBSRANSharingInformation			CRITICALITY reject	TYPE MBSRANSharingInformation			PRESENCE optional	},

...

}

BroadcastMRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetupMod-ItemIEs} }

BroadcastMRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeModified-ItemIEs} }

BroadcastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeReleased-ItemIEs} }

BroadcastMRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeModified-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

// skip unchanged part

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-MTCH-NeighbourCell-List,

// skip unchanged part

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoofSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (1..3)

Associated-SCell-Item ::= SEQUENCE {

sCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Associated-SCell-ItemExtIEs } }	OPTIONAL

}

Associated-SCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= SEQUENCE {

iP-MulticastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

iE-Extensions				ProtocolExtensionContainer { {AssociatedSessionID-ExtIEs} } 	OPTIONAL,

...

}

AssociatedSessionID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailablePLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF AvailablePLMNList-Item

AvailablePLMNList-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { AvailablePLMNList-Item-ExtIEs} } OPTIONAL

}

AvailablePLMNList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

// skip unchanged part

MBSF1UInformation ::= SEQUENCE {

mbs-f1u-info				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformation-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformation-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSInterestIndication ::= OCTET STRING

MBSRANSharingInformation ::= SEQUENCE (SIZE(1.. maxnoofMBSSessionIDinMOCN)) OF MBS-Session-ID-Item

MBSRANSharingInformation-Item ::= SEQUENCE {

mBS-Session-ID			MBS-Session-ID,

mBS-ServiceArea			MBS-ServiceArea		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBSRANSharingInformation-Item-ExtIEs} } 	OPTIONAL,

...

}

MBSRANSharingInformation-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Session-ID ::= SEQUENCE {

tMGI						TMGI,

nID							NID					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Session-ID-ExtIEs} } OPTIONAL,

...

}

MBS-Session-ID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBS-CUtoDURRCInformation		::= SEQUENCE {

mBS-Broadcast-Cell-List		MBS-Broadcast-Cell-List,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-CUtoDURRCInformation-ExtIEs } } OPTIONAL,

...

}

MBS-CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Broadcast-Cell-Item

MBS-Broadcast-Cell-Item ::= SEQUENCE {

nRCGI						NRCGI,

mtch-neighbourCell			OCTET STRING		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MTCH-NeighbourCell-List		CRITICALITY ignore	EXTENSION MTCH-NeighbourCell-List		PRESENCE optional	},

...

}

MBS-Broadcast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Broadcast-MRB-Item

MBS-Broadcast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MTCH-NeighbourCell-List	::= SEQUENCE (SIZE(1.. maxnoofMBSSessionIDinMOCN))	OF  MTCH-NeighbourCell-Item

MTCH-NeighbourCell-Item ::= SEQUENCE {

mBS-Session-ID				MBS-Session-ID,

mtch-neighbourCell			OCTET STRING		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MTCH-NeighbourCell-Item-ExtIEs} } 	OPTIONAL,

...

}

MTCH-NeighbourCell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-----------------Next of the Change-------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

// skip unchanged part

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-AssociatedSessionID								ProtocolIE-ID ::= XXX

id-MBSRANSharingInformation							ProtocolIE-ID ::= FFS

id-MTCH-NeighbourCell-List							ProtocolIE-ID ::= FFS

END

-- ASN1STOP

-----------------End of the Change------------------