3GPP TSG-RAN WG3 Meeting #121	R3-234092

Toulouse, France, 21st – 25th Aug, 2023

Title: 	Discussion on misalignment of TAI between ASN.1 and tabular in QoE

Source: 	Huawei, ZTE, InterDigital, China Unicom, Ericsson

Agenda item:	9.2

Document Type:	Discussion

1. Introduction

In Rel-17, area scope of QMC has been introduced, and it was decided that the area scope can be TAI based. However, in XnAP, the TAI refers to a wrong IE, and the correct TAI IE has not been introduced in XnAP.

2. Discussion

According to TS 38.423 below, the area scope of QMC can be cell based, TA based, TAI based or PLMN based, and this was achieved in RAN3 #112-e meeting, which was the very first meeting to discuss R17 QoE [1].

9.2.3.157 UE Application Layer Measurement Configuration Information This IE defines the information about the QoE Measurement Collection (QMC) configuration. IE/Group Name; Presence; Range; IE type and reference; Semantics description. QoE Reference; M; OCTET STRING (SIZE(6)); QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String.. <omit texts>. CHOICE Area Scope of QMC; O. >Cell based. >>Cell ID List for QMC; 1 .. <maxnoofCellIDforQMC>. >>>Global NG-RAN Cell Identity; M; 9.2.2.27; The included NG-RAN Cell Identity IE can only indicate the NR Cell Identity.. >TA based. >>TA List for QMC; 1 .. <maxnoofTAforQMC>. >>>TAC; M; 9.2.2.5; The TAI is derived using the current serving PLMN.. >TAI based. >>TAI List for QMC; 1 .. <maxnoofTAforQMC>. >>>TAI; M; 9.2.3.20. >PLMN based. >>PLMN List for QMC; 1 .. <maxnoofPLMNforQMC>. >>>PLMN Identity; M; 9.2.2.4.

It can be observed that TAI refers to 9.2.3.20 which is TAI support List, and includes the following sub-IEs.

#### 9.2.3.20 TAI Support List This IE indicates the list of TAIs supported by NG-RAN node and associated characteristics e.g. supported slices. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. TAI Support Item; 1..<maxnoofsupportedTACs>; –. >TAC; M; 9.2.2.5; Broadcast TAC; –. >Broadcast PLMNs; 1..<maxnoofsupportedPLMNs>; –. >>PLMN Identity; M; 9.2.2.4; Broadcast PLMN; –. >>TAI Slice Support List; M; Slice Support List 9.2.3.22; Supported S-NSSAIs per TAC, per PLMN or per SNPN.; –. >>NPN Support; O; 9.2.2.72; YES; reject. >>Extended TAI Slice Support List; O; Extended Slice Support List 9.2.3.139; Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN.; YES; reject. >>TAI NSAG Support List; O; 9.2.3.170; NSAG information associated with the slices per TAC, per PLMN or per SNPN.; YES; ignore. Range bound; Explanation. maxnoofsupportedTACs; Maximum no. of TACs supported by an NG-RAN node. Value is 256.. maxnoofsupportedPLMNs; Maximum no. of PLMNs supported by an NG-RAN node. Value is 12..

However, this does not match the provided ASN.1. Within which, only TAC and PLMN constitutes TAI.

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI-Item TAI-Item ::= SEQUENCE { tAC TAC, pLMN-Identity PLMN-Identity, iE-Extensions ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL, ... } TAI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= { ... }

- Observation 1: There is a misalignment of TAI between tabular and ASN.1 in XnAP.

With the misalignment found, the first step here is checking which part is incorrect. In NGAP, there is clear definition of TAI, which is TAC+PLMN. In addition, the same TAI within ‘TAI list for QMC’ in NGAP, refers to TAI instead of TAI support list.

In this specific case, TAI is used by RAN to check whether a certain QoE measurement is performed within the given area defined by TAIs. Therefore, the tabular is incorrect.

- Observation 2: TAI in TAI list for QMC should refer to TAC+PLMN instead of TAI support list

- For avoiding NBC changes, we can keep the ASN.1 as it is, and only fix the tabular part. Only change the referred IE is editorial and easy. However, we note that there is no such definition of TAI in XnAP, and we can introduce a new TAI IE in 9.2.2 correspondingly.

#### 9.2.2.X TAI This IE is used to uniquely identify a Tracking Area. IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.3.3.5. TAC; M; 9.3.3.10.

- Or alternatively, an option 2 is we can follow what we did in MDT, simply change the tabular from TAI to PLMN Identity+TAC directly.

Add a new TAI IE in XnAP, and let the TAI within TAI list for QMC refer to this IE

Alternatively, change the tabular from TAI to PLMN Identity+TAC directly.

A stage 3 CR adopt option 2 can be found in [2]

4. Conclusion

Based on the discussion in this paper, we propose the following:

- Observation 1: There is a misalignment of TAI between tabular and ASN.1 in XnAP.

- Observation 2: TAI in TAI list for QMC should refer to TAC+PLMN instead of TAI support list

- Add a new TAI IE in XnAP, and let the TAI within TAI list for QMC refer to this IE

Alternatively, change the tabular from TAI to PLMN Identity+TAC directly.

5. Reference

- RAN3 #112-e chairman notes

- R3-234093 Correction to misalignment of TAI between ASN.1 and tabular in QoE, Huawei et al.