# 3GPP TSG-RAN WG3 Meeting #121	R3-234110

# 21-25 August 2023, Toulouse, France

Agenda Item:	10.2.2

Source:	CATT

Title:	(TP for SON BL CR 38.413) Discussion on MRO enhancement for fast MCG recovery and voice fallback

Document for:	Other

Introduction

In last RAN3 meeting, we did general discussion and achieved some agreements [1]. In the document, we provide some further analysis on the below topic for MRO enhancements

MRO for the fast MCG recovery:

Agree to define Case c in R18. Solution can be further discussed.

Whether the network based solution or UE based solution should be used for Case c?

Whether the MRO for fast MCG recovery needs to support pre-R18 UEs?

MRO for the voice fall-back:

RAN3 assumes the indication is needed and draft the LS to RAN2 to check the feasibility of the solution for MRO for voice fallback.

RAN2 has decided to introduce the indication of voice fallback in the LTE RLF report.

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

Discussion

2.1 MRO for the fast MCG recovery

In the last meeting, RAN3 agreed to support Case c to assist network to adjust T316 value e.g., increase or decrease T316 to optimize fast MCG recovery. For example, increasing T316 can give more time for MCG failure information/RRC message transmission to avoid fast MCG recovery near failure.

The procedure of fast MCG failure recovery success but near failure follows the below steps:

- MCG failure, if UE is configured with fast MCG failure recovery, it will send MCG failure information to MCG via SCG.

- If the fast MCG failure recovery successes or near failure, UE will delete RLF report based on R16 principle.

Observation 1: In R16, UE will delete RLF report after successful fast MCG failure recovery

There are four options to support Case c were discussed in the summary [2].

UE based solution:

- Option 1: Introduce a new SHR-wise report.

- Option 2: Reuse the RLF report.

- Option 3: Reuse the SHR report.

- Option 4: Use UE Assistance Information.

Network based solution:

- SN reports to MN the elapsed time between receiving the MCG Failure Information and receiving confirmation from the UE of the reception of the reconfiguration from the MN.

Network based solution cannot record the Uu delay for UL MCG Failure Information transmission between UE and SN. Also, whether SN will send MCG failure information to MN or send RRC message to UE immediately is based on SN implementation. Therefore, UE based solution will be more accurate compared with network based solution.

Observation 2: UE based solution is preferred for Case c.

For UE based solution, UE should report elapsed T316 time between the transmission of MCG Failure Information and receiving RRC reconfiguration or RRC release message and/or cause value e.g., T316. It depends on which report is used.

Option 1: MN configures a trigger for T316 and UE only report SHR-wise with cause value or elapsed T316 time when meet the triggers.

Option 2: RLF report will be deleted after successful fast MCG failure recovery since R16.The UE behaviours should be updated.

Option 3: MN can configure a trigger for T316 and UE only report SHR with cause value when meet the triggers. It is similar as T310/T312/T304 optimization.

Option 4: UE assistance information is not used in SON optimization normally. It relates to UE detection/preference and it should be provided after RRC reconfiguration procedure.

Based on the above analyse, SHR (wise) and RLF report are both able to be enhanced to support near failure case. It is note that RAN2 is discussing this issue after the RAN2#122 meeting. We can leave it to RAN2 and no LS is needed to trigger RAN2 discussion.

Proposal 1: SHR (wise) or RLF report can be used to support Case c. RAN2 decides which report should be used. No LS is needed.

Case a: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running).

Case b: the signalling delay is longer than the time the UE waits for the response (T316 expired).

Case f1: the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation.

In R18, we consider keeping the RLF report and report it to network in fast MCG recovery failure scenario for optimization. There is some information for RLF report enhancement has been discussed but not achieved agreement in the last meeting.

- T316 elapsed time

- Time between MCG failure and SCG failure

- SCG status e.g. PSCell change/PSCell addition

a) We are not sure the benefit of reporting T316 elapsed time in RLF report to optimization T316 setting. For Case f1, T316 will not run. In Case of T316 running i.e., Case b, the network can identify too short T316 by the cause of T316 expiry. However, network cannot identify whether the T316 setting is too long. For Case a, the T316 elapsed time is not an essential point for fast MCG recovery failure. The problem is SCG failure not the T316. Therefore, we do need to optimize T316 value for failure case.

Proposal 2: No need to introduce T316 elapsed time in RLF report for T316 optimization.

b) If the time between MCG failure and SCG failure are close, the MCG recovery and SCG recovery may impact each other. For example, fast MCG recovery failed due to SCG failure before MCG failure (Case f1). Network should optimize such time to make sure that both failures are independent enough. It can increase success rate of fast MCG failure recovery.

Proposal 3: Enhance RLF report to recode Time between MCG failure and SCG failure.

c) We agreed that the SN status includes SCG failure, SCG was deactivated or other Cases that SCG is not available and include them in the LS to RAN2. However, it should be clarified that the “other Cases” includes the Case of PScell change/PScell addition.

If the PScell change/addition is performed during fast MCG recovery, the RRC message or MCG failure information cannot be sent to UE and leading to T316 expire. Therefore, the PScell change/addition is performed during fast MCG recovery can be covered by Case b. However, if the PScell change/addition is ongoing before MCG failure, the UE will perform RRC re-establishment instead of fast MCG failure recovery when MCG fails. In this Case, the fast MCG recovery cannot be performed because SCG is not available due to PScell change/PScell ongoing and lead to fast MCG recovery failure.

### 5.3.7	RRC connection re-establishment

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

Case f1 can be revised as “the SCG fails or is deactivated yet or PScell change/addition ongoing before the UE sends the MCGFailureInformation.

Proposal 4: Revise Case f1: the SCG fails or is deactivated yet or PScell change/addition ongoing before the UE sends the MCGFailureInformation.

WA: In order to enable support for pre-Rel.18 UEs (and help avoid wasting Uu resources), the T316 will be delivered from the MN to the SN.

Whether the MRO for fast MCG recovery needs to support pre-R18 UEs?

MN can identify three Cases (Case a/b/f1) by T316 elapsed time.

Case a: T316 running but not expire; Case b: T316 expire; Case f1: T316 does not start.

To make the pre-R18 UEs support the Rel-18 fast MCG recovery scenarios, SN can calculate T316 between it receives MCG failure information and receiving RRC message from MN though the Uu delay will not be reflect.

If the T316 elapsed sent from SN to MN is 0, MN will classify it to Case c. If the T316 elapsed sent from SN is bigger than 0 but smaller than T316, MN will classify it to Case a. If the T316 elapsed sent from SN is bigger than T316, MN will classify it to Case b. it assist MN to identify different fast MCG recovery scenarios in Rel-18.

Observation 3: MN can distinguish case a/b/f1 according to the T316 running states.

Proposal 5: T316 can be delivered from MN to SN for pre-Rel.18 UEs to support Rel-18 fast MCG recovery scenarios.

2.2 MRO for inter-system handover for voice fallback

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

In the last meeting, we achieve a WA that a new handover report type is needed in Inter-system HO Report over S1 and NG to support MRO for inter-system handover for voice fallback. There is no doubt that the E-UTRAN node can optimize coverage hole while the mainly concern is whether NR node should be optimized. From our understanding, the purpose of this topic is to differentiate handover for voice fallback from normal handover. In this case, NR node will not optimize the coverage of NR node to avoid UE handover to LTE and keep it resist in NR because the handover from NR cell to E-UTRAN cell was due to voice fallback rather than bad signal quality in NR cells. Furthermore, the target E-UTRAN cell selection also can be optimized in NR node. After the optimization, NR node will select a suitable target E-UTRAN cell to avoid failed/re-established inter-system handover for voice fallback.

Observation 4: NR node shall differentiate handover for voice fallback from normal handover, and it is benefit for NR node to optimize target E-UTRA cell selection.

For RLF in case 1, the failed/target LTE node should inform handover report type i.e., Inter-system Mobility Failure during Voice Fallback to NR node to assist NR node knows that a failure happens during inter-system voice fallback rather than normal RLF shortly after handover. The optimization in NR node should base on failed/target LTE node input e.g., re-establishment E-UTRAN cell because NR node cannot decode LTE RLF report.

Observation 5: For RLF in case 1, failed/target LTE node decodes the LTE RLF report and informing handover report type to NR node to assist NR node to differentiate voice fallback handover from legacy handover, and also sending the re-establishment E-UTRA cell to NR node for the optimization of target E-UTRA cell selection.

For HOF in case 1, UE report NR RLF report to NR node, NR node sends NR RLF report to NR node via failure indication.

For HOF in case 2, re-establishment NR node sends NR RLF report to NR node via failure indication.

Observation 6: For HOF in case 1 and case 2, there is no stage-3 impact.

Proposal 6: Turn the “WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.” into agreement. And introduce re-establishment E-UTRA cell in HO report. TP for 38.413 in the Annex can be used to support RLF in Case 1.

Conclusions

Observation 1: In R16, UE will delete RLF report after successful fast MCG failure recovery

Observation 2: UE based solution is preferred for Case c.

Proposal 1: SHR (wise) or RLF report can be used to support Case c. RAN2 decides which report should be used. No LS is needed.

Proposal 2: No need to introduce T316 elapsed time in RLF report for T316 optimization.Proposal 3: Enhance RLF report to recode Time between MCG failure and SCG failure.

Proposal 3: Enhance RLF report to recode Time between MCG failure and SCG failure.

Proposal 4: Revise Case f1: the SCG fails or is deactivated yet or PScell change/addition ongoing before the UE sends the MCGFailureInformation.

Observation 3: MN can distinguish case a/b/f1 according to the T316 running states.

Proposal 5: T316 can be delivered from MN to SN for pre-Rel.18 UEs to support Rel-18 fast MCG recovery scenarios.

Observation 4: NR node shall differentiate handover for voice fallback from normal handover, and it is benefit for NR node to optimize target E-UTRA cell selection.

Observation 5: For RLF in case 1, failed/target LTE node decodes the LTE RLF report and informing handover report type to NR node to assist NR node to differentiate voice fallback handover from legacy handover, and also sending the re-establishment E-UTRA cell to NR node for the optimization of target E-UTRA cell selection.

Observation 6: For HOF in case 1 and case 2, there is no stage-3 impact.

Proposal 6: Turn the “WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.” into agreement. Also, introduce re-establishment E-UTRA cell in HO report. TP for 38.413 and 36.413 in the Annex can be used to support RLF in Case 1.

# 4. Reference

[1] Chair notes, RAN3#120

[2] R3-233336, Summary_of_offline_disc_CB # SONMDT2_MRO, ZTE

# 5. Annex: TP for TS38.413

9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |
| >Too early Inter-system HO |  |  |  |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |
| >Inter-system Unnecessary HO |  |  |  |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |
| >>Candidate Cell List |  | 1 |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |
| >>>>>Candidate PCI |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNARFCN) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |
| > Inter-system Mobility Failure during Voice Fallback |  |  |  |  |
| >>Source Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for the Inter-system Mobility Failure during Voice Fallback. |
| >>Failure Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the failure cell for the Inter-system Mobility Failure during Voice Fallback. |
| >>Re-establishment Cell ID | O |  | E-UTRA CGI 9.3.1.9 | CGI of the re-establishment cell for the Inter-system Mobility Failure during Voice Fallback. |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 [18] |

9.4.5	Information Element Definitions

<<<unchanged part is omitted>>>

InterSystemHOReport ::= SEQUENCE {

handoverReportType		InterSystemHandoverReportType,

iE-Extensions			ProtocolExtensionContainer { { InterSystemHOReport-ExtIEs} }			OPTIONAL,

...

}

InterSystemHOReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InterSystemHandoverReportType ::= CHOICE {

tooearlyIntersystemHO				TooearlyIntersystemHO,

intersystemUnnecessaryHO			IntersystemUnnecessaryHO,

intersystemMobilityFailureduringVoiceFallback    IntersystemMobilityFailureduringVoiceFallback

choice-Extensions		ProtocolIE-SingleContainer { { InterSystemHandoverReportType-ExtIEs} }

}

InterSystemHandoverReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

IntersystemUnnecessaryHO ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

earlyIRATHO				ENUMERATED {true, false, ...},

candidateCellList		CandidateCellList,

iE-Extensions			ProtocolExtensionContainer { { IntersystemUnnecessaryHO-ExtIEs} }			OPTIONAL,

...

}

IntersystemUnnecessaryHO-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemMobilityFailureduringVoiceFallback ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

reestablishmentCellID	EUTRA-CGI                       OPTIONAL,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureduringVoiceFallback-ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureduringVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...