3GPP TSG-RAN WG3 Meeting #121	R3-234148

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	9.2

Source:	Huawei, Nokia, Nokia Shanghai Bell, CATT, Lenovo, Samsung

Title:	Discussion on the BH info configuration for DRBs support CA based duplication

Document for:	Discussion

1	Introduction

In this paper, we discuss the IAB related issue in current F1AP specification, regarding to the IAB BH information configuration of a F1-U tunnel in case the UE DRB supports more than two RLCs in CA based duplication. The details about potential solutions are provided in the discussion part.

2	Discussion

2.1	Problem in the current specification

In TS 38.473, the Additional PDCP Duplication TNL list was introduced for the UE DRB which may support more than two RLC channels. If such UE is served by IAB-node, the BH information is associated with each F1-U UL TEID which is identified by the Additional PDCP Duplication UP TNL Information IE, when the IAB donor CU provide configuration to the IAB-DU. Therefore, the BH information IE is added as optional child IE to the Additional PDCP Duplication TNL Items IE in the following F1AP messages: 9.2.2.1 UE context SETUP REQUEST, 9.2.2.7 UE CONTEXT MODIFICATION REQUEST, and 9.2.2.11 UE CONTEXT MODIFICATION CONFIRM.

AdditionalPDCPDuplicationTNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF AdditionalPDCPDuplicationTNL-Item

AdditionalPDCPDuplicationTNL-Item ::=SEQUENCE {

additionalPDCPDuplicationUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { AdditionalPDCPDuplicationTNL-ItemExtIEs } }	OPTIONAL,

...

}

AdditionalPDCPDuplicationTNL-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BHInfo		CRITICALITY ignore	EXTENSION BHInfo		PRESENCE optional	},

...

}

Accordingly, as shown in the previous ASN.1 coding copied from the F1AP specification, the BHInfo is added as an extension IE to the AdditionalPDCPDuplicationTNL-Item.

However, the AdditionalPDCPDuplicationTNL-Item is also used by the gNB-DU to indicate the DL TEID of the additional F1-U tunnels. In other words, the current ASN.1 design allows the IAB-DU to include the BH information when providing the DL TEID of the F1-U tunnel (using the Additional PDCP Duplication TNL List) to the IAB-donor-CU via 9.2.2.2 UE CONTEXT SETUP RESPONSE, 9.2.2.8 UE CONTEXT MODIFICATION RESPONSE and 9.2.2.10 UE CONTEXT MODIFICATION REQUIRED.

There are two consequent problems:

- The BH information can only be configured from the IAB-donor-CU to the IAB-DU and will be used by the IAB-DU for UL traffic transmission, it should not be carried in the reverse direction F1AP messages. If the per F1-U tunnel BH information is included in reverse direction, i.e. in the F1AP message from IAB-DU to IAB-donor-CU, such BH information may not be interpreted correctly, or may cause some other problems. For example, a bad implementation at the IAB-donor-CU may consider such BH information is suggested by the IAB-DU side, and use this value for next time configuration. But the “suggested” value of BH information may be meaningless, and such implementation will cause further problem (failure of BAP routing and BH RLC CH mapping, etc.) for UL traffic transmission.

- The ASN.1 part and tabular are misaligned for the following messages: 9.2.2.2 UE CONTEXT SETUP RESPONSE, 9.2.2.8 UE CONTEXT MODIFICATION RESPONSE, and 9.2.2.10 UE CONTEXT MODIFICATION REQUIRED.

To solve the above problems, there may be three options, which are introduced in the following sections 2.2-section 2.4.

2.2	Option 1: Change ASN.1 and tabular

Option 1: Adding a new IE to indicate the additional DL TNL information and not include the BH information in following messages: 9.2.2.2 UE CONTEXT SETUP RESPONSE, 9.2.2.8 UE CONTEXT MODIFICATION RESPONSE, and 9.2.2.10 UE CONTEXT MODIFICATION REQUIRED. If the new IE presents, the old one should be ignored. The change on tabular below can be taken as example.

| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  | This IE is ignored if the Additional PDCP Duplication TNL DL List IE is present. | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>Additional PDCP Duplication TNL DL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL DL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL DL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| SRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |

Such option requires change in both the tabular (9.2.2.2, 9.2.2.8, 9.2.2.10) and the ASN.1 part, the change is backward compatible.

2.3	Option 2: Change ASN.1

Option 2: Change the ASN.1 part, by defining a new IE to reflect the gNB-DU side endpoint of the F1-U tunnels indicated by Additional PDCP Duplication TNL List IE in the 9.2.2.2, 9.2.2.8, and 9.2.2.10. The new defined IE should not include the BHInfo, but only the UPTransportLayerInformation. An example is given below.

#### -- A

>>>>>>>>>>unrelated parts are skipped<<<<<<<<<<

AdditionalPDCPDuplicationTNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF AdditionalPDCPDuplicationTNL-Item

AdditionalPDCPDuplicationTNL-Item ::=SEQUENCE {

additionalPDCPDuplicationUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { AdditionalPDCPDuplicationTNL-ItemExtIEs } }	OPTIONAL,

...

}

AdditionalPDCPDuplicationTNL-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BHInfo		CRITICALITY ignore	EXTENSION BHInfo		PRESENCE optional	},

...

}

AdditionalPDCPDuplicationTNL-DL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF AdditionalPDCPDuplicationTNL-DL-Item

AdditionalPDCPDuplicationTNL-DL-Item ::=SEQUENCE {

additionalPDCPDuplicationUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { AdditionalPDCPDuplicationTNL-DL-ItemExtIEs } }	OPTIONAL,

...

}

AdditionalPDCPDuplicationTNL-DL-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{

}

>>>>>>>>>>unrelated parts are skipped<<<<<<<<<<

DRBs-Setup-Item ::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-DL-List			PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex									PRESENCE optional },

...

}

>>>>>>>>>>unrelated parts are skipped<<<<<<<<<<

DRBs-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DC-Based-Duplication-Configured		CRITICALITY reject	EXTENSION DCBasedDuplicationConfigured				PRESENCE optional }|

{ ID id-DC-Based-Duplication-Activation		CRITICALITY reject	EXTENSION DuplicationActivation						PRESENCE optional }|

{ ID id-DLPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE mandatory }|

{ ID id-ULPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-SDTRLCBearerConfiguration			CRITICALITY ignore	EXTENSION SDTRLCBearerConfiguration					PRESENCE optional },

...

}

However, the id-AdditionalPDCPDuplicationTNL-List is reused in several places. Some of them should be updated to refer the new defined AdditionalPDCPDuplicationTNL-DL-List, while others in the DL messages (e.g. contained in DRBs-ToBeSetup-ItemExtIEs) should still use the original AdditionalPDCPDuplicationTNL-List IE.

Not sure such kind of change which will result in one ID refers to two different IE in different places is allowed or not, more discussion is necessary for the feasibility.

2.4	Option 3: Change tabular

Option 3: Change the tabular in 9.2.2.2, 9.2.2.8, and 9.2.2.10, to align with the ASN.1, and add semantics description to indicate the added optional BH information will not be used in this release. The change is backward compatible.

| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |

In summary, option 1 provides BC change but requires change on both ASN.1 and tabular, the feasibility of option 2 stills need more discussion, option 3 is the simplest way to fix the problem and the change is BC. Therefore, option 3 is promising and we propose

Proposal 1:	In order to fix the issue regarding BH information configuration to the additional F1-U tunnel, using option 3, i.e. Change the tabular in 9.2.2.2, 9.2.2.8, and 9.2.2.10, to align with the ASN.1.

3	Conclusion

This document proposes:

Proposal 1:	In order to fix the issue regarding BH information configuration to the additional F1-U tunnel, using option 3, i.e. Change the tabular in 9.2.2.2, 9.2.2.8, and 9.2.2.10, to align with the ASN.1.