3GPP TSG-RAN WG3 Meeting #121	R3-234222

# Toulouse, France, 21st – 25th August, 2023

Title: 	Altitude correction for the NTN TRP

Source: 	Huawei, Thales, Ericsson

Agenda item:	9.2

Document Type:	Discussion

# 1. Introduction

This contribution is continuing the discussion on the problem of the TRP Altitude for Positioning in case of NTN [3]. It is also proposed to introduce an NTN Access Point Position based on Earth-centered, Earth-fixed (ECEF) coordinates instead of Universal Geographical Area Description (GAD) as defined in TS 23.032 [1], used today for the RAN access point.

The discussion was initiated in rel-18 NTN IoT WI [3], but after further checking it seems the correction might be needed in release-17.

# 2. Discussion

We would like to noticed that SA2 discussed the UE location verification for EPS (LTE) and 5GS (NR) in rel-17. As a consequence, we do understand that the MME or the AMF is able to trigger a Positioning UE verification to the E-SMLC or LMF for an UE under NTN in rel-17. That’s the reason why the following discussion in RAN3 is related to rel-17. However, we also understand that RAN3 might not be the well group to decide the relevance of the E-SMLC and AMF positioning method selection, and we propose anyway to LS SA2 to advise them on the issue of the NTN TRP Altitude (GAD) and ask them to clarify on the release of the correction of NTN TRP Altitude.

The NTN Altitude problem discussed last meeting [3] could be summarize as follow; the NTN supports the RAN based positioning method like e.g. the DL-TDOA which requires the knowledge of the geographical coordinates of the TRPs (and their relative downlink timing). The geographical coordinates of the TRPs is defined in LPPa (resp. NRPPa) by the E-UTRAN Access Point Position (resp. NG-RAN Access Point Position). These Access Point Positions are defined according to GAD TS 23.032 [1]. The maximal altitude provided by GAD is ~32 km (32,768 m) where some satellite is generally reaching an orbital above e.g. 400 km to 2000 Km for (LEO), 8 000 to 20 000 for NGSO and 36 000 km for GEO.

It seems then it is not possible for the E-SMLC/LMF to get the correct Altitude of the TRP for providing positioning. We hence propose to correct this issue;

- We noticed recent discussion on moving TRP for mobile IAB was introduced in rel-18 [5]. However this proposal is referring to the locationEstimate IE as defined in TS 37.355, which refer to GAD. It seems also from our view not beneficial to “mix” the IAB and NTN feature at this stage. The approach could not be re-used

- There was also a discussion related last meeting on the Verified UE Location for NTN [4]. We do agree that the E-SMLC/LMF is able to derive from Ephemeris the real time TRP position and velocity, however an initial information is needed the TRP location, including Altitude and the time stamp of the TRP location report (similarly at Mobile IAB [5]). All this information allows to the E-SMLC/AMF to know the TRP location in following time, then compute the UE positioning.

- In current specification, the GAD is referenced only for the methodology to derived the Altitude. The altitude parameters are none extendable in the LPPa/NRPPa/etc … Some new IEs are then required.

It seems then a best way is to introduce a NTN Access Point Position based on ECEF. The X, Y, Z are currently aligned with the RRC e.g. EphemerisInfo IE. The CRs against LPPa and NRPPa are providing to the meeting.

We do not foresee impact on F1 due to transparent payload usage in current release, but similar change could be implemented in F1 to align the specifications…. It is up to RAN3 to discuss and decide.

Finally, we propose to endorse the CRs to LPPa and NRPPa pending to a LS to SA2, copy RAN1 and CT, group to clarify that the GAD is not enough as geo-system for the 3GPP dealing with NTN problems and confirm that the change apply to rel-17.

# 3. Conclusion

We do ask RAN3 to acknowledge the problem above and introduce an NTN Access Point Position, based on ECEF reference and liaise SA2 for advising them on issue and clarifying on which release the correction should apply:

- R3-234225 LS to SA2 on NTN Access Point Altitude

The document to be endorsed and attached to the SA2 LS:

- R3-234223 NTN Access Point Position; TS 38.455

- R3-234224 an NTN Access Point Position, TS 36.455

# 4. Reference

[1] 3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[2] IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[3] R3-232997, Altitude correction for the NTN TRP, Huawei

[4] R3-232789, Discussion on Network Verified UE Location for NTN, Qualcomm Incorporated

[5] R3-232835, Support for mobile TRP Location Information, Ericsson, Xiaomi, Qualcomm Inc., CATT, Nokia, Nokia Shanghai Bell, ZTE