3GPP TSG-RAN3 Meeting #121	R3-234254

# Toulouse, France, Aug 21st – 25th 2023

Agenda item:	23.2.1

Source:	Xiaomi

Title:	Discussion on Sidelink Positioning

Document for:	Discussion and Decision

1	Introduction

For sidelink positioning, there’re two leftover issues according to the discussion in last RAN3 meeting.

RAN3#120:

RSPP/SLPP transport QoS parameters shall be provided from AMF to the NG-RAN for scheduled resource allocation mode resource management.

No need to provide the Ranging/SL Positioning QoS parameters from AMF to NG-RAN.

Authorization information for UE Type/Role wait the progress in other group

SL-PRS configuration from LMF wait the progress in other group

In this contribution, we’d like to discuss the following issues for sidelink positioning:

- authorization information IE design

-SL-PRS resource allocation by gNB

2	Discussion

2.1 Sidelink positioning authorization information

RAN3 agreed to introduce authorization information, but have different views on the detail IE design, because the discussion in RAN and SA are not aligned.

SA2 situation

5.1.2	Authorisation and policy/parameter provisioning to NG-RAN

The "Ranging/Sidelink Positioning authorised" information and the RSPP transport QoS parameters shall be provided to the NG-RAN node for scheduled resource allocation mode resource management.

The "Ranging/Sidelink Positioning authorised" information includes one or more of the following:

-	whether the UE is authorized to act as a Reference UE;

-	whether the UE is authorized to act as a Target UE;

-	whether the UE is authorized to act as a Located UE;

-	whether the UE is authorized to act as a SL Positioning Server UE.

According to SA2’s latest specification TS 23.586 [1], there’re four types of authorization information to be provided to NG-RAN.

RAN3 discussion situation

According to the previous discussion, RAN3 agree to use the authorization information to allocate resources for the UE, and there’re two kinds of resources, i.e. SL communication resources and SL-PRS resources.

RAN1/2 situation

According to RAN1/RAN2 discussion and agreement, Reference UE and Located UE defined in SA2 are the anchor UE from RAN perspective, and RAN2 haven’t agree to introduce a server-only UE (i.e. SL Positioning Server UE), and RAN2 has no stage 2 call flow the SL positioning, it’s not clear whether other resources needs for authorization at current stage, there’s no guidance for RAN3 to consider more.

According to the above situations, we have the following analysis table for the resources that can be allocated for different UE types.

| SA2 defined name | RAN1/2 defined name | Resource for RSPP transmission | Resource for SL PRS |
| --- | --- | --- | --- |
| Reference UE | Anchor UE | Yes | Yes |
| Target UE | Target UE | Yes | Yes |
| Located UE | Anchor UE | Yes | Yes |
| SL positioning server UE | Server-only UE | Yes | No |

Based on the above table, we propose the following options as way forward:

| Way forward | RAN3 impacts | SA2 impacts | Any issue |
| --- | --- | --- | --- |
| Option 1, introduce the authorization information defined by SA2 as it is, NG-RAN node can use the information accordingly and appropriately by its implementation. | Introduce different authorization UE types as defined by SA2 | No impact | No issue |
| Option 2, introduce two types of services for SL authorization information - RSPP communication service - SL PRS transmission service | Introduce two IEs for SL positioning authorization | SA2 needs to update their specification to align RAN3 | No issue |
| Option 3, introduce only one type SL authorization information. | Introduce one IE for SL positioning authorization | SA2 needs to update their specification to align RAN3 | Cannot differentiate the authorization for RSPP resource and SL PRS resource |

we prefer option 1 which is a simple way, but we’re open to discuss other options or solutions.

Observation 1, four types of UE authorization information are introduced in SA2 specification.

Observation 2, RAN3 identify two different resources for sidelink positioning, i.e. RSPP transmission resources and SL PRS resources.

Observation 3, there’s no useful progress from RAN1/2 to guide RAN3 think about the details for resource authorization.

Proposal 1, RAN3 discuss and down-select the following options as way forward.

- Option 1, introduce the authorization information defined by SA2 as it is, NG-RAN node can use the information accordingly and appropriately by its implementation.

- Option 2, introduce two types of services (i.e. RSPP communication service and SL PRS transmission service) for SL authorization information, which may have impact on SA2

- Option 3, introduce only one type SL authorization information, which may impact on SA2 and have issues.

2.2 SL-PRS resource allocation by gNB

For sidelink positioning resource allocation, we agree that the gNB is responsible for SL-PRS resource allocation in case mode 1 is used.

According to chapter 6.20 Ranging/Sidelink Positioning procedures in TS 23.273[2], although SA2 already specifies the stage 2 procedure with LMF involvement, it’s not clear how NG-RAN node allocate SL PRS resources.

On the other hand, SL communication resources allocation is between gNB and UE via RRC, while positioning resource allocation should involve LMF, which means the coordination between LMF and gNB may be needed. However, it’s not clear how to allocate SL-PRS resource, we think there are three possible ways for UE and serving gNB to negotiate SL-PRS resources:

- UE<-RRC signalling-> gNB

- UE<-Lower layer signalling-> gNB

- UE<- RSPP->LMF<-NRPPa-> gNB

According to current RAN1 discussion, most of the SL PRS parameters can be exchanged via lower layer signalling, so it’s hard to discuss the information transferred between LMF and gNB for SL-PRS resource allocation.

Observation 4, how to allocate SL-PRS resource is not clear according to RAN1/2 and SA2 progress.

Proposal 2, RAN3 discuss how NRPPa needs to be enhanced when RAN2 has a clear call flow for SL PRS positioning.

3	Conclusion

In this contribution, we had the following observations and proposals.

Sidelink positioning authorization information

Observation 1, four types of UE authorization information are introduced in SA2 specification.

Observation 2, RAN3 identify two different resources for sidelink positioning, i.e. RSPP transmission resources and SL PRS resources.

Observation 3, there’s no useful progress from RAN1/2 to guide RAN3 think about the details for resource authorization.

Proposal 1, RAN3 discuss and down-select the following options as way forward.

- Option 1, introduce the authorization information defined by SA2 as it is, NG-RAN node can use the information accordingly and appropriately by its implementation.

- Option 2, introduce two types of services (i.e. RSPP communication service and SL PRS transmission service) for SL authorization information, which may have impact on SA2

- Option 3, introduce only one type SL authorization information, which may impact on SA2 and have issues.

SL-PRS resource allocation by gNB

Observation 4, how to allocate SL-PRS resource is not clear according to RAN1/2 and SA2 progress.

Proposal 2, RAN3 discuss how NRPPa needs to be enhanced when RAN2 has a clear call flow for SL PRS positioning.

4	References

[1] TS 23.586 V18.0.0 Ranging based services and Sidelink Positioning, SA2

[2] TS 23.273 V18.2.0 5G System (5GS) Location Services (LCS), SA2