3GPP TSG-RAN3 Meeting #121	R3-234257

# Toulouse, France, Aug 21st – 25th 2023

Agenda item:	25.2

Source:	Xiaomi

Title:	Discussion on enhancement for XR

Document for:	Discussion and Decision

1	Introduction

According to the work item description [1] for Expanded and Improved NR Positioning, the aspects related to RAN3 are highlighted as below.

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

In this contribution, we’d like to discuss the following topics according to the WID and RAN2/SA2’s progress.

Discard operation of PDU Sets

PDU Set based handing

XR traffic assistance information

Congestion control

2	Discussion

2.1	Discard operation of PDU Sets

5.7.7.4	PDU Set Integrated Handling Information

The PDU Set Integrated Handling Information (PSIHI) indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. PSIHI is an optional parameter.

According to TS 23.501[2], a new QoS parameter related to discard operation is introduced, which is used to indicate whether all PDUs of the PDU set are needed, if not, and if one PDU is missed, other PDUs of the PDU set may not be useful and can be discarded.

RAN2 had already discussed PDU discard operation for UL, and agreed that the discard operation is performed in UE PDCP. For DL PDU discard, it can be performed in gNB, and the serving gNB of the UE needs to consider the PSIHI to decide whether discard all the PDUs of the PDU set if at least one PDU of the PDU set is missed. And this implies that the PSIHI should be introduced as part of QoS profile in UE context over NGAP and XnAP.

In addition, RAN3 needs to discuss which entity is responsible for PDU discard in CU-DU split architecture. In our view, at least CU-UP that receives the DL PDU from UPF should be responsible for PDU discard, this implies that the PSIHI should be introduced over E1AP.

Proposal 1, RAN3 agrees that the serving gNB is responsible for DL PDU discard operation.

Proposal 2, RAN3 agrees to introduce PSIHI as an optional IE in QoS Flow Level QoS Parameters IE over NGAP and XnAP.

Proposal 3, RAN3 agrees to introduce PSIHI as an optional IE in QoS Flow Level QoS Parameters IE over E1AP.

2.2	PDU Set based handing

According to TS 23.501[2], a PDU Set is comprised of one or more PDUs carrying an application layer payload such as, e.g. a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS parameters from SMF and PDU Set information provided by the PSA UPF.

The PDU set QoS parameters from SMF are the followings:

1.	PDU Set Delay Budget (PSDB).

2.	PDU Set Error Rate (PSER).

3.	PDU Set Integrated Handling Information (PSIHI).

They can be provided from SMF to NG-RAN as part of the QoS Profile over NGAP. In addition, the PDU set QoS parameters should also be transferred from source NG-RAN node to the target NG-RAN node, from MN to SN, and from the last serving gNB to the new serving gNB.

For PSDB, it can be used by the NG-RAN to support the configuration of scheduling and link layer functions, so in CU-DU split architecture, the PSDB needs to be provided to gNB-DU.

For PSER, “the purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access)”, so in CU-DU split architecture, the PSER needs to be provided to gNB-DU.

According to SA2, “When the PSDB is available, the PSDB supersedes the PDB for the given QoS Flow.” And “If the PSER is available, the PSER supersedes the PER.”, this indicate if the PSDB and/or PSER are present in QoS profile, the PDB and/or PSER should be ignored, this can be stated in the semantics descriptions in the IE.

Proposal 4, RAN3 agrees to introduce PSDB, PSER as part of QoS profile over NGAP, XnAP, F1AP.

Proposal 5, if PSDB/PSER is available, the PDB/PER should be ignored.

Regarding the PDU Set information from UPF, according to TS 23.501, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

In CU-DU split architecture, if supported, the CU-UP can identify the PDU set information by reading the GTP-U header, and generate new GTP-U packet with the PDU set information in the GTP-U header to DU, so that the DU can perform PDU set handling according to the PDU set information.

Proposal 6, RAN3 agree to introduce the following PDU set information over NG-U and F1-U

-	PDU Set Sequence Number

-	Indication of End PDU of the PDU Set

-	PDU Sequence Number

-	PDU Set Size

-	PDU Set Importance

2.3	XR traffic assistance information

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

The XR traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX, and the information includes the following:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst.

As mentioned in SA2’s specification and reply LS on TSCAI for XR [3], SA2 agree to use TSCAI for both GBR and non-GBR QoS flow, to provide the UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity.

And TSC Assistance Information (TSCAI) has already supported by specification for GBR QoS flow as below.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

To support non-GBR QoS flow, the simplest way is to remove the semantics description or change the description to support both kinds of QoS flow.

And the indication of end of Data Burst can be provided by UPF to NG-RAN, this implies that both NG-U and F1-U should be enhanced to support the transfer of this indication.

Proposal 7, RAN3 agree to remove the semantics description in TSC Traffic Characteristics IE over NGAP.

Proposal 8, RAN3 agree to introduce end of Data Burst indication over NG-U and F1-U.

2.4	Congestion control

5.37.3.2	Support of ECN marking for L4S in NG-RAN

ECN marking for L4S may be supported in NG-RAN as specified in TS 38.300 [27].

To enable support of ECN marking for L4S in NG-RAN, dedicated QoS Flow(s) are used for carrying L4S enabled IP traffic. The SMF may be configured to, based on PCC Rule provide an indication for ECN marking for L4S to NG-RAN for a corresponding QoS Flow(s), but in the absence of such configuration the use of L4S on a QoS flow is controlled by a coordinated configuration in NG-RAN and 5GC.

The criteria based on which NG-RAN decides to mark ECN bits for L4S is NG-RAN implementation specific.

For congestion control, ECN marking for L4S to expose the congestion information, and ECN marking for the L4S in the IP header is supported in either the NG-RAN or in the PSA UPF.

According to the above text in TS 23.501, both UPF and NG-RAN can perform ECN marking for L4S, if needed, the SMF needs to indicate NG-RAN to perform it. If an indication is received by NG-RAN, and if congestion happens, the NG-RAN can perform the ECN marking.

In addition, if congestion happens, the gNB can perform the PDU discard according to its own load and PSI received in PDU.

Proposal 9, RAN3 agree to introduce an indication over NGAP from SMF to NG-RAN node to trigger ECN marking in NG-RAN for congestion purpose.

Proposal 10, PSI should be considered when discard operation during congestion is performed.

3	Conclusion

In this contribution, we had the following observations and proposals:

Discard operation of PDU Sets

Proposal 1, RAN3 agrees that the serving gNB is responsible for DL PDU discard operation.

Proposal 2, RAN3 agrees to introduce PSIHI as an optional IE in QoS Flow Level QoS Parameters IE over NGAP and XnAP.

Proposal 3, RAN3 agrees to introduce PSIHI as an optional IE in QoS Flow Level QoS Parameters IE over E1AP.

PDU Set based handing

Proposal 4, RAN3 agrees to introduce PSDB, PSER as part of QoS profile over NGAP, XnAP, F1AP.

Proposal 5, if PSDB/PSER is available, the PDB/PER should be ignored.

Proposal 6, RAN3 agree to introduce the following PDU set information over NG-U and F1-U

-	PDU Set Sequence Number

-	Indication of End PDU of the PDU Set

-	PDU Sequence Number

-	PDU Set Size

-	PDU Set Importance

XR traffic assistance information

Proposal 7, RAN3 agree to remove the semantics description in TSC Traffic Characteristics IE over NGAP.

Proposal 8, RAN3 agree to introduce end of Data Burst indication over NG-U and F1-U.

Congestion control

Proposal 9, RAN3 agree to introduce an indication over NGAP from SMF to NG-RAN node to trigger ECN marking in NG-RAN for congestion purpose.

Proposal 10, PSI should be considered when discard operation during congestion is performed.

4	References

[1] RP-230786, Nokia et al, "Updated WID on XR Enhancements for NR"

[2] TS 23.501 V18.2.2 System architecture for the 5G System (5GS);

[3] S2-2308197, LS reply on TSCAI for XR, SA2