3GPP TSG-RAN3 #120	R3-234290

21st August – 25th August 2023

Toulouse, France

Title: 	(TP for AI/ML BLCR to TS38.300) Characteristics of the procedures for exchanging AI/ML-related information

Source: 	Ericsson, InterDigital

Agenda item:	12.2.1

Document Type:	Discussion and Decision

# TP for TS 38.300

Below, a draft TP is presented where the proposals made in R3-234328 are reflected

<<<<<< Start of CHANGE >>>>>>

## X.X	AI/ML for NG-RAN

### X.X.1	General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

### X.X.2	Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

Signalling procedures used for the exchange of AI/ML related information are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

Examples of data that can be requested from an NG-RAN node via these procedures are Predicted Resource Status Information (i.e., predicted radio resources, predicted number of active UEs, and predicted number of RRC Connections), Cell-based UE trajectory prediction, Energy Cost (EC).

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

For the scenario where AI/ML Model Training is located in the OAM, the inputs are sent from NG-RAN directly to OAM.

The prediction information as well as UE performance feedback can be configured to be reported once or periodically. This information can be used for e.g. the Load Balancing use case and it may be exchanged between NG-RAN nodes for e.g. subsequent UEs’ mobility decisions. The reporting is configured through AI/ML Information request/response procedure (FFS on the name), while actual reporting is performed through AI/ML Information update procedure (FFS on the name).

The cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Request message to provide information for e.g. subsequent mobility decisions.

The metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced as the AI/ML metric to be exchanged with the neighbouring NG-RAN nodes upon request.

<<<<<< End of CHANGE >>>>>>