3GPP TSG-RAN WG3 #121							R3-234312

Toulouse, FR, 21st  Aug – 25th Aug 2023

Agenda Item:	10.2.3

Source:	Ericsson

Title:	RACH Optimization enhancement

Document for:	Discussion, Decision

# 1	Introduction

During RAN3-120 meeting, discussions on RACH optimisation were carried out and some agreements were made. However, the following FFSs were captured:

RA report forwarding between network nodes-

  - FFS whether and how to forward the RA Report in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report.

RACH report information enhancements-

  - Option 1: Include the feature priority

  - Option 2: Include the RACH partition configuration related information (e.g., start preamble / number of preambles in the RA partition)

  - Option 3: Include the time between RACH access that led to the generation of a RA Report and reporting of the RA Report

  - Option 4: The network controls the UE to report RA information

  - Option 5: Mobility information

In this contribution, we present our opinions on the FFSs and open issues on these topics.

# RACH Optimization enhancement

## 2.1 RA report forwarding between network nodes

Based on the RAN2 LS received in RAN3#119bis-e meeting regarding discussion on inclusion of NR RA report into LTE RA report as an octet string and additionally different options of inclusion of PSCell IDs, it was agreed to include a list of unique PSCell identities in the RA report. To mirror the RAN2 agreement and for routing purpose, the following decision was agreed in RAN3#120-Incheon meeting.

Include the “PSCell list” optionally in addition to the “RACH Report Container” in ACCESS AND MOBILITY INDICATION over Xn.

However, inclusion of only a list of PSCell CGIs is incomplete if the eNB receiving the RA report does not have X2/Xn interfaces with the SN(s) serving the PSCell CGIs included in the report.

- A RA report forwarding solution based on PSCell information is incomplete if the eNB receiving the RA report does not have X2/Xn interface connections with the SN(s) hosting the PSCell(s) in the report.

In this scenario, a robust solution would comprise the eNB to forward the RA report using S1/NG interface to the target node. However, in RAN3#120 meeting it was agreed not to consider NG and S1 forwarding of the RA report, as captured in the agreement below.

Not consider the NG and S1 forwarding of RA report.

On the other hand, especially in EN-DC, it is likely that the node collecting the RA report from the UE has no Xn/X2 interface with the node hosting PSCell, but such node has a Xn/X2 interface with the node hosting the PCell associated to the MN connected to the SN hosting the PSCell. 
In this scenario, the node could forward the RA report to the node hosting the PCell (the MN at the time of RA Report creation) which would be able to re-distribute the report to the node(s) hosting the PSCell(s). Such mechanism could potentially reduce loss of RA report and reduce network interface signalling load that would derive from a trial and error type of RA Report forwarding, in the attempt to finally find the node hosting the PSCells.

UE logs the PCell CGI of the MN and reports them to the NG-RAN along with the PSCell information and the NR RA report container as part of the LTE RA report.

## 2.2 RACH partitioning feature in RA report

In this section we analyse each information so far proposed to be added in the RA report to enable the network to further optimise RACH configurations.

Option 1: Feature Priorities: namely the priority of features at the time RACH access was carried out

It needs to be reminded that RAN2 already agreed to the following:

Agree to add the following parameters into RACH report for RACH partitioning:

-     Feature or the combination of features that triggered the RACH

-     Used feature combination

With the above agreement, RAN2 agreed that the UE shall report the feature, or combination of features that triggered the UE to select a specific RACH partitioning and that triggered the UE to perform RACH access.

RAN2 also agreed that the UE shall report also the Feature Combination associated to the RACH partitioning where RACH access is carried out. In light of the agreements from RAN2 it makes only sense that the UE reports also the Feature Priority configured for features at the time of RACH access. This information, combined with the information agreed in RAN2 allows to understand the following:

- The RAN can understand which feature has triggered the UE to select a RACH partition and perform RACH access. In case many UEs are triggered by the same feature or set of features, a RACH optimisation action could be to create a dedicated RACH partition for the most demanded features

- The RAN can understand to which RACH partition the UE performed RACH access. This allows the RAN to understand which partitions are most demanded and therefore most overloaded

- The RAN can understand which feature priority was given to the feature or set of features that triggered RACH access. This allows the RAN to understand whether grouping of features is done optimally or not. For example, if a RACH partition is associated to features with different priorities and if such partition is overloaded, one RACH optimisation action may be to split the features in different priority groups

RAN3 to send an LS to RAN2 requesting the addition of feature priorities as part of the RA Report

Option 2, 3: The set of preambles allocated to the RA partition such as the start preamble index and/or the number of preambles in the partition (RACH configuration related information) and Time between RA attempt to RA Report

This information is meant to provide the RAN with information about the RACH configuration in use at the time the RA Report was generated.

The rationale is that if the RA report is reported after the RACH partitions configuration has changed, the RAN would not be able to determine what RACH configuration was in use at the time the RA report was generated. This not only prevents the RAN to derive the right optimisation action, but it might have negative effects on the system. Indeed, if the RAN assumes that the RACH configuration at the time the RA report was generated was the same as the actual RACH configuration, the RAN may determine RACH reconfiguration actions that might have negative impacts.

The best approach to solve this issue is to let the UE include in the RA report the start preamble index and the number of preambles in the partition the UE used at the time of performing RACH access. This has been called Option 2.

An alternative approach to reporting the start preamble index and the number of preambles in the RACH partition used would be to let the UE report the time between RA report generation and RA report signalling to the RAN. This has been called Option 3. With this time measure the RAN may be able to deduce what RACH configuration was in use at the time the RA report was generated. However, this approach is less efficient because it carries impacts on both the UE (which has to store the new time variable) and the network (which has to store and timestamp all the RACH configurations adopted in the past).

Option 4: The network controls the UE to report RA information

Another alternative solution, named Option 4, has been proposed in RAN3#119bis-e meeting. In this approach the network controls UEs to report RA information. In principle, the network indicates to the UE if the UE should send the RA report related to the latest RA configuration or for all RA configurations the UE may have encountered. We note that, in this solution, the UE needs to store details on the RACH configuration(s) the UE encounters at every RA Report generation, similar to the solution where the UE needs to include preamble index. This is needed for the UE to be able to filter the RA Reports according to the criteria given by the network. 
Moreover, in case the UE reports RA Reports from new and old RA configurations (as a consequence of the network asking for RA Reports for all configurations), and since the UE does not report the RACH configuration information along with the report, the network does not have the possibility to correlate the RA reports with the corresponding configurations and in turn, is not able to perform any meaningful optimization. Namely, in order to enable the network to understand to which RA partition configuration the RA Report belongs, Option 4 will have to be complemented with information similar to the one in Option 2. We therefore conclude that Option 2 would also include the functionalities of Option 4 and that Option 4 is at best a special version of Option 2.

- In the solution where network controls UEs to report RA information (Option 4), correlation of RA reports with RA partition configuration is not possible. Hence, network would be unable to perform any meaningful optimization.

Hence, we propose the following-

RAN3 does not include option-4 into the LS to RAN2

- Option 5: Mobility Information

In RAN3 119bis-e meeting, a proposal was introduced where network can store mobility information in the UE by means of signalling to the UE an undefined number of Octets and where such Octets are encoded proprietarily according to what the network vendor decides to store in it. The network may fetch the information from the UE at a later stage and perform optimization accordingly. As an example that applies to RACH optimisation, the network may store in such field the RA partition configuration of each RA partition for which the UE generates a RA Report.

The advantage of the solution is claimed to have a unified impact on all SON solutions.  The network, based on the stored information may deduce information concerning different SON reports, such as RA report, SPR, SHR, RLF report. However, in our opinion, such a solution requires further studies with a careful consideration of some aspects, namely-

- HOw does the solution work in cases like RACH Optimisation: will the UE store the Mobility Information field for every RA Report generated, just like it would store RA partition information e.g. in Option 2?

- What is the size of such field and what should it contain: is it one field per SON report, or should the field cater for enough space to host information about all SON reports existing?

- How is this field maintained across Releases? As SON features evolve, it is likely that more information from the UE may be needed and/or more SON reports would be specified. Would the field be extendible to cater for future demands?

- For how long would the UE maintain the information stored?

- What is the storage and reporting format of the information?

- What are the interoperability requirements for the content of such field? Take the RLF Report as an example: This report may be received by a first network node, which then forwards it to a second network node, therefore the report has to contain standardized information and its handling has to be inter-vendor interoperable. Would the same requirements appliy to this new field?

- Are there any possible inter-operator dependencies? For example, in a shared RAN network, could Operator 1 decide to encode the field in a given whay, while Operator 2 decide to encode the field in a different way? Would this generate interoperability issues?

- The solution of network storing mobility information in the UE is made of interesting concepts, but it requires further studies for the complete solution to be analysed and decided upon.

One general observation is that Option 1, 2, 3 4 have been under study and discussion in RAN3 since beginning of Rel-18 and thus their pros and cons are well known and analysed.

These options are mature to be described to RAN2 and for RAN2 to give their opinion on them. Companies in RAN3 shold not stop progress on options that have been on the table and are ready to be communicated to other groups only because other options introduced (much) later need further analysis and details.  The latter is the case of Option 5, which is not well explored and should not be a bottleneck to send an LS to RAN2 with the solution sRAN3 has already discussed.

We would like to point out that we see some benefits in Option 5, but as discussed, we think more details are needed to understand how this solution would work. We encourage further discussions on Option 5 and we suggest that, once the description of this solution becomes clearer, a separate LS can be sent to RAN2 asking RAN2 for feedback.

RAN3 to study the solution in Option 5, where the network stores mobility information at the UE and to achieve a clear position on the feasibility and benefits of this solution before sending an LS to RAN2 including such solution as a possible candidate for SON.

Hence, we propose the following-

RAN3 to send an LS to RAN2 requesting the addition of the start preamble index and the number of preambles in the partition the UE used at the time of performing RACH access (Option 2)

A draft LS mirroring proposal 2 and proposal 5 is attached in the annex.

## 2.3 SDT Use Cases

In past RAN3 meetings the possibility of applying SON and RA optimisatoin to SDT has been presented. It would be useful for the UE to include SDT information in the RA report for the purpose of revealing how much traffic was signalled as “small data”. In one example, if the amount of traffic signalled by the UE slightly exceeds the SDT threshold, the UE will transmit such traffic via normal data channels instead of via SDT transfer.

If the NG-RAN knew of how much the SDT threshold has been exceeded, the NG-RAN cold optimise the SDT threshold and allow for optimal SDT transfer utilisation. For example, the network may slightly increase the SDT threshold to trigger a much higher usage of SDT transfers.

RAN3 to send an LS to RAN2 requesting the addition in the RA report of small data traffic volume, when such small data volume exceeds the SDT threshold up to a configured amount

# Conclusion

In this paper we have analysed the remaining issues on RACH optimisation and derived the following Observations and Proposals:

Observation 1	A RA report forwarding solution based on PSCell information is incomplete if the eNB receiving the RA report does not have X2/Xn interface connections with the SN(s) hosting the PSCell(s) in the report.

Proposal 1	UE logs the PCell CGI of the MN and reports them to the NG-RAN along with the PSCell information and the NR RA report container as part of the LTE RA report.

Proposal 2	RAN3 to send an LS to RAN2 requesting the addition of feature priorities as part of the RA Report

Observation 2	In the solution where network controls UEs to report RA information (Option 4), correlation of RA reports with RA partition configuration is not possible. Hence, network would be unable to perform any meaningful optimization.

Proposal 3	RAN3 does not include option-4 into the LS to RAN2

Observation 3	The solution of network storing mobility information in the UE is made of interesting concepts, but it requires further studies for the complete solution to be analysed and decided upon.

Proposal 4	RAN3 to study the solution in Option 5, where the network stores mobility information at the UE and to achieve a clear position on the feasibility and benefits of this solution before sending an LS to RAN2 including such solution as a possible candidate for SON.

Proposal 5	RAN3 to send an LS to RAN2 requesting the addition of the start preamble index and the number of preambles in the partition the UE used at the time of performing RACH access (Option 2)

Proposal 6	RAN3 to send an LS to RAN2 requesting the addition in the RA report of small data traffic volume, when such small data volume exceeds the SDT threshold up to a configured amount

An LS mirroring Proposal 2, 5 and 6 is added in the Annex.

# Annex –

## LS to RAN2 on RACH optimisation

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, FR 21st  Aug – 25th Aug 2023

Title:	LS on RACH Optimisation

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name: 		Angelo Centonza

E-mail Address:	angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

As part of the work on SON for NG-RAN, RAN3 discussed enhancements to the RACH optimisation functions.

As part of such discussions RAN3 identified potential enhancements to the RA report collected by UEs and signalled to the NG-RAN. Such enhancements are listed below:

- Enable the addition in the RA Report of the feature priority of each feature in the feature combination used by the UE at the time RACH access is triggered. This enables the NG-RAN to determine whether any optimisation is needed with respect to how features with different priorities are combined in the same feature combination associated to a RACH partition

- Enable the addition in the RA Report of RACH partition configuration information. This information consists of the start preamble index and the number of preambles in the partition for which the RA Report was generated. This enables the NG-RAN to determine the RACH partition configuration at the time the RA Report was generated. This enhancement is needed because the RACH partitions configurations may change with time and a RA Report generated under certain RACH partition configurations may be reported to the NG-RAN after such RACH partition configurations have changed.

- Include the data volume in the RA report if the volume exceeds the SDT threshold up to a configured amount. Using this information, network would be able to determine if it needs some optimisations for the SDT threshold configuration.

- 2. Actions:

RAN3 respectfully asks RAN2 to take it the above into account and to provide feedback on whether the proposed additions are feasible and can be specified in Rel18.

3. Date of next TSG RAN WG3 meetings:

3GPP RAN2#121-bis	from 2023-10-09	to 2023-10-13		Xiamen , CN