3GPP TSG-RAN WG3 #121	R3-234319

# Toulouse, France, 21st  – 25th  August 2023

Agenda Item:	24.2

Source:	Ericsson

Title: 	Introduction of Network Energy Saving

Document for:	Discussion and Approval

# 1	Background

The new Work Item NES has been approved, refer to [1].

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

Objective 2 and object 6 have explicit RAN3 objectives, which we should work on the stage 3 specification.

At the last RAN3 meeting, we started the WID and had some discussion on the objectives. This paper continues the discussion.

This paper also include discussion on the incoming LSs from the other 3GPP groups.

# 2	Discussion

## 2.1	Incoming LSs

Related to the paging enhancement, we have got reply LS from SA2 in [2].

Q2 below is related to RAN3’s discussion on the enhancements to restrict paging in a limited area for UEs in RRC IDLE state based on the following mechanism.

Q2: RAN3 would respectfully like to ask SA2 if there are any issues from CN perspective with this mechanism. Answer: SA2 does not see any showstoppers from CN perspective for the enhancements to restrict paging in a limited area for UEs in RRC IDLE. SA2 requests RAN3 to inform SA2 when the related CRs are agreed in order to proceed with alignment CRs in specifications under SA2 control. SA2 would also like to validate the following assumption: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how paging escalation happen will be controlled by the AMF based on existing procedures. and ask the following questions: Q1: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation? Q2: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"?

Observation 1: SA2 confirmed that the RAN3 discussion related to NES paging enhancement for RRC_IDLE is not a showstopper.

Related to Q1, in our view gNB primarily uses the UE subscription information to determine if the UE is stationary. gNB may also conclude via other means, such as monitoring UE low mobility.

Proposal 1: RAN3 to reply to Q1 with “gNB primarily uses the UE subscription information to determine if the UE is stationary. gNB may also conclude via other means, such as monitoring UE low mobility”.

Related to Q2, in our view, when UE is static and low mobility, the “List of recommended beams” would not be changed often, or perhaps not changed at all. AMF could store the information until such a new list is received.

Proposal 2: RAN3 to reply to Q2 with AMF could store the information until such a new list is received.

## 2.2	Impact on XnAP TS 38.423

The below existing procedure could be used to implement inter-node information exchange on cell DTX/DRX:

### 8.4.2 NG-RAN node Configuration Update #### 8.4.2.1 General The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface. NOTE: Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified. The procedure uses non UE-associated signalling. #### 8.4.2.2 Successful Operation Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The cell DTRX information can be carried at cell level.

Proposal 3: RAN3 to discuss and agree that NG-RAN node1 uses “NG-RAN node configuration Update” procedure to update the cell DTRX information to NG-RAN node 2 over XnAP.

In [2], we have further discussed an enhancement on the inter-node coordination, that is the Cell DTRX can be coordinated among the neighbouring nodes:

The “Cell DTRX information Report” in the above is the NG-RAN node configuration Update, and the “Cell DTRX information coordination” is the NG-RAN node configuration Update Acknowledge.

Proposal 4: RAN3 to discuss and agree that to coordinate the cell DTRX information among the NG-RAN nodes over XnAP;

Proposal 5: RAN3 to discuss and agree that NG-RAN node x uses “NG-RAN node configuration Update Acknowledge” procedure to send the coordinated value to NG-RAN node 1 over XnAP.

In some cases, the SSB beam is activated so that the UE could perform measurement. After that, the SSBs can be deactivated again.

Proposal 6: It is proposed that RAN3 to discuss and agree to include a timer in the beam activation information in the Cell Activation request over XnAP and F1AP.

## 2.3	Impact on F1AP TS 38.473

There are two existing procedures over F1AP for gNB-CU and DU to update the configuration.

### 8.2.4 gNB-DU Configuration Update #### 8.2.4.1 General The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling. NOTE: Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified. #### 8.2.4.2 Successful Operation Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

Proposal 7: RAN3 to discuss and agree to include the Cell DTRX information in F1 Setup and gNB-DU over F1AP.

# 3	Proposals

Observation 1: SA2 confirmed that the RAN3 discussion related to NES paging enhancement for RRC_IDLE is not a showstopper.

Proposal 1: RAN3 to reply to Q1 with “gNB primarily uses the UE subscription information to determine if the UE is stationary. gNB may also conclude via other means, such as monitoring UE low mobility”.

Proposal 2: RAN3 to reply to Q2 with AMF could store the information until such a new list is received

Proposal 3: RAN3 to discuss and agree that NG-RAN node1 uses “NG-RAN node configuration Update” procedure to update the cell DTRX information to NG-RAN node 2 over XnAP

Proposal 4: RAN3 to discuss and agree that to coordinate the cell DTRX information among the NG-RAN nodes over XnAP

Proposal 5: RAN3 to discuss and agree that NG-RAN node x uses “NG-RAN node configuration Update Acknowledge” procedure to send the coordinated value to NG-RAN node 1 over XnAP

Proposal 6: It is proposed that RAN3 to discuss and agree to include a timer in the beam activation information in the Cell Activation request over XnAP and F1AP.

Proposal 7: RAN3 to discuss and agree to include the Cell DTRX information in F1 Setup and gNB-DU over F1AP.

The Text Proposal is submitted in [5].

# 4	Reference

[1]	RP-221443	Study on network energy savings for NR

[2]	S2-2307984	Reply LS on the enhancements to restricting paging in a limited area

[3]	R1-2306246	Reply LS on cell DTX/DRX activation/deactivation

[4]	R3-226457	Further discussion on SI Network Energy Saving, Ericsson

[5]	R3-234320	Text Proposal on F1AP: Introduction of Network Energy Saving, Ericsson