3GPP TSG-RAN WG3 Meeting #121	R3-234342

Toulouse, France, 21 – 25 August, 2023

Agenda item:	12.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	AI/ML Mobility Optimization

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss some remaining aspects of AI/ML Mobility Optimization use case.

In RAN3 #120 the following agreements were achieved with respect to predicted UE Trajectory:

In Rel18, the UE trajectory prediction is only limited to the next one hop target NGRAN node, and the source NGRAN node can collect measured UE trajectory (in the form of UE History Information) related to one or multiple cells within the single next hop target NGRAN node after handing over the concerned UE(s).

Take the procedure for UE performance feedback report as the baseline to also support the measured UE trajectory collection by the source gNB.

The details on how to request the measured UE trajectory collection needs to be further discussed.

Furthermore, we discuss possible standardization impacts from some of the agreed outputs for AI/ML Mobility Enhancements that were agreed in TR 37.817. Specifically, the following outputs were agreed:

- Estimated arrival probability in CHO and relevant confidence interval

- Predicted handover target node, candidate cells in CHO, may together with the confidence of the prediction

- Priority, handover execution timing, predicted resource reservation time window for CHO.

In this contribution, we discuss the above and provide our views.

# 2	UE Trajectory Prediction

In the last RAN3 meeting we agreed that UE Trajectory prediction is only limited to the next hop target NG-RAN node. A node can train a ML Model for cell-based UE Trajectory prediction by receiving UE trajectory in the form of UE History Information related to one or more cells within the next hop target NG-RAN node. A question that arises is how to collect data related to UE Trajectory for training a ML Model. A simple way to collect this data is to configure a measurement related to UE Trajectory information in the AI/ML Class 1 procedure (in the AI/ML INFORMATION REQUEST (name FFS) message), similarly to how we configure UE Performance Feedback.

Proposal 1: UE Trajectory information can be collected from a target NG-RAN node by configuring a measurement related to UE Trajectory information in the AI/ML INFORMATION REQUEST (name FFS) message.

Actual reporting of the UE trajectory information from the target node can be triggered per Handed over UE (in a similar way as UE Performance Feedback is collected).

Proposal 2: The source NG-RAN node triggers UE Trajectory information reporting by the target NG-RAN node by including the corresponding measurement ID in the HANDOVER REQUEST message.

For each UE being received at the target NG-RAN node for which a UE Trajectory information is requested during the Handover procedure, the target NG-RAN node provides UE Trajectory Information in the AI/ML INFORMATION UPDATE (name FFS) message.

Proposal 3: UE Trajectory information is sent by the target NG-RAN node to the source NG-RAN node in the AI/ML INFORMATION UPDATE (name FFS) for each UE being received at the target NG-RAN node for which a UE Trajectory information is requested during the Handover procedure.

A NG-RAN node in this way may receive enough information to train a ML Model for UE Trajectory prediction. One of the described solutions in the TR 37.817 considers AI/ML Training in the OAM. Therefore, training a ML Model for Trajectory Prediction in the OAM is also in the scope of the Rel.18 use cases.

Observation 1: Training a ML Model for Trajectory Prediction in the OAM is in the scope of Rel.18 use cases.

For data collection for cell-based UE trajectory prediction in OAM, OAM may trigger the data collection by requesting the RAN for UE Trajectory information. OAM could therefore request this information from a NG-RAN node which will be the source node of collected UE Trajectory information. The NG-RAN node could sent this information back to OAM, which can subsequently train the corresponding ML Model. Note that different NG-RAN nodes may be managed by different OAM systems. Therefore, by forwarding of UE Trajectory Information from a NG-RAN node to its managing OAM it is possible that different OAM systems train independently models for cell-based UE Trajectory prediction. The trained ML Models may be subsequently deployed to RAN for execution.

Proposal 4: For the solution where Model Training is in OAM, a NG-RAN node’s OAM may request the node for UE Trajectory information which allows different OAM systems to independently train models for cell-based UE Trajectory Prediction.

In RAN3, we have agreed that predicted UE Trajectory in the form of UE History Information is sent in the Handover Request message towards the target node. However, in certain situations it is possible that there is no unique trajectory that has maximum likelihood to happen and different (forking) points may exist in the prediction. For example, in certain cases, a UE may have the same probability to visit 2 cells in the trajectory leading to a forking point. This forking probability affects the overall confidence with which a trajectory is predicted since the Model may not decide a single trajectory from the forking point onwards.

Observation 2: Forking points may exist in the predicted trajectory when a UE may visit 2 (or more) cells with the same probability.

The current encoding of the predicted UE Trajectory assumes that a single predicted trajectory is possible and does not take into account a possibility that there are more than one equally likely cells that could be next hops at a forking point.

Observation 3: The current encoding of the predicted UE Trajectory does not take into account the possibility that there may be forks in the trajectory prediction, when there can be more than one trajectory with similar or significantly high likelihood.

Proposal 5: Extend the current trajectory prediction to comprise a set of possible future trajectories, allowing the possibility of forking points where more than one cell may be the next hop with significantly high probability.

# 3	AI/ML Mobility Optimization for CHO

In case of a CHO Handover, according to TS 38.423 a source NG-RAN node can send in Handover Request message a Conditional Handover Information Request IE that contains an Estimated Arrival Probability IE which can be used by the target NG-RAN node to allocate necessary resources. The Estimated Arrival Probability IE indicates a likelihood that the UE will end up accessing the target cell for which a handover request is sent. The algorithm used to calculate this probability is up to node’s implementation and could be based on internal legacy algorithms that could estimate such probability or done through AI/ML procedures. How this estimated arrival probability is calculated does not need to be indicated to a candidate target node.

Observation 4: The Estimated Arrival Probability IE in CHO can be calculated by legacy or by AI/ML methods.

When it comes to the confidence interval of the estimated arrival probability, once a probability is calculated for a given UE there is no more randomness and therefore the concept of confidence is not relevant. Therefore, indication of a confidence interval of an estimated arrival probability is not meaningful.

Observation 5: There is no need to indicate a confidence interval on the estimated arrival probability since it has no randomness.

A confidence interval would be meaningful to capture the randomness of the arrival process and the actual time when a UE arrives at a target node. If the arrival time at the target NG-RAN node can be predicted with a high confidence, this will allow further optimization of the resources at the target NG-RAN node. Currently there is no time indication as to when a UE is expected to arrive at the target node. It is possible that a UE has a very low confidence to arrive at a certain time interval (e.g., in the very near future), but a much higher confidence to arrive at another (e.g., a later time interval in the future).

Observation 6: If a target node knows information about the time interval during which different UEs are expected to arrive with high confidence, it can optimize its allocated resources.

For example, if the confidence of arrival at a future time interval [T1, T2] is very high, the target node could at time t< T1 accept a CHO handover even if it runs low in resources as long as it can expect that by the time that the UE will actually arrive at the target node it will have better resource availability.

Proposal 6: Introduce in Conditional Handover Request message a predicted time interval and corresponding confidence level during which a UE is expected to arrive.

Note that this proposal effectively provides a predicted resource reservation time window for CHO, which is one of the agreed outputs for AI/ML Mobility Enhancements in TR 37.817.

Handover execution timing information is not very clear in the context of normal handover. This is because handover happens immediately after Handover Conditions are met in which case the Handover timing is already determined.

Observation 7: There is no need to provide Handover Execution timing for normal Handover.

Also, for CHO Handover, indicating a handover priority is not necessary in the sense that it is unclear how different target nodes can be prioritized from the source perspective. Furthermore, it is also unclear how a target node can interpret different priority levels in its actions.

Observation 8: There is no need to provide a priority as part of an output of AI/ML Mobility Enhancements.

CHO is more robust in the sense that the risk that a UE loses coverage of the source cell is very small. However, this comes at a cost of reserving a much higher number of network resources which have to be reserved for a longer period of time than in normal Handover. This is because the CHO configuration is typically very early to achieve robustness. In addition, different target cells may be prepared, but handover will take place only in one of those cells. Therefore, robustness comes at the cost of a higher number of cell preparations which are longer than normal handover. Introduction of AI/ML in a NG-RAN node enables better resource optimization both at the source and target nodes.

Observation 9: AI/ML in the RAN can help to avoid too many and too long CHO cell preparations.

Another AI/ML Model output for AI/ML Mobility Optimization is to predict a handover target node as well as a set of candidate cells in CHO together with the confidence of the prediction. In our view, this will be an internally consumed output by the source node of the Handover. Such AI/ML Model output can further help the source node limit the number of CHO Handover initiations only to the candidate target cells where the handover is expected to happen with high confidence.

Proposal 7: Predicted handover target node and candidate cells in CHO are AI/ML Model Outputs consumed internally by the source node and have no standards impacts.

In the context of CHO, it would be also useful if the source NG-RAN node is able to cut down on the number of unnecessary CHO preparations (i.e., not even originate a Handover Request) to a potential target NG-RAN node if it knows in advance that the target NG-RAN node is not suitable to host the UE at this given point of time from the perspective of provided QoS for services established for the UE or a UE would end up with a potential failure when handing over to a target NG-RAN node. This information cannot be a static determination as the target NG-RAN nodes’ resource utilization/capacity is ever changing (including possible power saving actions etc.). One way to imagine this is to visualize the target NG-RAN nodes’ function as an estimator that can be invoked at the source NG-RAN node. The target NG-RAN nodes’ function can represent a list of values of achievable QoS indicators such as end user throughput, packet loss, rate, packet delay, etc., offered by target to source NG-RAN node or some indicators related to radio quality conditions that may point out on potential failure after handing over the UE to target NG-RAN node. By invoking this estimator and making a check with it, a source NG-RAN node would be able to potentially avert those preparations that are bound for failure. As such this requires transacting an estimator (as part of an AI/ML algorithm or otherwise) from the target NG-RAN node to the source NG-RAN node that can potentially allow the source NG-RAN node to perform this estimation. The estimator will of course go out of tune as the target NG-RAN node’s resource utilization/capacity changes over time, hence some kind of update is required back to the source NG-RAN node when that happens.

Proposal 8: To avoid unnecessary triggering of mobility procedures (e.g., CHO) towards a target NG-RAN node, it is desirable for the source NG-RAN node to be able to predict/foresee the performance (or failures) of a given UE at all possible candidate target NG-RAN nodes.

Using AI/ML, the source node can take into account in its cell preparations information about the impact of those in-vain preparations to different target NG-RAN nodes, which may not be the same across different nodes since different nodes may have different load situation, number of concurrent CHO preparations, proportion of in-vain preparations compared to successful, etc. Without any information measuring the impact of a CHO preparation for a specific target, a source NG-RAN node may only assume that the impact of in-vain preparations is the same across all candidate target NG-RAN nodes. To help source NG-RAN node predict the best candidate target gNBs for a CHO, feedback information is needed also from candidate target nodes that didn’t meet the execution condition.

Proposal 9: A configured candidate target NG-RAN node that didn’t meet the execution condition may send feedback to the source NG-RAN node to indicate the impact of the CHO preparation to the node’s performance e.g., with respect to a load related to CHO preparation, time duration of CHO configuration by this source compared to other sources, etc.

# 4 	Conclusion

In this paper we make the following observations and proposals:

Proposal 1: UE Trajectory information can be collected from a target NG-RAN node by configuring a measurement related to UE Trajectory information in the AI/ML INFORMATION REQUEST (name FFS) message.

Proposal 2: The source NG-RAN node triggers UE Trajectory information reporting by the target NG-RAN node by including the corresponding measurement ID in the HANDOVER REQUEST message.

Proposal 3: UE Trajectory information is sent by the target NG-RAN node to the source NG-RAN node in the AI/ML INFORMATION UPDATE (name FFS) for each UE being received at the target NG-RAN node for which a UE Trajectory information is requested during the Handover procedure.

Observation 1: Training a ML Model for Trajectory Prediction in the OAM is in the scope of Rel.18 use cases.

Proposal 4: For the solution where Model Training is in OAM, a NG-RAN node’s OAM may request the node for UE Trajectory information which allows different OAM systems to independently train models for cell-based UE Trajectory Prediction.

Observation 2: Forking points may exist in the predicted trajectory when a UE may visit 2 (or more) cells with the same probability.

Observation 3: The current encoding of the predicted UE Trajectory does not take into account the possibility that there may be forks in the trajectory prediction, when there can be more than one trajectory with similar or significantly high likelihood.

Proposal 5: Extend the current trajectory prediction to comprise a set of possible future trajectories, allowing the possibility of forking points where more than one cell may be the next hop with significantly high probability.

Observation 4: The Estimated Arrival Probability IE in CHO can be calculated by legacy or by AI/ML methods.

Observation 5: There is no need to indicate a confidence interval on the estimated arrival probability since it has no randomness.

Observation 6: If a target node knows information about the time interval during which different UEs are expected to arrive with high confidence, it can optimize its allocated resources.

Proposal 6: Introduce in Conditional Handover Request message a predicted time interval and corresponding confidence level during which a UE is expected to arrive.

Observation 7: There is no need to provide Handover Execution timing for normal Handover.

Observation 8: There is no need to provide a priority as part of an output of AI/ML Mobility Enhancements.

Observation 9: AI/ML in the RAN can help to avoid too many and too long CHO cell preparations.

Proposal 7: Predicted handover target node and candidate cells in CHO are AI/ML Model Outputs consumed internally by the source node and have no standards impacts.

Proposal 8: To avoid unnecessary triggering of mobility procedures (e.g., CHO) towards a target NG-RAN node, it is desirable for the source NG-RAN node to be able to predict/foresee the performance (or failures) of a given UE at all possible candidate target NG-RAN nodes.

Proposal 9: A configured candidate target NG-RAN node that didn’t meet the execution condition may send feedback to the source NG-RAN node to indicate the impact of the CHO preparation to the node’s performance e.g., with respect to a load related to CHO preparation, time duration of CHO configuration by this source compared to other sources, etc.