3GPP TSG-RAN WG3 Meeting #121	R3-234373

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	8.1

Source:	Huawei

Title:	Consideration on packet loss during multicast MBS delivery

Document for:	Discussion

# 1	Introduction

The incoming LS [1] addressing the packet loss during multicast MBS delivery, in this contribution, we analyse these aspects from RAN3 point of view and provide our proposals.

# 2	Discussion

In [1], the first part regards SA6’s concern of potential packet loss, from our view, in case the MB-UPF buffers the data for a sufficient long time period, before starting transmit multicast data over radio, the related UEs will be able to connect to the network and then there will be no potential packet loss. As SA2 already asked about the latency requirement of first packet(s) transmission for mission critical service, RAN3 can wait for SA6 reply on this.

Observation 1: for the mentioned packet loss issue raised by SA6, RAN3 can wait for SA6 feedback on the the latency requirement asked by SA2.

In the second part of the LS, SA2 pointed out that it is captured in TS 38.300: When there is temporarily no data to be sent to the UEs for a multicast session that is active, the gNB may move the UE to RRC_INACTIVE state. And they asked two questions to RAN2:

1. 	what are the conditions upon which gNB may move the UE involved in an active MBS Session to RRC_INACTIVE (as quoted in clause 16.10.5.2 of TS 38.300) and whether the QoS information (e.g. 5QI, ARP) of the multicast session is taken into account when deciding whether to move the UE to RRC_INACTIVE state or not.

2. 	what is the typical latency of first downlink packet(s) transmission if the UE is in RRC_INACTIVE state?

About the mentioned scenario (i.e. When there is temporarily no data to be sent to the UEs for a multicast session that is active, the gNB may move the UE to RRC_INACTIVE state), we would like to highlight about two RAN3 aspects:

1st aspect: the scenario is not supported in case E1 is deployed, because in case there is temporarily no data for an active multicast session, there is no E1AP procedure can be used by the gNB-CU-UP to inform the gNB-CU-CP, and also when there are DL packets arrive, the gNB-CU-UP cannot inform the gNB-CU-CP to trigger RAN paging as well.

2nd aspect: upon receiving new DL packets, the gNB triggers RAN Paging within the UE’s RNA, if the UE access from a new gNB, multicast data forwarding is not supported, packet loss may occur.

To avoid confusion for the scenario considering the 1st aspect, it is better for RAN3 to clarify the things in TS 38.300.

Observation 2: When there is temporarily no data to be sent to the UEs for a multicast session that is active, the gNB may move the UE to RRC_INACTIVE state”, this only applicable when E1 is not deployed.

Observation 3: multicast data forwarding is not supported once the inactive UE access to the network from a new gNB.

Proposal 1: clarify in TS 38.300 about observation 2.

Proposal 2: send reply LS to SA2/RAN2/SA6 to inform observation 2 and observation 3.

In the following, we focus on the scenario in which E1 is not deployed but F1 is.

For question 1 above, we think it is left to gNB implementation on how to make decision about moving a UE to RRC_INACTIVE, the QoS Information of the multicast session could be considered.

For question 2 above, after moving the UEs to RRC_INACTIVE state, assuming the MC bearer context and F1-U tunnels are kept, upon receiving the Multicast DL data packet(s), the gNB-CU needs to buffer the packet(s) and triggers RAN Paging to move the UEs to RRC_CONNECTED mode, besides the procedures in TS 38.300 section 9.2.2.4.2 and 9.2.2.4.1, we also need to consider the latency over F1 interface, i.e. the procedure in TS 38.401 section 8.6.2, that is to say, 6 times of F1AP message delievery latency need to be taken into account.

Proposal 3: send reply LS to SA2/RAN2/SA6 to provide RAN3’s inputs about the two questions from SA2.

# 3	Conclusion

In this contribution we analyse the mentioned aspects of SA2 incoming LS, get the following conclusion and proposals:

Observation 1: for the mentioned packet loss issue raised by SA6, RAN3 can wait for SA6 feedback on the the latency requirement asked by SA2.

Observation 2: When there is temporarily no data to be sent to the UEs for a multicast session that is active, the gNB may move the UE to RRC_INACTIVE state”, this only applicable when E1 is not deployed.

Observation 3: multicast data forwarding is not supported once the inactive UE access to the network from a new gNB.

Proposal 1: clarify in TS 38.300 about observation 2.

Proposal 2: send reply LS to SA2/RAN2/SA6 to inform observation 2 and observation 3.

Proposal 3: send reply LS to SA2/RAN2/SA6 to provide RAN3’s inputs about the two questions from SA2.

# 4	Reference

- R3-233728/S2-2307982 Reply to LS on addressing packet loss during multicast MBS delivery, Source: SA2, To: SA6, RAN2, Cc: RAN3.

- R3-23xxxx [Draft]Reply to LS on addressing packet loss during multicast MBS delivery, Source: Huawei (to be RAN3), To: SA2, SA6, RAN2

- R3-23xxxx Clarification on gNB moving UEs to RRC_Inactive for active multicast session, draftCR to TS38.300, Huawei