3GPP TSG-RAN WG3 Meeting #121	R3-234432

Toulouse , FR, 21–25 Aug, 2023

Agenda Item:	10.2.1

Source:	CMCC, Nokia, Nokia Shanghai Bell

Title:	Discussion on SON enhancement for Inter-RAT SHR

Document for:	Discussion and Decision

# 1 Introduction

In RAN3#120, the following issue was captured for Inter-RAT SHR and need to be continued in next meeting:

1. Correlation of inter-RAT SHR with RLF Report

Option 1: Support the correlation so that the network can discard SHR if it knows that there was RLF shortly after successful HO.

Option 1-1:  the source gNB performs the correlation based on target C-RNTI (no additional reporting from the UE is needed).

- UE may/may not reports a correlation indication to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE

Option 1-2:  the source gNB performs the correlation based on the source C-RNTI and time information between HO command and SHR retrieval

Option 1-3: UE assistance-based option to support the correlation indication for SHR and RLF based on new flag reported within the SHR

Option 2: Postpone correlation of inter-RAT SHR and RLF to Rel-19.

Option 3: Do not support SHR and RLF Report correlation

In this contribution, we would further discuss on correlation mechanism of SHR and RLF Report and provide some proposals on the issue of inter-RAT SHR.

# 2 Discussion

During the past few RAN3 meetings, there were discussions on whether and how to correlate SHR and RLF Report in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE and no conclusion achieved.

In our opinion, inappropriate configuration of inter-RAT handover in source NR node may cause RLF in target LTE node shortly after the successful handover. It is beneficial to correlate SHR and RLF Report in this case for the optimization in source node to avoid this kind of failure.

Observation 1: It is beneficial to correlate SHR and RLF Report in the case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE for the optimization in source node.

Based on the initial analysis, we can see drawbacks for the potential solution that the source gNB performs the correlation without correlation information from UE. For example, although the source NR node could receive Inter-RAT SHR and LTE RLF report respectively, but the time between the two reports received maybe a long time. The source node supporting correlation of SHR and RLF Report has to save one report for a long time to wait for the other one, since it has no information on the present of the report it waiting for. This solution would introduce extra burden and storage requirement to the source node. We think it is the main reason why some companies object to support SHR and RLF Report correlation.

We have the same opinion that there is no need to introduce extra burden and complexity to support the correlation mechanism of SHR and RLF Report. So we think it is a more feasible and efficient solution that UE provides the correlation indication for SHR and RLF. For the case (in Fig.1) there is a RLF shortly after a successful inter-RAT HO from NR to LTE, UE generates both NR SHR and LTE RLF report and the UE could know for sure whether there is a RLF after a successful inter-RAT HO. It is more easy for UE to provide the correlation indication of SHR and RLF Report.

Observation 2: For the case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, UE generates both NR SHR and LTE RLF report. And the UE could know for sure if there is a RLF after a successful inter-RAT HO. It is a more feasible and efficient solution that UE provides the correlation indication for SHR and RLF.

In UE assistance-based option, a correlation indication (e.g. 1 bit flag) of SHR and RLF report is introduce in SHR to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE. UE sets the correlation indication to TURE if the time between the successful inter-RAT HO from NR to LTE completed and the subsequent RLF occurred in target LTE cell is less than the time value configured by network or defined in spec. Based on the correlation indication from UE, how the network (e.g. the source NR node) to perform the correlation, root cause analysis and optimization could depend on gNB implementation.

Regarding the gNB implementation of correlation analysis of SHR and RLF report, there are two main solutions discussed in the past few RAN3 meetings as shown below:

- Opt1. The source gNB performs the correlation and analysis based on target C-RNTI or the source C-RNTI and time information between HO command and SHR retrieval.

- Opt2. The source gNB collects the number of HOs with SHR linked to RLF report [2]. An MRO KPI given as ratio of the measurement to total number of successful HOs for the same source NR and target LTE relationship can be defined. Having the KPI above the given threshold may be then the trigger condition to prioritize for focusing on fixing the HO to wrong cell and not the initial near failures that triggered the generation of the SHR.

For Opt1, the correlation indication from UE enable the network to know there are NR SHR and LTE RLF report generated by a specific UE. The source NR node just need to store one report for the specific UE and no need to store report for all UE for a long time. Therefore, this option based on the correlation indication from UE would not introduce high burden and storage requirement for the source node and target LTE node. But the target C-RNTI or the source C-RNTI need to be included in SHR.

For Opt2, the source gNB can directly use the SHR with new binary flag set to TRUE for optimization. And there is no other information need to be introduced in SHR.

Based on the above analysis, we can see that regardless of Opt1 and Opt2, the correlation indication from UE can make the gNB implementation of analysis and optimization more feasible and more flexible.

Observation 3: The correlation indication of SHR and RLF report from UE can make the gNB implementation of analysis and optimization more feasible and more flexible.

Proposal 1: Agree UE assistance-based option to support the correlation indication for SHR and RLF based on new flag reported within the SHR as the baseline for correlation mechanism of SHR and RLF Report.

Proposal 2: To introduce 1 bit flag as the correlation indication of SHR and RLF report in SHR to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

Proposal 3: Based on the correlation indication from UE, how the network (e.g. the source NR node) to perform the correlation, root cause analysis and optimization could depend on gNB implementation.

UE

Source NR node

Target LTE node

Receiving node(NR)

UE stores NR SHR

UE stores RLF report

NR SHR report (correlation indication)

LTE RLF report container

RLF report container

NR SHR container (correlation indication)

LTE RLF report

Receiving node(LTE)

RLF shortly after a successful inter-RAT HO

Successful Inter-RAT HO (SHR trigger condition met)

Figure 1

# 3 Conclusion

In this paper, we discuss correlation mechanism of NR SHR and LTE RLF Report, and we have the following observations and proposals:

Observation 1: It is beneficial to correlate SHR and RLF Report in the case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE for the optimization in source node.

Observation 2: For the case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, UE generates both NR SHR and LTE RLF report. And the UE could know for sure if there is a RLF after a successful inter-RAT HO. It is a more feasible and efficient solution that UE provides the correlation indication for SHR and RLF.

Observation 3: The correlation indication of SHR and RLF report from UE can make the gNB implementation of analysis and optimization more feasible and more flexible.

Proposal 1: Agree UE assistance-based option to support the correlation indication for SHR and RLF based on new flag reported within the SHR as the baseline for correlation mechanism of SHR and RLF Report.

Proposal 2: To introduce 1 bit flag as the correlation indication of SHR and RLF report in SHR to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

Proposal 3: Based on the correlation indication from UE, how the network (e.g. the source NR node) to perform the correlation, root cause analysis and optimization could depend on gNB implementation.

# Reference

- RAN3_120_agenda_20230530_EOM1.

- R3-232946, SHR and Configuration coordination for the successful PSCell change report, Nokia, Nokia Shanghai Bell, May 2023.