3GPP TSG-RAN WG3 Meeting #121	R3-234434

Toulouse , FR, 21–25 Aug, 2023

Agenda Item:	10.2.2

Source:	CMCC

Title:	Discussion on MRO for CPAC

Document for:	Discussion and Decision

# 1 Introduction

In RAN3#120, the following issue was captured for MRO for CPAC and need to be continued in next meeting:

Whether CPA/CPC candidate PSCell list can be sent from MN to the source/last serving SN?

In this contribution, we would discuss on the wrong candidate PSCell selection in CPA/CPC procedure and provide some proposals on the above issue of MRO for CPAC.

# 2 Discussion

There are the following scenarios considered for MRO for CPAC in Rel-18.

- CPA Execution to wrong PSCell

- Too Early CPA Execution

- Too Late CPC Execution

- Too Early CPC Execution

- CPC Execution to wrong PSCell

RAN3#119 has achieved the agreement about the definition to wrong PSCell change/addition for CPA/CPC Execution.

Update the definition to wrong PSCell change/addition should be splitting to sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the target node.

The definition of CPA/CPC Execution to wrong PSCell is that UE receives CPA/CPC configuration and CPA/CPC execution condition is satisfied, CPA/CPC execution fails or an SCG failure occurs shortly after a successful CPA/CPC execution; a suitable PSCell different with source PSCell (only for CPC) or target PSCell is found based on the measurements reported from the UE. And these two scenarios should be splitting to sub cases:

- The suitable PSCell is one of the proposed PSCell candidates recommended by the initiating node (MN or source SN), but the candidate PSCell was not accepted by the target node, that means it is wrong candidate PSCell selection at the target node.

- The suitable PSCell is not one of the proposed PSCell candidates recommended by the initiating node (MN or source SN), that means it is wrong candidate PSCell recommend at the initiating node (MN or source SN).

For sub-case 1), if it is CPA or MN initiated CPC, MN performs the initial analysis and inform the possible problem node (the target node) of the wrong candidate PSCell selection, the target node performs the root cause analysis; if it is SN initiated CPC,  source SN performs the initial analysis and inform MN of the wrong candidate PSCell selection at the target node, the target node performs the root cause analysis.

For sub-case 2), if it is CPA or MN initiated CPC, MN performs the root cause analysis; if it is SN initiated CPC, MN  informs the possible problem node (the source node) of the wrong candidate PSCell recommend at the initiating node, and source SN performs the root cause analysis.

Observation 1: For CPA/CPC Execution to wrong PSCell, the initiating node (MN/source SN) performs the initial analysis to identify the possible problem node (MN/source SN/target SN) and the possible problem node performs the root cause analysis.

RAN3#119 has agreed the signalling mechanism of CPA/CPC failure/ related information reporting as shown below.

Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN.

Therefore, after CPAC procedure failure occurs and MN receives SCGFailureInformation from UE, the SCG FAILURE INFORMATION REPORT message can be used by MN to inform the CPAC failure related information to the possible problem node (source SN or target SN). And the SCG FAILURE TRANSFER message can be used by source SN to inform MN of the wrong candidate PSCell selection at the target node after source SN performs the initial analysis for SN initiated CPC.

Proposal 1: The SCG FAILURE INFORMATION REPORT message is used by MN to inform the CPAC failure related information to the possible problem node (source SN or target SN).

Proposal 2: The SCG FAILURE TRANSFER message is used by source SN to inform MN of the wrong candidate PSCell selection at the target node after source SN performs the initial analysis for SN initiated CPC.

Regarding SCG FAILURE INFORMATION REPORT enhancement, CPA/CPC candidate PSCell list need to be included from MN to source SN or target SN for the root cause analysis. Since an SCG failure occurs shortly after a successful CPC execution, the source SN may have release the CPC related configuration of the UE, so the  CPA/CPC candidate PSCell list need to be sent from MN.

Proposal 3: CPA/CPC candidate PSCell list need to be included in SCG FAILURE INFORMATION REPORT message from MN to source SN or target SN for the root cause analysis.

# 3 Conclusion

In this paper, we discuss on the wrong candidate PSCell selection in CPA/CPC procedure for MRO for CPAC, and we have the following observation and proposals:

Observation 1: For CPA/CPC Execution to wrong PSCell, the initiating node (MN/source SN) performs the initial analysis to identify the possible problem node (MN/source SN/target SN) and the possible problem node performs the root cause analysis.

Proposal 1: The SCG FAILURE INFORMATION REPORT message is used by MN to inform the CPAC failure related information to the possible problem node (source SN or target SN).

Proposal 2: The SCG FAILURE TRANSFER message is used by source SN to inform MN of the wrong candidate PSCell selection at the target node after source SN performs the initial analysis for SN initiated CPC.

Proposal 3: CPA/CPC candidate PSCell list need to be included in SCG FAILURE INFORMATION REPORT message from MN to source SN or target SN for the root cause analysis.

# Reference

- RAN3_120_agenda_20230530_EOM1.