# 3GPP TSG-RAN WG3 Meeting #121	R3-234440

# Toulouse, France, Aug 21st - Aug 25th, 2023

# Agenda item:		12.2.2.1

# Source:				CMCC

# Title:	Discussion on AI/ML remaining issues

# Document for:		Discussion

Introduction

During the last RAN3 #120 meeting, the following agreements and FFS were captured:

Node level and cell level UE performance collection is not supported in R18.

The Source NG-RAN node UE XnAP ID reference IE within the UE Associated Info Result Item IE should be included in the update message.

The Failed Report Characteristics shall be split into two fields:

- Failed Report Characteristics for per cell measurements, reported with per cell granularity

- Failed Report Characteristics for per node measurements, reported with per node granularity

Introduce a Cause IE for the Measurement Failed Report Characteristics per cell and Measurement Failed Report Characteristics per node.

The group discussed and had the common understanding as below:

Cause value measurement not supported -> is covered by legacy cause “Measurement not Supported For The Object,”

Cause value measurement temporarily not available -> is covered by legacy cause “Measurement Temporarily not Available,”

It is FFS whether any more cause values should be added to the legacy list of causes.

There is consensus on the benefits of enabling the requesting node to optimize the measurement request by indicating whether requested measurements can be reported in full or in part. It needs to be further discussed whether such enhancements can be introduced in Rel18.

It needs to be further discussed whether and how any further granularity needs to be introduced for the selection of measurements to be reported as part of the UE Performance Feedback.

The problem of how to signal to the reporting node time configurations for the UE Performance Feedback measurement reporting is acknowledged.

Time configuration for UE Performance Feedback measurement reporting consists at least of:

- Time duration of the UE Performance Feedback measurement collection (the time duration starting at handover execution and including the last UE Performance Feedback report)

- Whether, within such time duration, measurements are reported periodically or one time

- In case of periodic reporting, the period of UE Performance Feedback measurements (whether existing of new IE)

Work on stage3 details

In this contribution, we provide our view on the FFS.

Discussion

Partial reporting

In RAN3#120 meeting, RAN3 reached consensus on the benefits of enabling the requesting node to optimize the measurement request by indicating whether requested measurements can be reported in full or in part, and whether such enhancement is needed should be further discussed. As RAN3 already agreed to introduce the failed measurement in the response message to indicate which requesting measurements cannot be provided in RAN3#119bis, the requesting node can know whether the response message meet it’s requirement with this information. If the requesting node must require all the requested measurements, but the response message includes failed measurements list, the requesting node can stop receiving the requested measurements and inform the requested node that it no longer needs to provide the requested measurements. Therefore, we think it’s no need to introduce such enhancement in the AI/ML INFORMATION REQUEST message to inform the requested node whether the requested measurements can be reported in full or in part.

Proposal 1: No need to introduce such enhancement in the AI/ML INFORMATION REQUEST message to inform the requested node whether the requested measurements can be reported in full or in part.

Based on the discussion in the last meeting, RAN3 has agreed the legacy cause value “Measurement not Supported For The Object” and “Measurement Temporarily not Available” can be reused for the failure causes in the response message. And FFS on whether any more cause values should be added to the legacy list of causes. From our understanding, the cause value is used for the requested node to inform the requesting node why the request information can not be provided. The legacy cause value “Measurement not Supported For The Object” and “Measurement Temporarily not Available” can separately represents the requested node not support the request measurements and the request measurement not available temporarily, these two types cover most possible scenarios and it’s enough in the current stage.

Proposal 2: No need to introduce more cause values to the failure causes in the current stage.

Besides, for the legacy cause values “Measurement Temporarily not Available”, propose to introduce time information to specify when the temporarily not available measurement can be available. The time information may provide the reference to the requesting node about when the requesting node can request these kind of failed measurements again. Introduce time information can reduce the signaling overhead caused by the frequent requests from the requesting node for the information that is not available temporarily.

Proposal 3: Introduce the time information with the failure cause value “Measurement Temporarily not Available” in response message to inform the requesting node when to request the failed measurements at the next time.

UE performance feedback

Depend on the discussion in RAN3#120, whether to enable the flexibility for UE performance feedback after handover has been discussed. And leaving FFS about whether and how any further granularity needs to be introduced for the selection of measurements to be reported as part of the UE Performance Feedback.

In the previous meetings, RAN3 has discussed to use new class 1 procedure to configure UE performance feedback metrics per node to enable the requested node has sufficient time to prepare, and include the Measurement ID pair in the HO request message to establish relationship with the UE performance feedback configuration in AI/ML INFORMATION REQUEST message. However, these configurations are per node configurations and cannot be differentiated for specific UE. If only part of the UE performance feedback metrics are needed for some of the handover UEs, the requested node may not know which UE performance feedback metrics the specific UEs required to feedback. So, the UE performance feedback for specific UE(s) suggests to be supported.

When using AI/ML INFORMATION REQUEST message to configure UE performance feedback, the required UE performance feedback for all the potential handover UEs are considered with the same configuration. To enable the configuration can be differentiated for specific UE (if needed), especially after the AI/ML INFORMATION REQUEST message is sent and the handover UE was not previously considered as a handover UE (e.g. just moving from another gNB), in order to ensure the previous configuration can be used for this specific UE, we propose to use AI/ML INFORMATION REQUEST message to configure UE performance feedback per node and introduce a new IE in HO request message to indicate which configured UE performance is required for the handover UE.

Proposal 4: Introduce UE feedback IE in HO request message to indicate which configured UE performance is required for the handover UE.

Given that many UE performances may required for the handover UE, the UE feedback IE should be a list or a bitmap to specify the UE performances that need to report.

Proposal 5: The UE feedback IE is configured as a list or bitmap to specify the UE performance that need to report.

Furthermore, RAN3 also discussed about the reporting duration for UE performance feedback and including the time configuration for UE performance feedback measurement reporting for both periodically or one time reporting. From our understanding, since stage 2 is discussing the configuration of time information, we propose the more detailed time configuration for UE performance feedback can wait for the progress of stage 2 discussion.

Proposal 6: Since stage 2 is discussing the configuration of time information, propose the more detailed time configuration for UE performance feedback can wait for the progress of stage 2 discussion.

Besides, for the UE performance feedback, we think that the requested node should be able to report the UE performance feedback for either one handover UE or a list of handover UEs, and also be able to report either one-time or periodically. And these could be configured through the AI/ML INFORMATION REQUEST message with the time duration and reporting periodicity. If requesting node is configured UE performance feedback with no time duration and no reporting periodicity, it’s more likely that the requested node feedback the UE performance for each UE after HO completion. If the requesting node is configured UE performance feedback with time duration and no reporting periodicity, it’s more likely that the requested node feedback the UE performance for a list of UEs within the configured time duration. If the requesting node is configured UE performance feedback with reporting periodicity and no time duration, it’s more likely that the requested node feedback the UE performance for each UE after HO completion periodically. If the requesting node is configured UE performance feedback with reporting periodicity and time duration, it’s more likely that the requested node feedback the UE performance for a list of handover UE(s) (if available) periodically. So, we think that the requested node should support various kinds of UE performance feedback reporting, and which kinds of reporting the requested node feedback the UE performance depends on the configuration.

Proposal 7: RAN3 agrees that the requested node should be able to report the UE performance feedback for either one handover UE or a list of handover UEs, and also be able to report either one-time or periodically.

Prediction accuracy

The issue of prediction accuracy was also discussed in the previous meeting. By providing the prediction accuracy to the requesting NG-RAN node, the requesting node could have an understanding of “how good” these predictions are and make use of the prediction result in different ways, e.g., for low-accuracy, the requesting node may just regard it as reference information, and for high-accuracy, the node may take it into consideration when make the decision. Also, the requesting node may stop requesting the prediction information if the prediction accuracy is unacceptable since sometimes an inaccurate prediction could be even worse than no prediction.

As for the procedure, the requested node could provide the accuracy information when requested by the requesting node.

Proposal 8: Prediction accuracy could be provided to the requesting node when requested.

Conclusion

In this contribution, we discussed the new procedure used for AI/ML information.

We propose the following proposals:

Proposal 1: No need to introduce such enhancement in the AI/ML INFORMATION REQUEST message to inform the requested node whether the requested measurements can be reported in full or in part.

Proposal 2: No need to introduce more cause values to the failure causes in the current stage.

Proposal 3: Introduce the time information with the failure cause value “Measurement Temporarily not Available” in response message to inform the requesting node when to request the failed measurements at the next time.

Proposal 4: Introduce UE feedback IE in HO request message to indicate which configured UE performance is required for the handover UE.

Proposal 5: The UE feedback IE is configured as a list or bitmap to specify the UE performance that need to report.

Proposal 6: Since stage 2 is discussing the configuration of time information, propose the more detailed time configuration for UE performance feedback can wait for the progress of stage 2 discussion.

Proposal 7: RAN3 agrees that the requested node should be able to report the UE performance feedback for either one handover UE or a list of handover UEs, and also be able to report either one-time or periodically.

Proposal 8: Prediction accuracy could be provided to the requesting node when requested.

- References

[1] TR 37.817, Study on enhancement for Data Collection for NR and EN-DC

[2] TR 38.423, Xn Application Protocol (XnAP)

- Annex: TP to 38.423

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

<Skip Unchanged>

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Cell Based UE Trajectory Prediction IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. mobility decisions.

If the UE feedback IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, send the indicated UE performance with the AI/ML Measurement ID to the source NG-RAN node after handover.

<Skip Unchanged>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| <Skip Unchanged> |  |  |  |  |  |  |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x |  | YES | ignore |
| AI/ML Measurement ID (FFS on the name) | O |  | 9.2.3.M |  | YES | ignore |
| UE feedback | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to feedback for the handover UE. First Bit = Average UE Throughput DL, Second Bit = Average UE Throughput UL, Third Bit = Average Packet Delay, Fourth Bit = Average Packet Loss | YES | ignore |