3GPP TSG-RAN WG3 Meeting #121          	          R3-234446

Toulouse, FR, 21 Aug – 25 Aug 2023

Agenda item:	14.2

Source:	CMCC

Title:	(TP to TS 38.401) L1/L2 based Inter-Cell Mobility

Document for:	Approval

1	Introduction

This contribution provides the TP to TS 38.401 for L1/L2 based Inter-Cell Mobility based on the discussion paper.

- Text Proposal to TS 38.401 BLCR

--------------------------------------------------------------Start of the change---------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID.

- If the gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID.

- FFS: How the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU may send the UE CONTEXT MODIFICATION message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

--------------------------------------------------------------Next change -----------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID, requesting TCI state, RACH resources and RS configuration.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration, TCI state, RACH resources and RS configuration for the accepted target candidate cell.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU, containing all candidate cells RS configurations in each candidate DUs, TCI states and RACH configurations.

- The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU, containing all cells RS configurations, serving cell LTM configuration and RS report configuration for serving cell.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the Candidate gNB-DU(s) containing all cells RS configurations, requesting RS report configuration for candidate cells in each DU.

- The Candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message, containing RS report configuration for candidate cells in each DU.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to trigger early TA acquisition.

- The source gNB-DU sends a PDCCH order to UE.

- The UE performs random access to the candidate gNB-DU.

- The candidate gNB-DU transfers TA value to the gNB-CU.

- The gNB-CU transfers TA value to the source gNB-DU.

- The source gNB-DU decides to execute LTM to a candidate target cell.

- The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID.

- FFS:  how the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

- The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

25. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

--------------------------------------------------------------End of the change----------------------------------------------------------