3GPP TSG-RAN WG3 #121                                                                         R3-234519

Toulouse, France, 21st-25th August 2023

Agenda Item:	9.2

Source:	CATT (moderator)

Title:	                Summary of Offline Discussion on CB: # 6_MRBinSplitCase

Document for:	Approval

# Introduction

CB: # 6_MRBinSplitCase - Check the proposals and solutions - Capture agreements if any (moderator - CATT) Summary of offline disc R3-234519

Please provide the comments before XX, XX, XX.

# For the Chairman’s Notes

<TBD>

# Discussion

## Core problem

The core problem is that, for the case that source MRB progress is not provided, the target gNB-CU(-CP) still needs to get aware of the association between a series of UE-associated messages and an F1AP Multicast Distribution Setup procedure for forwarded data, so that it can provide each data forwarding address (at step 13) toward the corresponding source RAN node respectively.

Considering that the gNB-DU is naturally aware of such association, the issue is on how the gNB-DU indicate such association toward the gNB-CU(-CP).

Two documents provided two flavours of solution:

- Option 1: Adding the “Multicast F1-U Context Reference F1” sent in Step 9 into Step 8.

- Option 2: Adding the “Multicast F1-U Context Reference CU” received in Step 12 into Step 8. (This implies that Step 8 must be sent after Step 12 if the latter one exists, changing the signalling flow in TS 38.401.)

The moderator wants to remind that there are also other potential solutions, e.g.

- Option 3: Adding the “Multicast F1-U Context Reference CU” sent in Step 12 into Step 7.

Q1: Do you acknowledge this problem? If agree, which option do you prefer.

| Companies | Option | Comments |
| --- | --- | --- |
| CATT | 1 (or 3) | Option 2 is not preferred as it causes extra handover delay than Option 1 (and 3) for the case. |

## Miscellaneous

R3-234243 proposed that (for step 7 above):

Proposal 1: To introduce a new IE into the “UE Multicast MRB to Be Setup Item IEs” structure of TS 38.473 for a gNB-CU-CP unaware of the MRB progress at source to indicate that a PTP tunnel needs to be established for forwarded packets of an MRB.

Q2: Do you agree with Proposal 1 in R3-234243?

| Companies | Option | Comments |
| --- | --- | --- |
| CATT | Prefer yes | It is the clearest way. |

R3-234243 also proposed that (for F1-U descriptor):

Proposal 4: Considering the F1AP Multicast Distribution Setup Request message, to revise the semantic description of the “ptp forwarding” code point of the “MC F1-U Context usage” IE and the description of the presence condition of the “MRB progress information” within the “Multicast F1-U Context To Be Setup Item” structure, so that the “ptp forwarding” code point can be used for the case that the source MRB progress is not provided. Proposal 5: Considering the E1AP “MC F1-U Context usage” structure, to revise the semantic description of the “ptp forwarding” code point, so that the “ptp forwarding” code point can be used for the case that the source MRB progress is not provided.

Q3: Do you agree with Proposal 4 and 5 in R3-234243?

| Companies | Option | Comments |
| --- | --- | --- |
| CATT | Prefer yes | Clearest way. And for the presence condition in F1AP, we are open on whether to change it to “O” or to change the condition description. |

R3-233850 proposed that (for future handover):

Proposal 1: agree the CR in [5] moving the Multicast F1-U Context Reference CU IE in the MRB Setup List IE of the UE context modification response message from optional to mandatory.

Q2: Do you agree with Proposal 1 in R3-233850?

| Companies | Option | Comments |
| --- | --- | --- |

# References

[1] R3-234243; Data forwarding handling of MRB in split target gNB; CATT, China Telecom, CBN.

[2] R3-233850; Correction of the F1-U context reference; Nokia, Nokia Shanghai Bell, Orange, Qualcomm Incorporated, Huawei.