3GPP TSG-RAN WG3 Meeting #121	R3-234613

- Toulouse, France, 21st – 25th August 2023

Agenda Item:	25.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # R18XR

Document for:	Approval

# Introduction

CB: # R18XR

- Capture the agreements to TP

- Discuss the open issues as above

- Capture agreements and open issues

(moderator - Nok)

Summary of offline disc R3-234613

It is proposed to complete the discussion by 21:59 Aug 24 (Thursday)

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## PDU Set QoS parameters in NGAP/XnAP/F1AP/E1AP

RAN3 agreed to enhance NGAP/XnAP/F1AP/E1AP to support following agreements

Add the PDU Set QoS Parameters in the QoS Flow Level QoS Parameters IE over NGAP, XnAP, F1AP and E1AP.

The PDU Set QoS parameters includes:

- PDU Set Delay Budget (PSDB).

- PDU Set Error Rate (PSER).

- PDU Set Integrated Handling Information (PSIHI).

Moderator suggest the new PDU Set QoS parameters will be added as a new IE in the QoS Flow Level QoS Parameters IE. The IE name can be FFS.

Q1: Please share your view on: Add a new XR IE (IE name FFS) in NGAP/XnAP/F1AP/E1AP QoS Flow Level QoS Parameters IE. The IE contains PSDB, PSER and PSIHI.

| Company | Comment |
| --- | --- |
| Nokia | Agree |

Summary

Potential proposals:

## TSCAI in NGAP/XnAP/F1AP/E1AP

RAN3 agreed

Enhance NGAP TSCAI to include the N6 Jitter information associated with DL Periodicity, and clarify TSCAI can also be used for non-GBR.

Enhance XnAP/F1AP/E1AP TSCAI to include the N6 Jitter information associated with DL Periodicity.

Same TSC Traffic Characteristics IE is defined in NGAP/XnAP/F1AP/E1AP. Only NGAP has the limitation that it is only for GBR.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

XnAP:

| >>>>TSC Traffic Characteristics | O |  | 9.2.3.114 |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

F1AP:

| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

E1AP:

| >>TSC Traffic Characteristics | O |  | 9.3.1.75 | Traffic pattern information associated with the QFI. Details in TS 23.501 [20]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

To support the new requirement that TSCAI can also be used for non-GBR, Moderator suggest to delete the limitation in NGAP Semantics Description, and use same semantics description to align with F1AP/E1AP. Other option is to delete the semantics description as XnAP. Moderator think it may be better to align with the majority 😊 F1AP/E1AP.

Q2: Please share your view on following

- 1: In NGAP, change semantics description as F1AP/E1AP, i.e.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | Traffic pattern information associated with the QFI. Details in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

- 2: In NGAP/XnAP/F1AP/E1AP, Add N6 Jitter information associated with DL Periodicity, e.g. use NGAP as an example:

#### 9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  | - |  |
| Survival Time | O |  | 9.3.1.221 |  | YES | ignore |
| N6 Jitter | O |  | FFS | Only applicable to downlink | YES | ignore |

| Company | Comment |
| --- | --- |
| Nokia | Agree with both |

Summary

Potential proposals:

## Container for PDU Set information

RAN3 agreed

PDU Set Information as below needs to be transferred over NG-U, Xn-U and F1-U:

  - PDU Set Sequence Number.

  - Indication of End PDU of the PDU Set.

  - PDU Sequence Number within a PDU Set.

  - PDU Set Size in bytes.

  - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

End of Data Burst information needs to be transferred over NG-U, Xn-U and F1-U.

Whether to create a new “PDU Set Container” or reusing “PDU Session Container"?

There are two options to be discussed:

- Option 1: extend existing container

- Option 2: introduce a new container

Q3: Please share your view on above two options, and anything can be captured in 38.415/425

| Company | Comment |
| --- | --- |
| Nokia | We prefer to have a further check with developers, and continue the discussion in next meeting. |

Summary

Potential proposals:

## Open issues to be discussed in next meeting

Due to the limited time for the online/offline discussion, Moderator suggest to first agree the list of open issues and potential options, so companies can prepare contribution for next meeting:

- Issue 1: whether NG-RAN provide an indication to SMF on whether a QoS Flow is successfully established with PDU Set QoS, during the QoS Flow establishment procedure.

- Issue 2: whether use RACS-like method to detect the peer node support PDU Set QoS handling.

- Issue 3: how to support the mobility from non-PDU Set capable source NG-RAN to a PDU Set capable target NG-RAN, and how to transfer the PDU Set QoS to target NG-RAN.

- Issue 4: how to support ECN marking in the NG-RAN

For example, whether adding a ECN marking Request during QoS flow establishment, providing a feedback when a QoS flow is setup with ECN marking accepted, DU providing congestion information to CU, etc.

- Issue 5: How to support ECN marking in UPF and network information exposure

For example, whether adding a congestion monitoring request during QoS flow establishment, providing a feedback when a QoS flow is setup with congestion monitoring accepted, DU provide congestion information CU, CU provide congestion information to SMF, etc.

- Issue 6: How to support PDU Set discard

- Issue 7:

Q4: Please share your view on above issue, and add any missing issues

| Company | Comment |
| --- | --- |

Summary

Potential proposals:

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-233731, LS reply on TSCAI for XR (SA2(VIVO))

- R3-233732, Reply LS on the N6 PDU Set Identification (SA2(OPPO))

- R3-233733, Reply LS on RAN information exposure for XRM (SA2(China mobile))

- R3-233734, LS Reply on Design of RTP Header Extension for PDU Set handling (SA2(Huawei))

- R3-233735, Non-homogeneous deployment of PDU Set based handling (SA2(Qualcomm))

- R3-233950, Discussion on the support of enhancement for NR XR (Samsung)

- R3-233951, [TP for BLCR TS37.483] Introducing enhancement for NR XR (Samsung)

- R3-233952, [TP for BLCR TS38.413, TS38.423] Introducing enhancement for NR XR mobility (Samsung)

- R3-233981, TP for Stage-2 TS 38.300 for XR Enhancements (Qualcomm Incorporated)

- R3-234019, Discussion on support for NR XR  (Nokia, Nokia Shanghai Bell)

- R3-234020, (TP for TS38.413) Support for NR XR	 (Nokia, Nokia Shanghai Bell)

- R3-234021, Discussion on multiple LSs  (Nokia, Nokia Shanghai Bell)

- R3-234119, Discussion on Support Enhancements on NR XR (CATT)

- R3-234120, Discussion on XR LSin from SA2 (CATT)

- R3-234121, (TP for XR for 38.413, 37.483 and 38.473) Support Enhancements on XR awareness (CATT)

- R3-234153, Discussion on discard operation for XR (Huawei, Qualcomm Inc.)

- R3-234154, (TP for NR_XR_enh BL CRs) Discussion on the enhancement for XR awareness (Huawei, Qualcomm Inc.)

- R3-234155, (TP for NR_XR_enh BL CR of TS 38.413) Discussion on incoming LS  from other WGs (Huawei, Qualcomm Inc.)

- R3-234162, Discussion on PDU Set QoS Parameters and PDU Set Information (Lenovo)

- R3-234163, Discussion on PDU Set based Discard (Lenovo)

- R3-234164, Discussion on ECN marking for L4S (Lenovo)

- R3-234257, Discussion on enhancement for XR (Xiaomi)

- R3-234258, (TPs for TS 38.300 and TS 38.413) Non-homogenous support of PDU set based handling (Xiaomi)

- R3-234275, Discussion on XRM support (Ericsson)

- R3-234276, Introduction of XRM enhancement (Ericsson)

- R3-234302, Support for L4S in NG-RAN (Ericsson, Deutsche Telekom)

- R3-234406, Disucssion on Support of XR Enhancement in Split Architecture (China Telecom)

- R3-234408, (BLCR to 38.401) Introduction of XR enhancements (China Telecom)

- R3-234452, Discussion on non-homogeneous deployment of PDU Set based handling (CMCC)

- R3-234482, (TP to TS 38.413) Discussion on support of XR enhancements (ZTE)

- R3-234483, (XR BL CR to TS 38.415) Introduction of XR enhancements (ZTE)

- R3-234484, (TP to TS 38.423) Support of XR enhancements (ZTE)