3GPP TSG-RAN WG3 Meeting #121	R3-23XXXX

21th – 25th Aug 2023, Toulouse,FR

Agenda item:	10.2.3

Source: 	ZTE

Title: 	(TPs for SON BLCRs for TS 38.473 ): RACH enhancements

Document for:   Discussion

# Introduction

- This contribution provides TP fot TS38.473.

# Discussion

# TP for TS 38.473

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.x	RACH INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about one or more random access procedures performed at the gNB-DU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RA Report Indication List |  | 1 |  |  | YES | reject |
| >RA Report Indication List Item |  | 1..<maxnoofRAReport Indications> |  |  | - | - |
| >>RA Report Retrieval UE IDgNB-CU UE F1AP ID | M |  | gNB-CU UE F1AP ID 9.3.1.4 | Indicate the UE ID need to retrieve RA Report. | - |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

RAReportIndicationList ::= SEQUENCE (SIZE(1..maxnoofRAReportIndications)) OF RAReportIndicationList-Item

RAReportIndicationList-Item ::= SEQUENCE {

RA-Report-Retrieval-UE-ID					GNB-CU-UE-F1AP-ID,

iE-Extensions						ProtocolExtensionContainer { { RAReportIndicationList-Item-ExtIEs} } OPTIONAL,

...

}

RAReportIndicationList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< end of the Change >>>>>>>>>>>>>>>>>>>>