3GPP TSG-RAN WG3 Meeting #122                                                              R3-237281

Chicago, IL, USA, November 13 – 17, 2023

Agenda item: 		13.5

Source: 		Qualcomm Incorporated

Title: 	Discussion on Reply to LS from RAN2 on RACH-less HO

Document for:	Discussion

# 1	Introduction

This document discusses the reply to RAN2 LS on UE RACH-less handover for mobile IAB in R3-233713/R2-2306817.

# 2	Discussion

This LS by RAN2 in R3-233713/R2-2306817 has the following content:

1. Overall Description: RAN2 has discussed the UE RACH-less handover in mobile IAB and achieved the following agreements: - RAN2#121bis meeting agreements: Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB). RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration: - Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details. - Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command). RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO. - RAN2#122 meeting agreements: RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured). For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report). for the UL grant and HO completion in RACH-less HO: - 1. Both type-1 configured grant and dynamic grant are supported - 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI). Send LS to RAN3 to check whether there are issues / feasibility concerns RAN2 would like to ask RAN3 to take those agreements into account and provide feedbacks if there are any issues or feasibility concerns. 2. Actions: To RAN3. ACTION: RAN2 respectively asks RAN3 to take the above agreements into account and provide feedbacks if there are any issues or feasibility concerns.

After sending this LS to RAN3, RAN2 agreed to support RACH-less handover [1]:

RACH-less HO to be supported for UEs connected to a mIAB node (intended case: DU migration) RACH-less HO for mIAB is expected to reuse most parts from other WI, such as NTN.

The following issues can be identified:

(1) During DU migration, UE handovers may not only occur from the source logical DU’s cell but also from other cells to the target logical DU’s cell. RAN3 assumes that RACH-less handover can only be applied to those UEs that are handed over from the source logical DU’s cell. The target logical DU therefore needs to be able to derive from the information contained in the F1AP UE Context Setup Request message received from the target CU during UE handover preparation, whether the UE is presently connected to the source logical DU. It is not certain if this F1AP message holds sufficient information for this purpose.

(2) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover, it needs to identify the beam configuration this UE presently uses in the source logical DU’s cell. For this purpose, it needs to able to derive the identifier this UE uses in the source logical DU’s cell from the information contained in the target CU’s F1AP UE Context Setup Request. It is not certain is not certain if this F1AP message holds sufficient information for this purpose.

These issues should be raised to RAN2. It is possible that the information necessary is already contained in the CellGroupConfig passed from the source CU via the target CU to the target logical DU. RAN2 should investigate the matter and get back to RAN3 in case additional information needs to be explicitly included in Xn or F1.

Proposal 1: RAN3 to reply to RAN2 on RACH-less handover during DU migration:

“RAN3 identified the following issues:

(1) During DU migration, UE handovers may not only occur from the source logical DU’s cell but also from other cells to the target logical DU’s cell. RAN3 assumes that RACH-less handover can only be applied to those UEs that are handed over from the source logical DU’s cell. The target logical DU therefore needs to be able to derive from the information it receives during UE handover preparation, whether the UE is presently connected to the source logical DU.

(2) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover, it needs to identify the beam configuration this UE presently uses in the source logical DU’s cell. For this purpose, it needs to able to derive from the information it receives during UE handover preparation an identifier the UE uses the source logical DU’s cell.

During UE handover preparation, the target logical DU receives the F1AP UE Context Setup Request message from the target CU containing the RRC container sent in the handover request. RAN3 kindly asks RAN2 to ensure that the above issues can be addressed through the information provided in this RRC container, and to get back to RAN3 in case explicit signaling via Xn or F1 is needed.”

Proposal 2: Agree to draft Reply LS to RAN2 on RACH-less handover provided in the Annex.

# Conclusion

This contribution discussed the Reply LS to RAN2 on RACH-less HO for mobile IAB. The following proposals have been made:

Proposal 1: RAN3 to reply to RAN2 on RACH-less handover during DU migration:

“RAN3 identified the following issues:

(1) During DU migration, UE handovers may not only occur from the source logical DU’s cell but also from other cells to the target logical DU’s cell. RAN3 assumes that RACH-less handover can only be applied to those UEs that are handed over from the source logical DU’s cell. The target logical DU therefore needs to be able to derive from the information it receives during UE handover preparation, whether the UE is presently connected to the source logical DU.

(2) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover, it needs to identify the beam configuration this UE presently uses in the source logical DU’s cell. For this purpose, it needs to able to derive from the information it receives during UE handover preparation an identifier the UE uses the source logical DU’s cell.

During UE handover preparation, the target logical DU receives the F1AP UE Context Setup Request message from the target CU containing the RRC container sent in the handover request. RAN3 kindly asks RAN2 to ensure that the above issues can be addressed through the information provided in this RRC container, and to get back to RAN3 in case explicit signaling via Xn or F1 is needed.”

Proposal 2: Agree to draft Reply LS to RAN2 on RACH-less handover provided in the Annex.

# Reference

[1]  Chairman notes, 3GPP TSG RAN2 #123, Toulouse, France, August 2023

# Annex

3GPP TSG RAN WG3#121	R3-23xxxx

Chicago, IL, USA, November 13 – 17,2023

Title:	[Draft] Reply LS on UE RACH-less handover for mobile IAB

Response to:	LS in R3-233713 (R2-2306817) on UE RACH-less handover for mobile IAB

Release:	Rel-18

Work Item:	NR_mobile_IAB-Core

Source:	Qualcomm [to be: RAN3]

To:	RAN2

Cc:

Contact person:

Name:	Georg Hampel

Tel. Number:

E-mail Address:	ghampel@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 would like to thank RAN2 for their LS on UE RACH-less handover for mobile IAB (R3-233713/R2-2306817).

RAN3 has discussed RACH-less handover for mobile IAB during DU migration. RAN3 identified the following issues:

(1) During DU migration, UE handovers may not only occur from the source logical DU’s cell but also from other cells to the target logical DU’s cell. RAN3 assumes that RACH-less handover can only be applied to those UEs that are handed over from the source logical DU’s cell. The target logical DU therefore needs to be able to derive from the information it receives during UE handover preparation, whether the UE is presently connected to the source logical DU.

(2) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover, it needs to identify the beam configuration this UE presently uses in the source logical DU’s cell. For this purpose, it needs to able to derive from the information it receives during UE handover preparation an identifier the UE uses the source logical DU’s cell.

During UE handover preparation, the target logical DU receives the F1AP UE Context Setup Request message from the target CU containing the RRC container sent in the handover request. RAN3 kindly asks RAN2 to ensure that the above issues can be addressed through the information provided in this RRC container, and to get back to RAN3 in case explicit signaling via Xn or F1 is needed.

RAN3 asks RAN2 to take the above feedback into account.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks RAN2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #123, Feb 26 to March 1, 2024       Athens, Greece

TSG-RAN3 Meeting	 #123bis, April 15 to 19, 2024       China